Aero Vodochody L-29 Delfin — L29CHECKLIST

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "AFTER ENTERING THE COCKPIT" item count differs between passes (44 vs 21); verify nothing was dropped.

content_gap: Section "LANDING" item count differs between passes (21 vs 2); verify nothing was dropped.

INTERIOR INSPECTION R/C (for solo flight)

[ ]  Ejection seat .... secured

[ ]  Safety pins .... installed

[ ]  Parachute .... removed or secure

[ ]  Harness and lap belt .... secure

[ ]  ISOLATION VALVE Switch .... off

[ ]  FIRE EXT cover .... safety wired

[ ]  F/C STAT PRESS Lever .... NORMAL safety wired

[ ]  F/C DYN PRESS Lever .... NORMAL safety wired

[ ]  F/C ATT IND BANK Switch .... NORMAL

[ ]  F/C ATT IND PITCH Switch .... NORMAL

[ ]  LANDING LIGHT Switch .... NORMAL

[ ]  F/C LDG GEAR LOCK OUT .... OFF

[ ]  RAM AIR shut off lever .... CLOSED

ENGINE+FUEL GAGES

[ ]  Priority switch .... F/C

[ ]  EXT STORES JETTISON .... safety wired

[ ]  SEC B switch .... ON

[ ]  MASTER switch .... ON

[ ]  ICS switches--(avionics--) .... AS REQUIRED

[ ]  EMERG BRAKE .... OFF, safety wired

[ ]  Pressure gauges .... proper reading

[ ]  Hand valves .... closed, safety wired

[ ]  Canopy .... closed

[ ]  External CABIN SEAL lever .... INFLATE

AFTER ENTERING THE COCKPIT

[ ]  Parachute and harness .... tight and secure

[ ]  Shoulder harness lock .... as required

[ ]  Rudder pedals .... as required

[ ]  Oxygen equipment .... connect and hooked to

LEFT CONSOLE CHAIN

[ ]  Oxygen panel .... check pressure

[ ]  Emergency lever (RED) .... NORMAL

[ ]  Diluter demand lever (WHITE) .... NORMAL OXYGEN

[ ]  Supply lever (GREEN) .... ON

[ ]  CAB PRESS DUMP valve .... closed (CW)

[ ]  FIRE COCK LEVER .... OPEN, safety wired

[ ]  HYD DUMP VALVE lever .... CLOSED safety wired

[ ]  Throttle position .... idle

[ ]  Throttle friction .... as required

[ ]  FUEL SHUT OFF lever .... CLOSED

[ ]  DE-ICING lever .... OFF

[ ]  PNEU FILL DE -PRESS .... CLOSED (CW)

[ ]  ISOLATION VALVE switch .... OFF

[ ]  AIR START button .... covered, safety wired

[ ]  FIRE EXT COVER .... safety wired

[ ]  PITOT BOOM .... RIGHT safety wired

[ ]  PARACHUTE .... NO DELAY LANYARD ATT

INSTRUMENT PANEL

[ ]  TAXI LIGHT switch .... OFF

[ ]  LANDING LIGHT switch .... OFF

[ ]  HP OXYGEN indication .... check min 100 kg's

[ ]  LDG GEAR buttons .... check out

[ ]  All instrument readings .... normal

[ ]  EXT STORES switch .... SAFE, safety wired

[ ]  EXT TANKS switch .... OFF

[ ]  Arming switch .... BLANK

[ ]  RAM AIR shut off lever .... as required

RIGHT CONSOLE

[ ]  Electrical switches .... OFF

[ ]  EMERG BRAKE lever .... OFF, safety wired

[ ]  VHF & AVIONICS .... ALL OFF

[ ]  ICS panel --(avionics--) .... as required

[ ]  EMERG GEAR EXTENSION valve .... Closed, safety wired

[ ]  EMERG FLAP EXTENSION valve .... Closed, safety wired

[ ]  CABIN PRESS/ HEATING .... OFF

[ ]  CABIN HEAT .... AUTOMATIC

[ ]  Inverter select switch .... MAIN INV (down)

[ ]  All C/B's .... ON (in)

[ ]  Ejection pins .... in sockets

BEFORE STARTING ENGINE

[ ]  Helmet .... on

[ ]  Oxygen hose .... connect

[ ]  Radio cable .... connect

[ ]  Oxygen check .... complete

[ ]  Brake check .... 4kg/cm2

[ ]  Control stick and rudder .... free and correct movement

[ ]  BATT switch .... ON

[ ]  VOLT/AM METER check button .... depress, 24v

[ ]  COMMS switch .... master on

[ ]  Intercome .... check

[ ]  External power unit .... connect

[ ]  APU ON light (green) .... illuminated

[ ]  VHF radio .... ON

[ ]  Start up clearance .... request

[ ]  GEN switch .... ON

[ ]  ENG switch .... ON

[ ]  C&W panel .... push to test, all lights

[ ]  Electrical gear position indicators .... push to test, all red lights illuminated

ENGINE START

CAUTION: Be sure that the danger areas are clear before starting.

[ ]  GENERATOR light .... blinking

[ ]  DO NOT START light .... OUT

[ ]  GND START button .... depress for 3 sec

[ ]  Spark plugs igniting .... listen for sparking

[ ]  Throttle .... idle

[ ]  FUEL SHUT-OFF lever .... fully OPEN

[ ]  EGT (APU 700 degrees
BATT 750 degrees) .... keep manually within

[ ]  Fuel/ oil pressure .... rising limits

[ ]  Idle RPM .... 35% within 60 sec's

[ ]  APU .... disconnect

[ ]  Fuel pressure .... 12 kg's

[ ]  Oil pressure .... 0.5 kgs

[ ]  EGT (idle) .... 550'c MAX

[ ]  GENERATOR light .... out

FALSE START/ HOT START  [ABNORMAL]

CAUTION: Stop the engine immediately if one of the following occurs

NOTE: - no sparking of spark plugs

NOTE: - No oil pressure indication

NOTE: - EGT exceeds 750’c

NOTE: The engine must be stopped if there is;

NOTE: - no RPM rise

NOTE: - Idle RPM 35% is not reached after 60 sec’s

NOTE: - Oil pressure stays below 0.5 kg’s

TO STOP THE ENGINE

[ ]  Throttle idle

[ ]  FUEL SHUT OFF lever .... CLOSED

IN CASE OF FIRE CONTINUE WITH THE FOLLOWING STEPS;

[ ]  FIRE COCK lever .... SHUT

[ ]  FIRE EXT button .... DEPRESS

[ ]  All electric's switches .... OFF

[ ]  Aircraft .... Abandon

AFTER ENGINE START

[ ]  SEC B switch .... ON

[ ]  INV switch (PO-250) .... ON

[ ]  ICS panel .... recheck

[ ]  VHF/ AVIONICS .... ON

[ ]  Ext tank switch .... ON

ENGINE RUN-UP (first flight of day)

[ ]  Brakes .... apply

[ ]  Throttle .... idle 35%

[ ]  EGT .... 550’c MAX

[ ]  Fuel pressure .... 12 kg’s

[ ]  Oil pressure .... 0.5 kg’s

[ ]  Oil temperature .... 20’c MIN

[ ]  Throttle .... 52% (EGT, OP, OT, FP)

[ ]  C+W panel (T-9) .... check

[ ]  Attitude indicator (horizon) .... Erect

[ ]  RMI Compass .... synchronise with standby

[ ]  Fire warning .... check

[ ]  VOLTS/AMPS .... check 28V

[ ]  Throttle .... 65% (EGT, OP, OT,

[ ]  Flaps/ stabiliser .... check operation ,FP)

[ ]  Speed brakes .... check operation

[ ]  HYD PRESS gage .... check pressure

[ ]  EXT TANKS light (C+W) .... out

[ ]  Throttle .... 94% (EGT, OP, OT, FP)

[ ]  ISOLATION VALVE switch .... on (check light)

[ ]  RPM rise .... 1 to 3% RPM

[ ]  Throttle .... 97% (EGT, OP, OT, FP)

[ ]  Throttle .... 100%(EGT, OP, OT, FP)

[ ]  EGT .... 700'c max

[ ]  Fuel pressure .... 79 kgs max

[ ]  Oil pressure .... 1.8 to 2.5 kg's

[ ]  Oil temperature .... 110'c max

[ ]  Throttle .... idle (RPM stabilised)

[ ]  Throttle slam in 2 secs .... 12 to 14 sec's

[ ]  RPM swing .... 2% RPM max (5 times)

[ ]  EGT .... 730'c peak

[ ]  Throttle .... idle

[ ]  Flight controls .... recheck

[ ]  Taxi clearance .... request

ENGINE RUN UP (subsequent flight)

[ ]  Brakes .... apply

[ ]  Throttle .... idle 35%

[ ]  EGT .... 550'c max

[ ]  Fuel pressure .... 12 kg's

[ ]  Oil pressure .... 0.5 kg's

[ ]  Oil temperature .... 20'c min

[ ]  Throttle .... 52% (EGT, OP, OT, FP)

[ ]  C+W panel (T-9) .... check

[ ]  Attitude indicator .... erect

[ ]  Compass .... sycronize check standby

[ ]  Fire warning .... check

[ ]  VOLTS/ AMPS .... check 28V

[ ]  Throttle .... 65% (EGT, OP, OT, FP)

[ ]  Flaps/ stabiliser .... check operation (elc+mech

[ ]  Speed brake .... check operation ind)

[ ]  HYD PRESS gage .... check pressure

[ ]  EXT TANKS light (C+W) .... out

[ ]  Throttle .... idle

[ ]  Flight controls .... full & free

[ ]  Taxi clearance .... request

BEFORE TAXI

[ ]  Altimeter .... set

[ ]  Engine instruments .... in green

[ ]  All safety pins .... removed and stowed

[ ]  Canopy .... closed

[ ]  Pressurisation .... as required

TAXING

[ ]  Brakes .... apply

[ ]  Chocks .... removed

[ ]  Throttle .... 80%

[ ]  Brakes .... release and test

[ ]  Turn check left/ right .... perform

PRE TAKEOFF

[ ]  Trim .... 2' rear

[ ]  Speed brakes .... check in

[ ]  Flaps .... 15' set

[ ]  Fuel quantity .... check

[ ]  Engine and flight instruments .... T's & P's check

[ ]  Oxygen .... check

[ ]  Canopy .... locked

[ ]  Harness .... secure

[ ]  Hydraulic pressure .... check

[ ]  C+W panel .... check

[ ]  Flight controls .... full & free

[ ]  All pins .... removed and stowed

[ ]  Cockpit pressurisation .... check (both cockpits)

TAKEOFF

[ ]  Brakes .... apply

[ ]  Altimeter .... recheck

[ ]  Attitude indicator .... reset

[ ]  Compass .... RWY heading

[ ]  Radio compass needle .... check direction

[ ]  Throttle .... 90% RPM

[ ]  Engine instruments .... check

[ ]  Brakes .... release

[ ]  Throttle .... 100% max power

[ ]  Nose wheel lift off .... 140km/h

[ ]  Lift off .... 180km/h

AFTER TAKEOFF

[ ]  LANDING GEAR UP button .... push (min 50ft AGL)

[ ]  FLAPS 0' button .... push (min 150ft AGL)

[ ]  Throttle .... 97% upon reaching 250

[ ]  ELEV TRIM lever .... set km/hr

[ ]  Engine instruments .... check

[ ]  All warning lights .... out

CLIMB

[ ]  100% RPM max of 6 mins, then reduce to 97%

[ ]  initial climb speed 350 kph

[ ]  CABIN ALT PRESSURE .... CHECK

[ ]  Oxygen .... check

[ ]  Engine instruments .... check (EGT, OP, OT, FP)

CRUISE

[ ]  Level of maintain CAS or MACH number power about 90%

[ ]  Fuel quantity .... check

[ ]  Oxygen .... check

[ ]  Engine instruments .... check (EGT, OP, OT, FP)

[ ]  Location .... identify

DESCENT

NOTE: Normal decent is at 65% AT 300 KPH

[ ]  Fuel quantity .... check

[ ]  Engine & flight instruments .... check (EGT, OP, OT, FP)

[ ]  Approach .... request

[ ]  Altimeter .... set QFE

[ ]  De-mist .... as required

BEFORE LANDING

[ ]  Fuel quantity .... check

[ ]  All warning lights .... out

[ ]  Pneumatic & hydraulic gages .... check

LANDING

[ ]  Check speed/ distance for particular weight

[ ]  Approach .... 300 kph

DOWN WIND LEG

[ ]  Airspeed .... 290 kph

[ ]  LANDING GEAR .... select down

[ ]  Landing gear .... check indicators

[ ]  Airspeed .... 270 kph

[ ]  FLAPS 15' button .... select 15'

[ ]  Flaps indicator .... check

[ ]  Airspeed .... 250 kph

[ ]  Throttle .... 90-94%

BASE LEG

[ ]  Airspeed .... 240 kph

[ ]  Throttle .... 80-85%

[ ]  FLAPS 30' button .... select 30'

[ ]  Flaps indicator .... check

[ ]  Airspeed in turn .... 220 kph

FINAL APPROACH

[ ]  Landing gear + flaps .... check indicators

[ ]  Airspeed .... 200 kph

[ ]  Throttle .... as required

AFTER TOUCH DOWN

NOTE: Max braking speed 1300 lts 140 kph

NOTE: 1000 lts 145 kph

NOTE: 700 lts 150 kph

NOTE: 400 lts 160 kph

AFTER LANDING

[ ]  Trim .... neutral

[ ]  Speed brakes .... in

[ ]  Pitot heat .... off

[ ]  Flaps .... up

[ ]  Pressurisation .... off

ENGINE SHUT DOWN

[ ]  Chocks .... in place

[ ]  Canopies .... open

[ ]  Ejector seat pins .... install

[ ]  Throttle .... idle for 30 sec's

[ ]  FUEL SHUT OFF lever .... closed

[ ]  SEC B switch .... off

[ ]  INV switch .... off

[ ]  Engine run down time .... check min 50 sec's

AFTER ENGINE STOP

[ ]  ENG switch .... off

[ ]  GEN switch .... off

[ ]  BATT switch .... off

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Aero Vodochody/L-29 Delfin/L29CHECKLIST.txt

Rights: licensed

Checklist hash: 968a8e433fff23cd34195bf938db9a4590be5d1337926ff38148d86d02d9a413

Verify against the aircraft's own approved documentation before use.