American Champion Aircraft 8KCAB Decathlon — Paul_Yust_8KCAB_Decathlon_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

PREFLIGHT

[ ]  Gust Lock .... REMOVE

[ ]  If Empty Rear Seat .... FASTEN BELT

[ ]  Fuel Gauges .... CHECK

[ ]  ELT and Lights for Night .... CHECK

[ ]  Tie Downs & Chocks .... REMOVED

[ ]  Right Wing & Strut .... CHECK

[ ]  Right Wheel & Brake (24 PSI) .... CHECK

[ ]  Right Fuel Qty .... CHECK & SECURE

[ ]  Gascolator (1st Flight) .... DRAIN

[ ]  Oil Level .... 6-8 QTS

[ ]  Windshield .... CLEAN

[ ]  Prop/Spinner/Filter/Light .... CHECK

[ ]  Left Wing-same as right .... CHECK

[ ]  Pitot Tube/Stall Warning .... CHECK

[ ]  Static Port/Antennas .... CHECK

[ ]  Aft Belly Sump (1st Flight) .... SAMPLE

[ ]  Rudder & Elevator .... CHECK

[ ]  Tail Wheel (40 PSI) .... CHECK

[ ]  Tach. Time .... RECORD

ENGINE START

[ ]  Passenger Brief .... COMPLETE

[ ]  Seats, Harness, Door .... SECURE

[ ]  Fuel Shut-Off Valve .... ON

[ ]  Brakes .... TEST & SET

[ ]  Electrics/Avionics .... OFF

[ ]  Beacon (Always On) .... ON

[ ]  Alternate Air .... COLD

[ ]  Throttle .... 1/2” OPEN

[ ]  Prop .... FULL FORWARD

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Master .... ON

[ ]  Prime (*details below) .... AS REQ

[ ]  Prop Area .... CLEAR

[ ]  Control Stick .... FULL AFT

[ ]  Mags .... BOTH ON

[ ]  Starter (15 sec limit) .... ENGAGE

[ ]  Mixture (as engine starts) .... FULL RICH

[ ]  Throttle .... 1000 RPM

[ ]  Oil pressure .... CHECK

[ ]  Mixture (for taxi) .... LEAN

[ ]  Avionics .... ON & SET

[ ]  Brakes .... RELEASE

NOTE: * Cold Start Prime: Turn on fuel pump and push mixture to full rich. Hold
for 3 seconds then return mixture to idle cut-ff and turn off fuel pump.

NOTE: * Hot Start Prime: Not usually required.

NOTE: * Flooded Start: With throttle full open and mixture idle cut-off turn prop
10 revs then do half prime and try start again.

RUN-UP

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Elevator Trim Position .... TAKEOFF

[ ]  Flight Instruments .... SET

[ ]  Fuel Shut-Off Valve .... ON

[ ]  Mixture (<3000’) .... RICH

[ ]  Throttle .... 1800 RPM

[ ]  - Mags L/R (175/50) .... CHECK

[ ]  - Prop .... CYCLE TWICE

[ ]  - Alternate Air .... CYCLE

[ ]  - Engine Gauges .... GREEN

[ ]  - Throttle .... 1000 RPM

TAKE-OFF

[ ]  Doors/Windows/Belts .... SECURE

[ ]  Transponder/Strobes/Lights .... ON

[ ]  Fuel Pump (optional) .... ON

[ ]  Throttle/Prop/Mixture .... FULL FORWARD

[ ]  Rotate .... 56 RPM

[ ]  Climb Speed .... 80-90 MPH

[ ]  Fuel Pump (at 1000’) .... OFF

[ ]  Landing Light .... OFF

CRUISE

[ ]  Power .... 23”/2400 OR AS REQ

[ ]  Mixture .... LEAN BELOW 75% PWR

AEROBATICS

[ ]  Altitude .... FINISH ABOVE 3000’ AGL

[ ]  Clear Area .... COMPLETE

[ ]  Engine .... 24” /2500 & GREEN

[ ]  Mixture .... RICH

ENTRY SPEED

[ ]  - Acrobatic VA .... 130 MPH

[ ]  - Aileron or Barrel Roll .... 120 MPH

[ ]  - Loop .... 140 MPH

[ ]  - Immelmann .... 145 MPH

[ ]  - Hammerhead .... 145 MPH

[ ]  - Snap Roll .... 90 MPH

LANDING

[ ]  Brakes .... CHECK FIRM

[ ]  Parking Brake .... OFF

[ ]  Fuel Pump (optional) .... ON

[ ]  GUMPS Check .... COMPLETE

[ ]  Final Approach .... 70-80 MPH

SHUTDOWN

[ ]  S  Avionics/Electrics .... OFF

[ ]  L  Mixture .... IDLE-CUTOFF

[ ]  I   Mags .... OFF

[ ]  M Master .... OFF

[ ]  Tach Time .... RECORD

[ ]  Gust Lock (Seat Belt) .... SECURE

[ ]  Flight Plan .... CLOSED

ENGINE FAILURE AT TAKEOFF

[ ]  Throttle .... CLOSED

[ ]  Land .... ASAP

EMERGENCY LANDING

[ ]  Best Glide Speed .... 75 MPH

[ ]  Landing field .... SELECTED

[ ]  Seats & Belts .... READY

[ ]  Door .... AJAR

TIME PERMITTING:

[ ]  - Engine (details below) .... AIR START

[ ]  - Radio (121.5) .... MAYDAY

[ ]  - Transponder .... 7700

[ ]  - ELT .... ON

IF NO LUCK RESTARTING ENGINE THEN

[ ]  - Mixture .... IDLE CUTOFF

[ ]  - Fuel Shut-Off Valve .... OFF

[ ]  - Master & Mags .... OFF

[ ]  Final Approach .... 75 MPH

[ ]  Before Touchdown .... DO NOT STALL

DITCHING OVER WATER

[ ]  Cabin Side Door .... JETTISON

[ ]  If High Swells .... LAND PARALLEL

[ ]  Else .... LAND INTO WIND

[ ]  Before Touchdown .... DO NOT STALL

EMERGENCY EXIT/BAIL OUT

[ ]  Throttle .... CLOSED

[ ]  Door .... JETTISON

[ ]  Dive .... OUT & SLIGHTLY AFT

[ ]  Parachute .... OPEN WHEN CLEAR

ENGINE AIR START

[ ]  Airspeed .... 80+ MPH

[ ]  Mags .... BOTH ON

[ ]  Throttle .... 3/4 FORWARD

[ ]  Mixture (<5000) .... FULL FORWARD

[ ]  Prop .... FULL FORWARD

[ ]  Fuel Shut-Off Valve .... CHECK ON

[ ]  Alternate Air .... FULL HOT

[ ]  Fuel Pump .... ON

IF NO RESTART    THEN

[ ]  - Mixture .... IDLE THEN RICH

IF PROP STOPS WINDMILLING    THEN

[ ]  - Starter .... ENGAGE

V-SPEEDS (MPH @ Max Wt)

NOTE: |VS 53   |VR 56   |VA 121  |

NOTE: |VFINAL  |VX   64 |VNO 160?|

NOTE: |75      |        |        |

NOTE: |VGLIDE  |VY 76   |VNE 180 |

NOTE: |75      |        |        |

ENGINE FIRE – GROUND

[ ]  Starter .... CONTINUE CRANKING

[ ]  Mixture .... IDLE CUTOFF

[ ]  Throttle .... FULL OPEN

IF FIRE PERSIST  THEN

[ ]  - Fuel Shut-Off Valve .... OFF

[ ]  - Mags & Electrics .... OFF

[ ]  Exit Plane .... ASAP

[ ]  Fire Extinguisher .... AS REQ

ENGINE FIRE – IN FLIGHT

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Shut-off Valve .... OFF

[ ]  Mags & Electrics .... OFF

[ ]  Cabin Heat .... OFF

[ ]  Fire Extinguisher .... AS REQ

[ ]  Land .... ASAP

ELECTRICAL FIRE

[ ]  Mags .... REMAIN ON

[ ]  Master & All Other Electrics .... OFF

[ ]  Cabin Vents/Window .... OPEN IF REQ

[ ]  Fire Extinguisher .... AS REQ

[ ]  Master (if power required) .... ON

[ ]  - Electric Switches .... 1 AT TIME

[ ]  Land .... ASAP

ALTERNATOR/ELECTIC FAILURE

[ ]  Master .... OFF, then ON

IF DISCHARGE CONTINUES THEN

[ ]  - All Non-Essential Electric .... OFF

[ ]  - Fuses .... CHECK

[ ]  Land .... ASAP

ABNORMAL OIL PRESS/TEMP

[ ]  Throttle .... REDUCE POWER

[ ]  Land .... ASAP

ICING CONDITIONS

[ ]  Pitot Heat .... ON

[ ]  Cabin Heat .... OFF

[ ]  Direction and/or Altitude .... CHANGE

WEIGHT & BALANCE (LBS)

[ ]  Empty Weight .... 1326

[ ]  Normal Max Gross Weight .... 1950

[ ]  - Half Fuel Payload (19.5 gals) .... 504

[ ]  - Full Fuel Payload (39 gals) .... 384

[ ]  Acrobatic Max Gross Weight .... 1800

[ ]  - Half Fuel Payload (19.5 gals) .... 354

[ ]  - Full Fuel Payload (39 gals) .... 234

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: American Champion Aircraft/8KCAB Decathlon/Paul_Yust_8KCAB_Decathlon_checklist.txt

Rights: licensed

Checklist hash: e2c320b9fbc99fec717e4206f81f30192d4a8f3aae367523b2a30421edd2b879

Verify against the aircraft's own approved documentation before use.