ATR 72-200 — MAX_Atr72_Check

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Pre-Start Checklist

[ ]  Parking Brakes .... SET

[ ]  Battery Switch .... OFF

[ ]  Generator Switches .... OFF

[ ]  Power Management Switch .... CHECK SET TO (take off)

[ ]  Automatic Torque Switches .... CHECK OFF !!!

[ ]  Throttle .... IDLE

[ ]  Landing Gear Lever .... CHECK DOWN

[ ]  Propeller .... HIGH RPM

[ ]  Fuel Cond. Lever .... FUEL CUTOFF

[ ]  Idle Gate Lever .... GROUND IDLE

[ ]  Flaps .... UP

[ ]  Battery Switch .... ON

[ ]  Panel Lights .... ON if required

[ ]  Fuel Quantity .... CHECK

[ ]  Flight controls .... CHECK

[ ]  Avionics Master Switch .... ON

[ ]  Check Weather .... (ATIS, Flight Services)

[ ]  Anti-ice .... TEST/CHECK

[ ]  Request Clearance

[ ]  Transponder .... STANDBY

[ ]  Beacon .... ON

Startup Checklist

[ ]  Engine and Propeller Area Throttle .... CLEAR

[ ]  Throttle .... IDLE

[ ]  Fuel Valves .... OPEN

[ ]  Fuel Cond. Lever .... AUTO

[ ]  Idle Gate Lever .... FLIGHT IDLE

[ ]  Prop Brake .... OFF

[ ]  Engine Start Selection Switch .... CRANK few seconds

[ ]  Engine Start Selection Switch .... START A+B

[ ]  Start Button Engine 2 .... START (hold until engine starts)

[ ]  Generator 2 .... ON

[ ]  Start Button Engine 1 .... START (hold until engine starts)

[ ]  Generator 1 .... ON

[ ]  Loadmeter+Voltmeter .... CHECK for CHARGE

[ ]  Engine Start Selection Switch .... OFF

Before Taxi Checklist

[ ]  Nav Lights .... ON

[ ]  Taxi Lights / Logo Lights / Wing Lights .... ON

[ ]  Heading Indicator / Altimeter .... SET

[ ]  Instruments .... NORMAL OPERATION

[ ]  Radios and Avionics .... ON, CHECKED and SET

[ ]  Autopilot .... SET and OFF

[ ]  F/D .... ON

[ ]  Request Taxi Clearance

Taxi Checklist

[ ]  Parking Brake .... RELEASE

[ ]  Brakes .... CHECK during taxi

[ ]  Directional Gyro .... PROPER IND. during turns

[ ]  Turn Coordinator .... PROPER IND. during turns

[ ]  Artificial Horizon .... ERECT during turns

[ ]  Taxi to assigned runway .... SPEED Max. 20 knots

Before Take-off Checklist

[ ]  Parking Brake .... SET

[ ]  Fuel Quantity .... CHECK

[ ]  Throttle .... IDLE

[ ]  Elevator Trim .... SET for takeoff

[ ]  Flaps .... 15 deg

[ ]  Flight Controls .... FREE AND CORRECT

[ ]  Engine / Flight Instruments .... CHECK

[ ]  Radios and Avionics .... SET

[ ]  Landing Lights .... ON

[ ]  Taxi Lights .... OFF

[ ]  Strobe Light .... ON

[ ]  Pitot Heat .... ON

[ ]  Anti-Ice .... AS REQUIRED

[ ]  Power Management Switch .... SET TO (take off)

[ ]  Annunciator Lights .... CHECK

[ ]  Transponder .... ON

[ ]  Request Takeoff Clearance

Take-off Checklist

[ ]  Automatic Torque Switches .... BOTH ON

[ ]  Brakes .... RELEASE

[ ]  V1 = Vr = .... 110 KIAS

[ ]  Pitch .... 10 degrees

[ ]  V2 = .... 115 KIAS

[ ]  At Positive Climb Rate .... Touch Brakes

[ ]  Landing Gear .... RETRACT

[ ]  Power Management Switch .... SET CLB (climb)

[ ]  At 130 KIAS .... RETRACT FLAPS

[ ]  Annunciator Lights / Eng. Instruments .... CHECK

Climb-out Checklist

[ ]  Power Management Switch .... CHECK CLB

[ ]  Autopilot .... CHECK and SET

[ ]  Yaw Damper .... ON

[ ]  Landing Lights .... OFF

[ ]  Airspeed .... 190 KIAS / 1800 fpm / 89%

[ ]  above 12’000 ft .... 180 KIAS / 1500 fpm / 77%

[ ]  Engine Instruments .... MONITOR

[ ]  ATC .... AS REQUIRED

[ ]  At Transition Altitude (FL180) set Altimeter to 29,92" (1013mb)

Cruise Checklist

[ ]  Accelerate to cruise speed

[ ]  Power Management Switch .... SET CRZ (cruise)

[ ]  at 6’000 ft .... 245 KIAS / 78%

[ ]  at 9’000 ft .... 240 KIAS / 76%

[ ]  at 12’000 ft .... 235 KIAS / 73%

[ ]  at 18’000 ft .... 220 KIAS / 68%

[ ]  at 24’000 ft .... 205 KIAS / 63%

[ ]  Service Ceiling .... 25’000 ft

[ ]  Engine+Instruments .... CHECK

[ ]  Fuel Quantity .... CHECK

[ ]  Radios .... TUNED and SET

[ ]  Autopilot .... CHECK and SET

Descent Checklist

[ ]  Atis / Airport Information .... CHECK

[ ]  Altimeter .... CHECK

[ ]  Radios .... SET

[ ]  Automatic Torque Switches .... OFF !!!

[ ]  Descent Speed .... 200 - 240 KIAS / ~15%

[ ]  Descent Rate .... -1500 fpm

[ ]  Flaps / Landing Gear .... CHECK UP

[ ]  At Transition Altitude (FL180) reset Altimeter to local

[ ]  Check Weather .... (ATIS, Flight Services)

Approach Checklist

LOCALIZER LEVEL FLIGHT :

[ ]  Speed: Establish .... 160 KIAS / 28%

[ ]  Landing Lights .... ON

[ ]  Flaps .... 15 deg

[ ]  Speed: Establish .... 145 KIAS / 41%

[ ]  Turning towards runway set flaps .... 30 deg

[ ]  Speed: Establish .... 130 KIAS / 44%

[ ]  Landing Gear .... DOWN

FINAL GLIDESLOPE DESCENT :

[ ]  Speed: Establish .... 120 KIAS / 28%

[ ]  Descent Rate .... -600 fpm

[ ]  Elevator Trim .... AS DESIRED

[ ]  Parking Brake .... VERIFY OFF

[ ]  Anti-Ice .... AS REQUIRED

[ ]  Yaw Damper .... OFF

Landing Checklist

[ ]  Landing Gear .... CHECK DOWN, 3 GREEN

[ ]  Autopilot .... OFF

[ ]  Landing Speed .... 115 KIAS

[ ]  Touchdown .... MAIN WHEELS FIRST

[ ]  Landing Roll .... LOWER NOSE WHEEL

[ ]  After touchdown .... Apply Reverse Thrust, at 60 kts: Cancel Reverse Thrust

Taxi To Ramp

[ ]  Strobe Light .... OFF

[ ]  Taxi Lights .... ON

[ ]  Landing Lights .... OFF

[ ]  Flaps .... RETRACT

[ ]  Anti-Ice .... OFF

[ ]  Transponder .... 1200

[ ]  Speed .... Max. 20 knots

[ ]  Elevator Trim .... TAKEOFF SETTING

[ ]  Avionics/Radios .... AS REQUIRED

Shutdown Checklist

[ ]  Parking Brake .... SET

[ ]  Throttle .... IDLE

[ ]  Pitot Heat .... OFF

[ ]  F/D .... OFF

[ ]  Avionics .... OFF

[ ]  Taxi Lights .... OFF

[ ]  Logo Lights / Wing Lights .... OFF

[ ]  Nav Lights .... OFF

[ ]  Fuel Cond. Lever .... FUEL CUTOFF

[ ]  Idle Gate Lever .... GROUND IDLE

[ ]  Fuel Valves .... CLOSE

[ ]  Power Management Switch .... SET TO (take off)

[ ]  Beacon .... OFF

[ ]  Generators .... OFF

[ ]  Prop Brake .... ON

[ ]  Panel Light .... OFF

[ ]  Battery Switch .... OFF

Securing Aircraft

[ ]  Parking Brake .... Verify SET

[ ]  Throttle .... Verify IDLE

[ ]  All Switches .... Verify OFF

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: ATR/72-200/MAX_Atr72_Check.txt

Rights: licensed

Checklist hash: 0c7e6af3aec327e6ecb71da32614f02ad850c0359b5a5c2bc6e7b40f75880140

Verify against the aircraft's own approved documentation before use.