Beechcraft 23 _ A23 _ B23 _ C23 _ 23-24 Musketeer _ Musketeer II _ Sundowner _ Musketeer Super III — Carlson_Beechcraft_BE23_Checklist_MPH

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Before Landing" item count differs between passes (9 vs 8); verify nothing was dropped.

content_gap: Section "Cabin Fire" item count differs between passes (6 vs 5); verify nothing was dropped.

content_gap: Section "EMERGENCY DESCENT" item count differs between passes (4 vs 3); verify nothing was dropped.

content_gap: Section "Engine Failure – In Flight" item count differs between passes (28 vs 25); verify nothing was dropped.

content_gap: Section "Landing Technique (calm)" item count differs between passes (25 vs 22); verify nothing was dropped.

content_gap: Section "Leaning For Ground Ops" item count differs between passes (4 vs 3); verify nothing was dropped.

content_gap: Section "Lineup" item count differs between passes (15 vs 14); verify nothing was dropped.

content_gap: Section "Rough Running Engine" item count differs between passes (4 vs 3); verify nothing was dropped.

content_gap: Section "Shutdown" item count differs between passes (13 vs 11); verify nothing was dropped.

content_gap: Section "Speeds" item count differs between passes (18 vs 11); verify nothing was dropped.

content_gap: Section "Starting Engine" item count differs between passes (21 vs 20); verify nothing was dropped.

content_gap: Section "Takeoff" item count differs between passes (4 vs 2); verify nothing was dropped.

Before Start

[ ]  Preflight .... COMPLETE

[ ]  Passenger Briefing .... COMPLETE

[ ]  Seats ……………...…….......... .... ADJUST/LOCK

[ ]  Seat Belts/Shoulder Strap .... FASTEN

[ ]  Electrical Equip. .... OFF

[ ]  Avionics Power .... OFF

[ ]  Brakes .... TEST AND SET

[ ]  Circuit Breakers .... CHECK IN

[ ]  Hobbs Meter .... RECORD TIME

Starting Engine

[ ]  Fuel Selector .... .SET TO FULLEST TANK

[ ]  Mixture .... FULL RICH (FORWARD)

[ ]  Carb Heat .... OFF (FORWARD)

[ ]  Battery Switch .... ON

[ ]  Alternator Switch .... ON

[ ]  Rotating Beacon .... ON

[ ]  Fuel Boost Pump .... ON

[ ]  Prime .... 3 STROKES

NOTE: THEN IN & LOCK

[ ]  Throttle .... OPEN ½ INCH

[ ]  Propeller Area .... CLEAR

[ ]  Ignition Switch .... START

[ ]  Oil Pressure .... WITHIN 30 SEC.

[ ]  Warm Up .... 800-1000 RPM

[ ]  All Engine Gages .... CHECK

[ ]  Navigation Lights .... ON (Night)

[ ]  Avionics Power .... ON

[ ]  Radios .... ON

[ ]  Transponder .... STAND-BY

[ ]  ATIS/AWOS .... OBTAIN

[ ]  CLEARANCE .... OBTAIN

Leaning For Ground Ops

[ ]  Throttle .... 1500 RPM

[ ]  Mixture .... LEAN TILL RPM PEAKS

[ ]  Throttle .... 1000 RPM OR

NOTE: AS NEEDED FOR GROUND OPS

Taxi Checks

[ ]  Brakes .... CHECK

[ ]  Instruments .... CHECK

[ ]  Lights .... AS REQUIRED

Before Takeoff & Run Up

[ ]  Cabin Doors .... CHECK

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Stabilator Trim .... TAKEOFF

[ ]  Flight Instruments .... CHECK & SET

[ ]  Radios/Avionics .... SET

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Mixture .... RICH

[ ]  Throttle .... 2200 RPM

[ ]  Engine Gages

[ ]  Ammeter

[ ]  Instrument Air

[ ]  Magnetos .... CHECK L & R

NOTE: 125 RPM max drop

NOTE: 50 RPM max variance

[ ]  Carb. Heat .... ON (FULL AFT)

NOTE: CHECK FOR RPM DROP

[ ]  Throttle .... FULL OUT

NOTE: Engine should not stall

[ ]  Carb. Heat .... OFF (FULL FWD)

[ ]  Throttle .... <1000 RPM

[ ]  Mixture .... LEAN FOR GROUND OPS

[ ]  Lights .... AS REQUIRED

[ ]  Throttle Friction Lock .... ADJUST

[ ]  Flaps .... SET & CHECK

[ ]  Window .... LATCHED

[ ]  Radio .... ANNOUNCE INTENTIONS

Lineup

[ ]  Dir. Gyro .... Runway Heading

[ ]  Stabilator Trim .... TAKEOFF

[ ]  Flaps .... SET & CHECK

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Transponder .... ALT

[ ]  Mixture .... RICH

[ ]  Carb Heat .... OFF (FORWARD)

[ ]  Fuel Boost Pump .... ON

[ ]  Landing Light .... ON

[ ]  Heading .... RUNWAY

[ ]  Radio .... ANNOUNCE DEPARTURE

[ ]  Throttle .... FULL POWER – 2700 RPM

[ ]  Brakes .... HOLD FOR 2000 RPM

NOTE: THEN RELEASE

[ ]  Centerline .... HOLD WITH RUDDER

Takeoff

[ ]  Rotate .... 75 MPH

NOTE: (85 MPH SHORT FIELD)

[ ]  Climb .... 86 MPH

NOTE: (79 MPH SHORT FIELD)

Climb - 700 ft AGL

[ ]  Climb Cruise .... 94 MPH

[ ]  Throttle .... FULL

[ ]  Mixture .... FULL RICH

[ ]  Oil Temp .... MONITOR

[ ]  Mixture .... LEAN AS REQ’D

[ ]  Lights .... AS REQUIRED

Cruise

[ ]  Power/RPM .... SET

[ ]  Fuel Boost Pump .... OFF

[ ]  Landing Light .... OFF

[ ]  Trim .... ADJUST

[ ]  Mixture .... LEAN EGT TO 1320 DEG. F

[ ]  Engine Gauges .... CHECK

[ ]  Ammeter .... CHECK

[ ]  Instrument Air .... CHECK

[ ]  Throttle Friction Lock .... TIGHTEN

Descent

[ ]  Altimeter .... SET

[ ]  Mixture .... ADJUST

[ ]  Power .... AS DESIRED

[ ]  Carb. Heat .... AS REQUIRED

[ ]  Fuel Selector .... FULLEST TANK

Before Landing

[ ]  Seats .... ADJUST

NOTE: AND LOCK

[ ]  Seat Belts/Shoulder Strap .... FASTEN

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Carb. Heat .... ON

[ ]  Mixture .... LEAN FOR PEAK RPM

[ ]  Fuel Boost Pump .... ON

[ ]  Landing Light .... ON

[ ]  Throttle .... 1700 RPM

Landing Technique (calm)

DOWNWIND

[ ]  Flaps .... FIRST NOTCH

[ ]  Speed .... 92 MPH

NOTE: & 600 FPM DESCENT

BASE

[ ]  Flaps .... SECOND NOTCH

[ ]  Speed .... 86 MPH

NOTE: & 600 FPM DESCENT

FINAL

[ ]  Flaps .... THIRD NOTCH (if needed)

[ ]  Speed .... 78 MPH

NOTE: HOLD GLIDESLOPE

NOTE: Pitch for Altitude

NOTE: Power for Airspeed

SHORT FINAL (OVER TREES)

[ ]  Speed .... 75 MPH

[ ]  Mixture .... RICH

[ ]  Carb Heat .... OFF (FORWARD)

[ ]  Touchdown .... 59 MPH

[ ]  Landing Roll .... LOWER NOSE

NOTE: WHEEL GENTLY

[ ]  Flaps .... FULL UP

[ ]  Stabilator .... FULL BACK

[ ]  Braking .... MINIMUM REQUIRED

[ ]  Mixture .... LEAN FOR GROUND OPS

Speeds

NOTE: Max Climb

NOTE: Best Angle (VX) 86 MPH

NOTE: Best Rate (VY) 79 MPH

NOTE: Best Glide

NOTE: Clean 90 MPH

NOTE: Full flaps 78 MPH

NOTE: Maneuvering 136 MPH

NOTE: (VA)

NOTE: Flaps Extended 110 MPH

NOTE: (VFE)

NOTE: Never Exceed 175 MPH

NOTE: (VNE)

NOTE: Max. Cruise 157 MPH

NOTE: (VNO)

NOTE: Stall

NOTE: No Flaps (VS1) 72 MPH

NOTE: Full Flaps 59 MPH

NOTE: (VS0)

After Landing

[ ]  Flaps .... RETRACT

[ ]  Stabilator Trim .... TAKEOFF

[ ]  Transponder .... STAND-BY

[ ]  Fuel Boost Pump .... OFF

[ ]  Landing Light .... OFF

[ ]  Lights .... AS REQUIRED

Shutdown

[ ]  Parking Brake .... DO NOT SET

[ ]  Avionics Power .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Throttle .... 1800 RPM

[ ]  Mixture .... PEAK RPM

NOTE: HOLD 1800 RPM FOR 30 SEC.

[ ]  Mixture & Throttle .... FULL OFF

[ ]  Ignition Switch .... OFF AFTER

NOTE: ENGINE STOPS

[ ]  Battery Switch .... OFF

[ ]  Alternator Switch .... OFF

[ ]  Control Lock .... INSTALL

[ ]  Tachometer .... RECORD TIME

Pre-Flight (Cockpit)

[ ]  Ignition Switch .... OFF

[ ]  Ckt. Breakers .... CHECK

[ ]  Avionics .... OFF

[ ]  Battery Switch .... ON

[ ]  Fuel Gages .... DO NOT TRUST

[ ]  Rotating Beacon .... CHECK

[ ]  Landing Light .... CHECK

[ ]  Nav Lights .... CHECK

[ ]  Pitot Heat .... CHECK

[ ]  Flaps .... FULL DOWN

[ ]  Throttle .... FULL OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Battery Switch .... OFF

[ ]  Controls .... FREE

Pre-Flight (Exterior)

[ ]  Wing Surfaces & Tips .... CHECK

[ ]  Flaps .... CHECK

[ ]  Ailerons .... CHECK

[ ]  Fuel Sump (L, C & R) .... SAMPLE

[ ]  Fuel Quantity (L & R) .... CHECK

[ ]  Engine Oil .... CHECK (7-6 qts)

[ ]  Fuel Strainer .... SAMPLE

[ ]  Propeller & Spinner .... CHECK

[ ]  Alternator Belt .... CHECK

[ ]  Engine Cowling .... CHECK

[ ]  Air Intake .... CHECK

[ ]  Stall Warning .... CHECK

[ ]  Pitot Cover .... REMOVE

[ ]  Pitot Mast .... CHECK

[ ]  Landing Gear (L, R & nose) .... CHECK

[ ]  Static Ports .... CHECK

[ ]  ELT .... Armed

[ ]  Stabilator .... CHECK

[ ]  Rudder .... CHECK

[ ]  Antennas .... CHECK

[ ]  Baggage Door .... CHECK

[ ]  Skin/Rivets .... CHECK

[ ]  Tie-downs .... REMOVE

[ ]  Chocks .... REMOVE

Engine Fire – During Start  [EMERGENCY]

IF ENGINE STARTS

[ ]  Cranking .... CONTINUE

[ ]  Power .... 1700 RPM

[ ]  Engine .... SHUTDOWN

IF ENGINE FAILS TO START

[ ]  Throttle .... FULL OPEN

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Cranking .... CONTINUE

[ ]  Fire Extinguisher .... OBTAIN

[ ]  Engine .... SECURE

[ ]  Master Switch .... OFF

[ ]  Ignition Switch .... OFF

[ ]  Fuel Selector .... OFF

[ ]  Fire .... EXTINGUISH

[ ]  Fire Damage .... INSPECT

Engine Fire – IN FLIGHT  [EMERGENCY]

[ ]  Fuel Selector .... OFF

[ ]  Mixture .... IDLE CUT OFF

[ ]  Throttle .... CLOSE

[ ]  Cabin Air (Red Knob) .... PULL OFF

[ ]  Defrost Valve (Red Knob) .... PUSH OFF

[ ]  Battery Switch .... OFF

[ ]  Ignition Switch .... OFF

WARNING: Do Not Attempt Restart!

Cabin Fire  [EMERGENCY]

[ ]  Master Switch .... OFF

[ ]  Vents / Cabin Air .... CLOSED

[ ]  Fire Extinguisher .... ACTIVATE

[ ]  Vents/Cabin Air .... OPEN

NOTE: (when fire is out)

[ ]  Flight .... TERMINATE ASAP

Engine Failure – Takeoff Roll  [EMERGENCY]

[ ]  Throttle .... FULL OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Brakes .... MAXIMUM

[ ]  Flaps .... FULL UP

[ ]  Fuel Selector .... OFF

[ ]  Fuel Boost .... OFF

[ ]  Battery Switch .... OFF

[ ]  Alternator Switch .... OFF

[ ]  Ignition Switch .... OFF

Rough Running Engine  [ABNORMAL]

[ ]  Carb Heat .... FULL ON

[ ]  Mixture .... FULL RICH

[ ]  Ignition Switch .... CHECK

NOTE: LEFT, RIGHT & BOTH

EMERGENCY DESCENT  [EMERGENCY]

[ ]  Carb Heat .... FULL ON

[ ]  Throttle .... FULL OFF

[ ]  Airspeed .... 175 MPH CLEAN

NOTE: 110 MPH (FULL FLAPS)

Engine Failure – In Flight  [EMERGENCY]

WARNING: LAND STRAIGHT AHEAD unless able to maneuver

ESTABLISH GLIDE

NOTE: 90 MPH (NO FLAPS)

NOTE: Pick landing site.

ATTEMPT RESTART

[ ]  Airspeed .... 90 MPH NO FLAPS

[ ]  Fuel Selector .... SWITCH TANKS

[ ]  Mixture .... RICH

[ ]  Carb. Heat .... FULL ON (AFT)

[ ]  Fuel Boost .... ON

[ ]  Primer .... IN & LOCKED

[ ]  Ignition Switch .... CHECK

NOTE: LEFT, RIGHT & BOTH

NOTE: OR START IF PROP STOPS

IF NO RESTART

[ ]  Airspeed .... 90 MPH (NO FLAPS)

[ ]  Fuel Selector .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Fuel Boost .... OFF

[ ]  Primer .... IN & LOCKED

[ ]  Ignition Switch .... OFF

[ ]  Radio Call .... 121.5 - “MAYDAY!”

[ ]  Transponder .... SQUAWK 7700

WHEN LANDING IS ASSURED

[ ]  Flaps .... AS REQUIRED

[ ]  Battery Switch .... OFF

[ ]  Airspeed .... 78 MPH(FULL FLAPS)

NOTE: Hold nose off as long as possible.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Beechcraft/23 _ A23 _ B23 _ C23 _ 23-24 Musketeer _ Musketeer II _ Sundowner _ Musketeer Super III/Carlson_Beechcraft_BE23_Checklist_MPH.txt

Rights: licensed

Checklist hash: fdf7059509f012c346cff4e7e8952ec539b79323aafa8831484da04fa6cfe1a1

Verify against the aircraft's own approved documentation before use.