Beechcraft 23 _ A23 _ B23 _ C23 _ 23-24 Musketeer _ Musketeer II _ Sundowner _ Musketeer Super III — Rolf_Arnold_Beech_Musketeer_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

AIRSPEEDS FOR SAFE OPERATION

NOTE: All airspeeds quoted in this section are indicated airspeeds (IAS)

TAKE-OFF

[ ]  Lift-off .... 65 KTS / 75 MPH

[ ]  50 Ft. .... 73 KTS / 84 MPH

MAXIMUM CLIMB

[ ]  Best Rate (VY) .... 75 KTS / 86 MPH

[ ]  Best Angle (VX) .... 68 KTS / 78 MPH

[ ]  Cruise Climb .... 87 KTS / 100 MPH

MAXIMUM TURBULENT AIR PENETRATION

[ ]  Balked Landing .... 114 KTS / 131 MPH

[ ]  Landing Approach .... 64 KTS / 74 MPH

[ ]   .... 69 KTS / 79 MPH

[ ]  Emergency Glide speed .... 78 KTS / 90 MPH

[ ]  Maximum Demonstrated Crosswind .... 13 KTS / 15 MPH

Preflight Inspection

[ ]  Emergency Locator Transmitter .... ARMED

1. CABIN

[ ]  Parking Brake .... SET

[ ]  Control Lock .... REMOVED

[ ]  All Switches .... OFF

[ ]  Flaps .... DOWN

2. RIGHT FUSELAGE

[ ]  Baggage Door .... CLOSED

[ ]  Static Pressure Button .... UNOBSTRUCTED

3. EMPENNAGE

[ ]  Control Surfaces .... CHECK

[ ]  Position Light .... CHECK

[ ]  Tie Down .... REMOVED

4. LEFT FUSELAGE

[ ]  All Antennas .... CHECK

[ ]  Static Pressure Button .... UNOBSTRUCTED

5. LEFT WING TRAILING EDGE

[ ]  Flap .... CHECK

[ ]  Fuel Vent line .... UNOBSTRUCTED

[ ]  Aileron .... CHECK

[ ]  Wing Tip .... CHECK

[ ]  Position Light .... CHECK

6. LEFT WING LEADING EDGE

[ ]  Pitot Tube .... CHECK

[ ]  Landing Light .... CHECK

[ ]  Tie Down and Chocks .... REMOVED

[ ]  Stall Warning .... CHECK

[ ]  Fuel Tank .... CHECK QUANTITY

[ ]  Filler Cap .... SECURE

7. LEFT LANDING GEAR

[ ]  Tire and Brake .... CHECK

[ ]  Fuel Sump .... DRAIN

8. NOSE SECTION

[ ]  Left Cowl .... SECURE

[ ]  Engine Oil .... CHECK

[ ]  Cap and Dipstick .... SECURE

[ ]  Induction Air Intake .... CHECK

[ ]  Filter .... CHECK

[ ]  Propeller .... CHECK

[ ]  Tire and Nose Gear .... CHECK

[ ]  Right Cowl .... SECURE

[ ]  Chocks and Tow Bar .... REMOVED

[ ]  Fuel Strainer .... DRAIN

9. RIGHT LANDING GEAR

[ ]  Fuel Sump .... DRAIN

[ ]  Tire and Brake .... CHECK

10. RIGHT WING LEADING EDGE

[ ]  Fuel Tank .... CHECK QUANTITY

[ ]  Filler Cap .... SECURE

[ ]  Tie Down and Chocks .... REMOVED

[ ]  Wing Tip .... CHECK

[ ]  Position Light .... CHECK

11. RIGHT WING TRAILING EDGE

[ ]  Aileron .... CHECK

[ ]  Flap .... CHECK

[ ]  Fuel Vent line .... UNOBSTRUCTED

BEFORE STARTING ENGINE

[ ]  Seats .... POSITION AND LOCK

[ ]  Seat Belts .... FASTEN

[ ]  Parking Brake .... SET

[ ]  All Avionics .... OFF

[ ]  Circuit Breakers .... IN

[ ]  Flaps .... UP

[ ]  Clock .... SET

[ ]  Fuel Selector .... ROTATE thru 360 o check for freedom and movement

[ ]  Fuel Selector .... LEFT TANK

[ ]  Light Switches .... AS REQUIRED

[ ]  Battery Switch .... ON

[ ]  ALT Switch .... ON

[ ]  If external power is used .... OFF

[ ]  Horizontal Indicator .... SET

WARNING: Do not take off if gages indicate in yellow arc on either gage (airplanes after compliance with BEECHCRAFT S.I. No. 0624-281).

STARTING ENGINE

CAUTION: Starter cranking period should be limited to a maximum of 30 seconds, with at least 3 to 5 minutes between cranking periods.

NORMAL START:

[ ]  Mixture .... FULL RICH

[ ]  Throttle .... IDLE position

[ ]  Fuel Boost Pump .... ON (until fuel pressure stabilizes at 2.5 psi)

[ ]  Start Switch .... START position (release to BOTH position when engine fires)

[ ]  Fuel Boost Pump .... OFF (After engine is running smoothly)

[ ]  Oil Pressure .... IN YELLOW position within 30 seconds

[ ]  Engine Instruments .... CHECK

[ ]  Warm-up .... 800 to 1200 RPM

HOT START:

[ ]  Mixture .... FULL RICH

[ ]  Throttle .... IDLE position

[ ]  Starter Switch .... START position (release to BOTH position when engine fires)

[ ]  Fuel Boost Pump .... ON

[ ]  Fuel Boost Pump .... OFF (After engine is running smoothly)

[ ]  Oil Pressure .... IN YELLOW position within 30 seconds

[ ]  Engine Instruments .... CHECK

[ ]  Warm-up .... 800 to 1200 RPM

BEFORE TAXI

[ ]  Avionics Equipment .... ON, AS REQUIRED

[ ]  Anti-Collision Light .... ON

[ ]  Lights .... AS REQUIRED

[ ]  Radio .... SET

BEFORE TAKEOFF

[ ]  Parking Brake .... SET

[ ]  Seat Belts .... CHECK

NOTE: All reclining seats must be in the upright position during take-off.

[ ]  Flight Instruments .... CHECK AND SET

[ ]  Throttle .... 1700 RPM

[ ]  Magnetos .... CHECK at 1700 RPM, maximum drop of 150 RPM on each magneto, variance between individual magnetos should not exceed 50 RPM

[ ]  Vacuum .... CHECK

[ ]  Throttle .... 1500 RPM

[ ]  Ammeter .... CHECK stabilized indication within 1/4 scale of zero

[ ]  Throttle .... FAST IDLE

[ ]  Trim .... TAKEOFF

[ ]  Flaps .... CHECK AND SET 10 0

[ ]  Controls .... CHECK

[ ]  Doors .... SECURE

[ ]  Fuel Boost Pump .... OFF

[ ]  Mixture .... FULL RICH

[ ]  Oil Temperature .... CHECK

[ ]  Parking Brake .... RELEASE

TAKEOFF

[ ]  Power .... FULL THROTTLE

[ ]  Airspeed .... ACCELERATE 65 KTS

[ ]  Climb Speed .... CLEARED OBSTACLES

BEFORE LANDING

[ ]  Seat Belts .... SECURE

NOTE: All reclining seats must be in the upright position during landing.

[ ]  Mixture .... FULL RICH

[ ]  Landing Light .... ON

[ ]  Flaps .... DOWN

[ ]  Airspeed .... 69 KTS

AFTER LANDING

[ ]  Flaps .... UP

[ ]  Trim Tab .... SET to O0

SHUTDOWN

[ ]  Electrical and Avionics .... OFF

[ ]  GPS .... OFF

[ ]  Throttle .... CLOSE

[ ]  Mixture .... IDLE CUT OFF

[ ]  Magneto/Start Switch .... OFF

[ ]  Battery & ALT Switch .... OFF

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Beechcraft/23 _ A23 _ B23 _ C23 _ 23-24 Musketeer _ Musketeer II _ Sundowner _ Musketeer Super III/Rolf_Arnold_Beech_Musketeer_checklist.txt

Rights: licensed

Checklist hash: 59dd519ee0f713c30af4f20fb4b600abb9c8adab71e48839fb5e66c04a764c98

Verify against the aircraft's own approved documentation before use.