Beechcraft 95 Travelair — Beech95-checklist_selkirk_college

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Cabin Fire" item count differs between passes (6 vs 5); verify nothing was dropped.

Pre Start

[ ]  Hobbs .... Checked

[ ]  Dispatch log .... Completed

[ ]  Logs and documents .... On board & valid

[ ]  Passenger briefing .... Completed

[ ]  Seats and belts .... Locked and secure *

[ ]  Controls .... Free and correct

[ ]  Parking brake .... Set

[ ]  Circuit breakers .... All in

[ ]  Electrics .... Off

[ ]  Alternators .... Off

[ ]  Avionics emergency .... Off

[ ]  Avionics master .... Off

[ ]  Gear selector .... Down

[ ]  Battery .... On, Gear locked

[ ]  Fuel Quantities .... Checked

[ ]  Fuel selectors .... On Aux.

[ ]  Flaps .... Up

[ ]  Cowl flaps .... Open

[ ]  Beacon .... On

Cold Engine Start

[ ]  Mixtures .... Full rich

[ ]  Propellers .... Full fine

[ ]  Throttles .... Set

[ ]  Boost pumps .... Checked & primed

[ ]  Left engine (start) .... Started

[ ]  Oil Pressure .... Normal

[ ]  Left Suction .... Normal

[ ]  Left Alternator .... On

[ ]  Right engine .... (start) Started

[ ]  Oil pressure .... Normal

[ ]  Right Suction .... Normal

[ ]  Right alternator .... On, Load split

[ ]  Engine gauges .... Normal

Hot or Flooded Start

[ ]  Mixtures .... Full rich

[ ]  Propellers .... Full fine

[ ]  Throttles .... Full forward

[ ]  Boost Pumps .... Checked & primed

[ ]  Mixtures .... Idle cut off

[ ]  Throttles .... Set

[ ]  Left engine (start) .... Started

[ ]  Oil pressure .... Normal

[ ]  Left Suction .... Normal

[ ]  Left Alternator .... On

[ ]  Right engine .... (start) Started

[ ]  Oil Pressure .... Normal

[ ]  Right Suction .... Normal

[ ]  Right alternator .... On, Load split

[ ]  Engine gauges .... Normal

Post Start

[ ]  Avionics master .... On

[ ]  PN-101 .... On and slave set

[ ]  Electrics .... On (as req=d)

[ ]  ELT .... Checked

[ ]  Instrument panel .... Panel checked

[ ]  Radios .... Radios checked

[ ]  Left fuel selector .... On crossfeed

Taxi

[ ]  Brakes .... Checked *

[ ]  Fuel crossfeed .... Checked

[ ]  Instruments .... Checked

Runup

[ ]  Parking brake .... Set

[ ]  Fuel selectors .... Quantity checked, on mains

[ ]  Mixtures .... Full rich

[ ]  Propellers .... Full fine

[ ]  Engine gauges .... Normal

[ ]  Throttles .... 2200

[ ]  Engine gauges .... Normal

[ ]  Suction .... Normal

[ ]  Propellers .... Exercised

[ ]  Throttles .... 2000

[ ]  Magnetos .... Checked

[ ]  Mixtures .... Checked

[ ]  Alternators .... Checked

[ ]  Alternate airs .... Checked

[ ]  Throttles .... 1500

[ ]  Feather .... Checked

[ ]  Throttles .... Idle checked

[ ]  1300 Set

Pre-Takeoff

[ ]  Seats & belts .... Locked & secure *

[ ]  Controls .... Free

[ ]  Trims .... Three set

[ ]  Fuel selectors .... Quantity checked, on Mains

[ ]  Alternate airs .... Off

[ ]  Flaps .... Set for takeoff

[ ]  Cowl flaps .... Open

[ ]  Circuit breakers .... All in

[ ]  Alternate static .... Checked and off

[ ]  Magnetos .... Both on both

[ ]  Alternators .... On

[ ]  Mixtures .... Full rich

[ ]  Propellers .... Full fine

[ ]  Altimeters .... _____ set

[ ]  Clock .... ______checked

[ ]  Heading indicators .... Checked & set

[ ]  Rear windows .... Closed & locked

[ ]  IFR clearance .... Clearance reviewed

[ ]  Crew briefing .... Briefing completed

[ ]  Cabin door & window .... Closed & locked *

[ ]  Boost pumps .... On

[ ]  Landing light .... On (if req=d)

[ ]  Pulse light .... On

[ ]  Pitot heat .... On (if req=d)

[ ]  Transponder .... On Alt ______

[ ]  Takeoff time .... Noted

[ ]  Heading indicator .... Checked on runway

Post Takeoff

[ ]  Gear .... Up

[ ]  Flaps .... Up

[ ]  Power .... Set

[ ]  Fuel selectors .... Quantity checked, on Aux

[ ]  Boost pumps .... Off

[ ]  Cowl flaps .... Set

[ ]  Landing light .... Off

Pre-Landing

[ ]  Seats & belts .... Locked & secure

[ ]  Parking brake .... Off

[ ]  Mixtures .... Full rich

[ ]  Propellers .... 2500

[ ]  Magnetos .... Both on both

[ ]  Electrics .... On (as req=d)

[ ]  Boost pumps .... On

[ ]  Fuel selectors .... Quantity checked, on mains

[ ]  Landing light .... On (if req=d)

[ ]  Gear .... Down & locked

Post Landing

[ ]  Flaps .... Up

[ ]  Cowl flaps .... Open

[ ]  Boost pumps .... Off

[ ]  Minimum electrics .... Set

[ ]  Transponder .... Standby

Shut Down

[ ]  Parking brake .... Set

[ ]  Throttles .... 1100

[ ]  ELT .... Checked

[ ]  Electrics .... Off

[ ]  Avionics master .... Off

[ ]  Battery .... Off

[ ]  Alternators .... Off

[ ]  Throttles .... Idle

[ ]  Live/dead mag .... Checked

[ ]  Mixtures .... Idle cut off

[ ]  Magnetos .... off

Rejected Takeoff Before Lift-off  [EMERGENCY]

[ ]  -Throttles .... closed

[ ]  -Braking .... maximum

[ ]  -Stop on remaining runway

NOTE: If there is insufficient runway to stop:

[ ]  -Switches (master and alternators) .... off

[ ]  -Fuel Selectors .... off

Rejected Takeoff - Airborne  [EMERGENCY]

[ ]  -Throttles .... as required, then off

[ ]  -Braking .... maximum

[ ]  -Stop on remaining runway

NOTE: If there is insufficient runway to land and stop:

[ ]  -Switches (master and alternators) .... off

[ ]  -Fuel Selectors .... off

Engine Failure - Continuing Takeoff  [EMERGENCY]

[ ]  -Control .... Yaw and roll.

[ ]  -Attitude .... to control airspeed

[ ]  -Power .... full

[ ]  -Drag .... retract flaps and gear

[ ]  -Identify .... retard throttle on suspected engine

[ ]  -Feather .... propeller on failed engine

[ ]  -Mixture .... ico on failed engine

[ ]  -Climb .... at best S/E climb speed

[ ]  -Trim .... rudder, elevator, ailerons

[ ]  Complete Failed Engine Shutdown Checklist

Engine Failure ..Cruise  [EMERGENCY]

[ ]  -Control .... Adverse yaw and roll

[ ]  -Attitude .... altitude... (airspeed above Vyse)

[ ]  -Power .... to climb power

[ ]  -Drag .... confirm flaps and gear retracted

[ ]  -Identify .... retard throttle on suspected engine

[ ]  -Check for Fire .... if no fire

[ ]  -Cause .... check fuel pressure

[ ]  boost pumps on

[ ]  check fuel quantity

[ ]  switch fuel tank as req=d

[ ]  alternate air both on

[ ]  magnetos on

[ ]  -Test .... advance throttle on affected engine.

NOTE: If engine does not re-start --- 9 to 10

[ ]  -Feather .... prop control to feather detent

[ ]  -Mixture .... Idle cut-off

[ ]  Complete Failed Engine Shutdown Checklist

Failed Engine Shutdown Checklist  [EMERGENCY]

[ ]  Throttle .... closed on failed engine

[ ]  Prop control .... feather detent

[ ]  Mixture control .... idle cut-off

[ ]  Drag .... confirm flaps and gear as req=d

[ ]  Cowl flap .... closed

[ ]  Boost pump .... off

[ ]  Magnetos .... off (one at a time)

[ ]  Alternator .... off

[ ]  Fuel selector .... off

[ ]  Reduce electrical load

[ ]  Monitor condition of remaining engine

[ ]  Declare an emergency

[ ]  Land at nearest suitable aerodrome

Precautionary Engine Shutdown  [ABNORMAL]

[ ]  Throttle .... closed on failed engine

[ ]  Prop control .... feather detent

[ ]  Mixture control .... idle cut-off

[ ]  Drag .... confirm flaps and gear as req=d

[ ]  Cowl flap .... closed

[ ]  Boost pump .... off

[ ]  Magnetos .... off (one at a time)

[ ]  Alternator .... off

[ ]  Fuel selector .... off

[ ]  Reduce electrical load

[ ]  Monitor condition of remaining engine

[ ]  Declare an emergency

[ ]  Land at the nearest suitable aerodrome

Engine Restart In Flight  [ABNORMAL]

[ ]  Fuel Selector .... on main or aux.

[ ]  Magnetos .... on

[ ]  Boost pump .... on

[ ]  Mixture control .... idle cut-off

[ ]  Throttle .... open 2 inch

[ ]  Propeller control .... move  into governing range

[ ]  Once engine is wind-milling, advance mixture to the full rich position.

[ ]  Warm up engine at 2000 RPM and 15" MAP

[ ]  Alternator .... on

[ ]  All cockpit switches and controls normal.

Engine Fire on Start-up  [EMERGENCY]

[ ]  -Mixture .... idle cut-off

[ ]  -Throttle .... full open

[ ]  -Cranking .... continue

NOTE: If fire continues longer than a few seconds

[ ]  -Fuel selectors .... both off

[ ]  -Mixture controls .... both idle cut-off

[ ]  -Switches .... off

[ ]  -Evacuate the aircraft

NOTE: Extinguish the fire with ground equipment if practical.

Engine Fire in Flight  [EMERGENCY]

[ ]  -Fuel selector .... off

[ ]  -Mixture .... idle cut-off

[ ]  -Propeller .... feather

[ ]  Boost pump .... off

[ ]  Magneto switch .... off

[ ]  Alternator .... off

NOTE: If fire continues, consider increasing airspeed in an attempt to extinguish it.

[ ]  Declare an emergency and indicate intentions.

[ ]  Expedite your approach to the nearest suitable landing area.

Cabin Fire  [EMERGENCY]

[ ]  -Vents .... close

[ ]  -Extinguisher .... activate if necessary.

NOTE: (Get assistance from passengers if needed).

[ ]  -Ventilate the cabin

[ ]  Land as soon as practicable.

[ ]  Inspect for damage.

Landing Gear Manual Extension  [ABNORMAL]

[ ]  Reduce speed to 120 KIAS or less.

[ ]  Landing gear motor circuit breaker .... pulled

[ ]  Landing gear selector .... down

[ ]  Engage the hand- crank and turn counter-clockwise as far as possible (approx. 50 turns).

[ ]  Check mechanical indicator to ascertain that the gear is down.

[ ]  If electrical system is operative, check gear position light and warning horn.

[ ]  Disengage hand-crank.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Beechcraft/95 Travelair/Beech95-checklist_selkirk_college.txt

Rights: licensed

Checklist hash: c5e4d938277dca0fe94e74177295d3c840995208ffe214c5298c3f8407faaba5

Verify against the aircraft's own approved documentation before use.