Beechcraft Baron 58 — Normal Operating Procedures

NOT FOR FLIGHT — source fidelity: not_applicable | operational review: none | completeness: normal_only | 1 unresolved safety defect(s)

safety_defect: Contains no airspeeds, power settings, fuel quantities, or other numerical limitations — all must come from the specific aircraft POH.

content_gap: Equipment variations exist across Baron 58 airframes. Verify every item against the specific aircraft and its POH before use.

Preflight Walk-Around

[ ]  Aircraft documents (ARROW) .... ON BOARD AND CURRENT

[ ]  Weight and balance — computed for planned load .... WITHIN LIMITS PER POH

[ ]  Fuel quantity — visually inspect both tanks each wing; compare with fuel gauges .... VERIFIED; ADEQUATE FOR FLIGHT PLUS REQUIRED RESERVES

[ ]  Fuel grade and appearance — sample all sump drains including nacelle sumps on both engines .... CORRECT GRADE PER POH; CLEAR, NO WATER OR SEDIMENT

[ ]  Fuel caps — all filler ports .... SEATED AND LOCKED SECURELY

[ ]  Left engine oil level — dipstick pulled, checked, cap secured .... WITHIN OPERATING RANGE PER POH; CAP SECURED

[ ]  Left engine compartment — fuel injection lines, oil lines, induction ducting, and wiring .... NO LEAKS, CHAFING, OR LOOSE CONNECTIONS

[ ]  Left engine induction air inlet and air filter .... UNOBSTRUCTED; FILTER SERVICEABLE

[ ]  Left engine exhaust stacks and heat muffs .... NO CRACKS, LOOSE CLAMPS, OR HEAVY CARBON DEPOSITS

[ ]  Left engine cowling — all fasteners .... CLOSED AND LATCHED

[ ]  Left propeller — both blades; leading edges, tips, and faces for nicks, cracks, or erosion .... SERVICEABLE

[ ]  Left propeller spinner and back plate .... SECURE; NO CRACKS OR LOOSE SCREWS

[ ]  Left main gear — strut extension, torque links, tire condition and inflation, brake assembly, and gear doors .... STRUT NORMAL; TIRE SERVICEABLE; BRAKES ADEQUATE; DOORS UNDAMAGED

[ ]  Left wing leading edge and upper/lower skin .... NO DEFORMATION, FUEL STAINING, OR IMPACT DAMAGE

[ ]  Left fuel filler cap and wing vent .... CAP LOCKED; VENT CLEAR

[ ]  Left aileron — hinges, control rod, balance weight, and travel .... SECURE; FULL AND FREE MOVEMENT

[ ]  Left flap — hinge pins, actuator linkage, and condition .... SECURE; NO DAMAGE OR PLAY

[ ]  Left wingtip — nav light and strobe lens .... UNDAMAGED; LENS INTACT

[ ]  Nose gear — strut extension, torque links, shimmy damper, and nose tire .... STRUT INFLATED NORMALLY; TIRE SERVICEABLE; HARDWARE SECURE

[ ]  Nose gear doors and retraction linkage visible through wheel well .... NO DAMAGE; LINKAGE INTACT

[ ]  Pitot tube — cover removed, inlet clear of obstructions and insects .... COVER REMOVED; UNOBSTRUCTED

[ ]  Static ports — fuselage ports on both sides .... CLEAR AND UNOBSTRUCTED

[ ]  Fuselage — skin condition, antennas, ELT, cabin entry door latched, pilot door seals .... NO DAMAGE; ANTENNAS SECURE; DOORS LATCHED

NOTE: The Baron 58 has a separate aft cabin entry door on the right side — inspect its latch and seal as well as the forward pilot door.

[ ]  Horizontal stabilizer, elevator, and trim tab — both sides .... UNDAMAGED; HINGES SECURE; NO EXCESSIVE PLAY

[ ]  Vertical stabilizer and rudder — hinges and travel .... UNDAMAGED; FREE MOVEMENT

[ ]  Tail lighting — nav and beacon .... LENS INTACT AND SECURE

[ ]  ELT — armed switch position and antenna connection .... ARMED; ANTENNA CONNECTED

[ ]  Right main gear — strut, tire, brakes, and gear doors .... STRUT NORMAL; TIRE SERVICEABLE; BRAKES ADEQUATE; DOORS UNDAMAGED

[ ]  Right flap and right aileron — condition and full travel .... SECURE; FULL AND FREE MOVEMENT; NO DAMAGE

[ ]  Right wing leading edge, fuel cap, and vent .... NO DAMAGE; CAP LOCKED; VENT CLEAR

[ ]  Right wingtip — nav light and strobe lens .... UNDAMAGED; LENS INTACT

[ ]  Right engine oil level — dipstick pulled, checked, cap secured .... WITHIN OPERATING RANGE PER POH; CAP SECURED

[ ]  Right engine compartment — fuel injection lines, oil lines, induction ducting, and wiring .... NO LEAKS, CHAFING, OR LOOSE CONNECTIONS

[ ]  Right engine induction air inlet and air filter .... UNOBSTRUCTED; FILTER SERVICEABLE

[ ]  Right engine exhaust stacks and heat muffs .... NO CRACKS, LOOSE CLAMPS, OR HEAVY CARBON DEPOSITS

[ ]  Right engine cowling — all fasteners .... CLOSED AND LATCHED

[ ]  Right propeller — both blades; leading edges, tips, and faces .... SERVICEABLE

[ ]  Right propeller spinner and back plate .... SECURE; NO CRACKS OR LOOSE SCREWS

[ ]  Tiedowns, chocks, and any external gust locks .... ALL REMOVED

Cockpit Preparation / Before Start

[ ]  POH — aboard and accessible to crew .... ON BOARD

[ ]  Seats — position adjusted, seat-back locked; seat belts and shoulder harnesses for all occupants .... ADJUSTED, LOCKED, AND FASTENED

[ ]  Control lock .... REMOVED

[ ]  Flight controls — full travel check for freedom of movement .... FREE AND CORRECT

[ ]  Avionics master switch .... OFF (prior to applying battery power)

[ ]  Master switch (battery) .... ON

[ ]  Fuel quantity gauges — both engines .... CHECK; CONSISTENT WITH VISUAL INSPECTION

[ ]  Annunciator panel — press-to-test .... ALL LAMPS ILLUMINATE ON TEST; NONE REMAIN LIT

[ ]  Landing gear position indicators .... THREE GREEN; DOWN-AND-LOCKED CONFIRMED

[ ]  Gear warning horn — throttles retarded, confirm audible warning .... HORN SOUNDS

[ ]  Left fuel selector .... LEFT TANK (or fullest tank per POH; NOT OFF)

[ ]  Right fuel selector .... RIGHT TANK (or fullest tank per POH; NOT OFF)

[ ]  Elevator trim .... SET FOR TAKEOFF PER POH

[ ]  Rudder trim .... SET NEUTRAL OR PER POH

[ ]  Aileron trim .... SET NEUTRAL OR PER POH

[ ]  Flaps .... UP — confirmed on position indicator

[ ]  Propeller controls — both .... FULL FORWARD (high RPM)

[ ]  Mixture controls — both .... RICH (or as required for field elevation per POH)

[ ]  Throttles — both .... CLOSED; THEN CRACKED SLIGHTLY OPEN

[ ]  Parking brake .... SET

Engine Start

[ ]  Area around aircraft — clear of persons and obstructions on both sides .... CLEAR

CAUTION: The IO-550 engines are fuel-injected. Use the hot-start and cold-start procedures from the POH — priming with the boost pump followed by a specific throttle position differs between hot and cold conditions.

[ ]  Left fuel selector .... LEFT TANK — CONFIRMED ON

[ ]  Left mixture .... RICH (or as required per POH)

[ ]  Left boost pump — energise briefly to prime per POH cold-start procedure .... ON MOMENTARILY; FUEL FLOW INDICATED; THEN OFF

[ ]  Left throttle .... CRACKED OPEN PER POH START SETTING

[ ]  Left ignition switch .... START — engage starter

[ ]  Left oil pressure — confirm pressure rise after start .... RISING; WITHIN LIMITS PER POH

[ ]  Left throttle — after engine fires .... REDUCE TO IDLE PER POH

[ ]  Right fuel selector .... RIGHT TANK — CONFIRMED ON

[ ]  Right mixture .... RICH (or as required per POH)

[ ]  Right boost pump — energise briefly to prime per POH cold-start procedure .... ON MOMENTARILY; FUEL FLOW INDICATED; THEN OFF

[ ]  Right throttle .... CRACKED OPEN PER POH START SETTING

[ ]  Right ignition switch .... START — engage starter

[ ]  Right oil pressure — confirm pressure rise after start .... RISING; WITHIN LIMITS PER POH

[ ]  Right throttle — after engine fires .... REDUCE TO IDLE PER POH

[ ]  Avionics master switch .... ON

[ ]  Alternators — both; confirm charging indication .... BOTH CHARGING; LOADMETERS INDICATING NORMALLY

After Start / Before Taxi

[ ]  Oil pressures and oil temperatures — both engines .... WITHIN NORMAL RANGE PER POH

[ ]  Engine instruments — vacuum, fuel flow, CHT, both engines .... ALL INDICATING NORMALLY

[ ]  Annunciator panel — confirm no warning lights remain .... CLEAR

[ ]  Flight instruments — altimeter, heading indicator, attitude indicator, VSI .... ALTIMETER SET; GYROS ERECTING; INSTRUMENTS ALIVE

[ ]  Heading indicator — aligned with magnetic compass .... ALIGNED

[ ]  Navigation and communication radios — all sets .... FREQUENCIES SET; VOLUMES ADJUSTED

[ ]  Transponder .... CODE SET; IN STANDBY

[ ]  Taxi and position lights as required .... ON

[ ]  Parking brake .... RELEASE WHEN READY TO TAXI

Run-Up / Before Takeoff

[ ]  Parking brake .... SET

[ ]  Flight controls — full travel sweep .... FREE AND CORRECT

[ ]  Trim tabs — elevator, rudder, and aileron .... SET FOR TAKEOFF PER POH

[ ]  Left fuel selector .... LEFT TANK OR PER POH; CONFIRM NOT OFF

[ ]  Right fuel selector .... RIGHT TANK OR PER POH; CONFIRM NOT OFF

[ ]  Mixtures — both engines .... RICH (or as required for field elevation per POH)

[ ]  Engine run-up — advance both throttles together to run-up power per POH .... RPM STABILISED AT RUN-UP SETTING ON BOTH ENGINES

[ ]  Left magneto check — select LEFT, then RIGHT, then BOTH .... RPM DROP ON EACH MAG WITHIN LIMITS PER POH; SPREAD ACCEPTABLE

[ ]  Right magneto check — select LEFT, then RIGHT, then BOTH .... RPM DROP ON EACH MAG WITHIN LIMITS PER POH; SPREAD ACCEPTABLE

[ ]  Left propeller cycle — advance to full forward, reduce RPM to observe drop, advance back to full forward .... RPM DROP NOTED; OIL CIRCULATING; CONTROL RETURNS SMOOTHLY

[ ]  Right propeller cycle — advance to full forward, reduce RPM to observe drop, advance back to full forward .... RPM DROP NOTED; OIL CIRCULATING; CONTROL RETURNS SMOOTHLY

[ ]  Propeller controls — after cycling, both .... FULL FORWARD (high RPM) FOR TAKEOFF

[ ]  Engine instruments at run-up power — both engines .... OIL PRESSURE, OIL TEMP, FUEL FLOW, AND CHT ALL WITHIN LIMITS

[ ]  Annunciator panel .... NO AMBER OR RED LIGHTS

[ ]  Throttles — reduce to idle after run-up .... SMOOTH IDLE ON BOTH ENGINES

[ ]  Landing gear handle — confirm DOWN position .... DOWN; THREE GREEN INDICATORS

[ ]  Flaps — set for takeoff .... TAKEOFF POSITION PER POH; INDICATOR CONFIRMED

[ ]  Doors — pilot door and cabin entry door .... LATCHED AND SECURED

[ ]  Seat belts and shoulder harnesses — all occupants .... FASTENED

[ ]  Boost pumps — both, as required for takeoff per POH .... ON OR OFF PER POH

[ ]  Takeoff briefing — engine failure procedure, abort point, Vmc and Vyse review .... COMPLETED

Takeoff

[ ]  Strobes and landing lights .... ON

[ ]  Transponder .... ALT (or ON for non-Mode-C)

[ ]  Throttles — advance smoothly to full takeoff power, both engines together .... FULL POWER; POWER SYMMETRIC

[ ]  Engine instruments — brief crosscheck during roll .... MANIFOLD PRESSURE, RPM, AND FUEL FLOW NORMAL BOTH ENGINES

[ ]  Rotation — at rotation speed per POH .... ROTATE SMOOTHLY

[ ]  Landing gear — after positive rate of climb established .... GEAR UP; THREE GREEN EXTINGUISHED; GEAR-IN-TRANSIT LIGHT OUT

[ ]  Airspeed — climb at or above Vmc; target Vy or Vx per POH .... POSITIVE CLIMB CONFIRMED

Climb

[ ]  Airspeed — establish climb speed per POH .... ON SPEED PER POH

[ ]  Power — set climb power per POH (manifold pressure and RPM) .... CLIMB POWER SET BOTH ENGINES

[ ]  Mixtures — adjust per POH for altitude and climb power .... ADJUSTED PER POH

[ ]  Boost pumps — off when clear of traffic area per POH .... OFF PER POH

[ ]  Flaps — retract on schedule per POH if takeoff flaps used .... UP; CONFIRMED ON INDICATOR

[ ]  Engine instruments — monitor both engines throughout climb .... OIL PRESSURE, OIL TEMP, CHT, AND FUEL FLOW WITHIN LIMITS

[ ]  Fuel balance — check both tank gauges; consider switching tanks if imbalance developing .... WITHIN BALANCE LIMITS PER POH

[ ]  Altimeter — set to current altimeter setting; transition to 29.92 at transition altitude as applicable .... SET

Cruise

[ ]  Power — set cruise power per POH (manifold pressure then RPM) .... CRUISE POWER SET; BOTH ENGINES MATCHED

[ ]  Mixtures — lean per POH for cruise altitude and power setting .... LEANED TO PEAK EGT OR PER POH PROCEDURE

[ ]  Fuel selectors — confirm tank selection per POH fuel management schedule .... CORRECT TANKS SELECTED; NEITHER SELECTOR IN OFF

[ ]  Engine instruments — both engines; oil pressure, oil temp, fuel flow, CHT, EGT .... ALL WITHIN NORMAL CRUISE LIMITS PER POH

[ ]  Fuel quantities — both tanks; compute remaining endurance and compare with plan .... FUEL BURN CONSISTENT WITH PLANNED CONSUMPTION

[ ]  Alternators and electrical load — both bus voltages and load indicators .... BOTH ALTERNATORS CHARGING; LOADS BALANCED

[ ]  Vacuum / suction gauge — confirm pressure within operating range .... WITHIN LIMITS PER POH

[ ]  Autopilot — engage if desired after aircraft is in trim .... ENGAGED OR HAND FLYING

[ ]  Outside air temperature — monitor for icing conditions .... NOTED; NO ICING CONDITIONS INDICATED

Descent

[ ]  Fuel selectors — both engines; confirm active tanks have adequate fuel for approach and alternate .... TANK SELECTION CONFIRMED; FUEL ADEQUATE

[ ]  Mixtures — enrich per POH as altitude decreases .... ADJUSTED FOR ALTITUDE PER POH

[ ]  Power — reduce gradually; avoid shock-cooling by keeping power changes gentle .... DESCENT POWER SET PER POH

[ ]  Engine instruments — confirm CHT not falling below limits during descent .... CHT WITHIN RANGE PER POH

[ ]  Altimeter — set to current destination altimeter setting .... SET

[ ]  ATIS / destination weather — obtain and brief .... OBTAINED; RUNWAY AND APPROACH TYPE NOTED

[ ]  Avionics — communications and navigation frequencies set for arrival .... SET

Before Landing

[ ]  Fuel selectors — both engines; fullest available tank or per POH .... CORRECT TANKS CONFIRMED; NOT OFF

[ ]  Mixtures — both engines; rich for landing .... RICH (or adjusted for field elevation per POH)

[ ]  Propellers — both controls full forward for maximum RPM availability .... FULL FORWARD

[ ]  Boost pumps — both; on for approach per POH .... ON PER POH

[ ]  Landing gear — DOWN at or below gear extension speed per POH .... GEAR DOWN; THREE GREEN; HORN SILENT

[ ]  Flaps — extend to landing position at or below flap extension speed per POH .... FLAPS SET PER POH; INDICATOR CONFIRMED

[ ]  Seat belts and shoulder harnesses — all occupants .... FASTENED AND SNUG

[ ]  Landing lights .... ON

Gear Check — Final

[ ]  Landing gear handle position .... DOWN

[ ]  Three green gear-down indicator lights .... ALL THREE GREEN ILLUMINATED

[ ]  Gear warning horn — confirm silent at approach power; if sounding, verify gear handle is down .... SILENT (GEAR DOWN AND LOCKED)

Landing

[ ]  Final approach — stabilised on speed per POH .... ON SPEED AND GLIDEPATH

[ ]  Gear — confirm three green on short final .... THREE GREEN

[ ]  Flares and touchdown — round out over the runway threshold; land on main gear first .... SMOOTH TOUCHDOWN

[ ]  Throttles — retard to idle at touchdown .... CLOSED

[ ]  Brakes — apply smoothly after mains are firmly on the runway .... APPLIED AS NEEDED

[ ]  Directional control — maintain centreline with rudder; use nosewheel steering once speed permits .... CONTROLLED

Go-Around / Missed Approach

[ ]  Throttles — advance smoothly to full takeoff power on both engines simultaneously .... FULL POWER

[ ]  Pitch — establish a climb attitude; airspeed at or above Vmc immediately .... POSITIVE CLIMB ATTITUDE; AIRSPEED ABOVE VMC

[ ]  Flaps — retract to takeoff position on first increment per POH .... TAKEOFF FLAP SETTING

[ ]  Landing gear — retract after positive rate of climb established .... GEAR UP; IN TRANSIT CONFIRMED

[ ]  Flaps — retract fully after obstacle clearance and positive climb confirmed .... UP

[ ]  Airspeed — accelerate to Vy per POH once clear of obstacles .... VY PER POH

[ ]  ATC — advise of go-around and intentions .... NOTIFIED

After Landing

[ ]  Flaps .... RETRACT — UP

[ ]  Boost pumps — both .... OFF (unless POH specifies otherwise)

[ ]  Transponder .... STANDBY OR AS APPROPRIATE

[ ]  Landing lights .... OFF OR TAXI SETTING AS REQUIRED

[ ]  Mixtures — both engines; adjust to rich for taxi as appropriate .... RICH FOR TAXI (OR PER POH)

[ ]  Brakes — test gently during taxi clear of runway .... EFFECTIVE

Engine Shutdown and Securing

[ ]  Parking brake .... SET

[ ]  Avionics master switch .... OFF

[ ]  Electrical loads — non-essential switches .... OFF

[ ]  Engine cooling — run both engines at idle for a brief cooling period per POH before shutdown .... COOL-DOWN PERIOD COMPLETE

[ ]  Left mixture — pull to idle cutoff to stop left engine .... IDLE CUTOFF; ENGINE STOPS

[ ]  Left ignition switch — off after engine stops .... OFF

[ ]  Right mixture — pull to idle cutoff to stop right engine .... IDLE CUTOFF; ENGINE STOPS

[ ]  Right ignition switch — off after engine stops .... OFF

[ ]  Master switch .... OFF

[ ]  Fuel selectors — both .... OFF

[ ]  Control lock .... INSTALLED

[ ]  Doors — pilot door and cabin entry door .... SECURED; WINDOWS CLOSED

[ ]  Tiedowns and chocks .... INSTALLED

[ ]  Pitot tube cover .... INSTALLED

NOT FOR FLIGHT — source fidelity: not_applicable | operational review: none | completeness: normal_only | 1 unresolved safety defect(s)

Source: authored — no source document

Rights: original_expression CC-BY-4.0

Checklist hash: 754659d9dbe88dd62ff72c07e9d20f780861b46a6629f4e64b7402862cad3d9a

Verify against the aircraft's own approved documentation before use.