Beechcraft BE-55 _ BE-58 Baron — Beech_Baron_58_Study_Guide

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Instruments" item count differs between passes (13 vs 11); verify nothing was dropped.

content_gap: Section "V-Speeds" item count differs between passes (25 vs 21); verify nothing was dropped.

content_gap: Section "Weight/Balance" item count differs between passes (9 vs 8); verify nothing was dropped.

Engines

NOTE: IO-520-CB (285 HP @ 2700 RPM)

NOTE: (I) Fuel-injected (O) Horizontally Opposed (520) displacement (CB) specific model #

NOTE: Starter: 24V direct gear drive

NOTE: Wet sump, pressure type oil system with capacity of 12 quarts (SAE 50)

NOTE: Ger driven oil pump  powered by accessory drive with output of 30-60 PSI

NOTE: Magnetos: no drop more than 150 RPM or more than 50 RPM between both magnetos

NOTE: Filtered ram air from top air scoops, if clogged door opens providing alternate air from nacelle

Electrical System

NOTE: 28V System with 5.5 amps, 24V lead acid battery (or 2 25 amp 12V batteries)

NOTE: 28V Alternators (2) of 50-60 amps, separate from system, except tied Main Batt Bus

NOTE: Two voltage regulators, only one functioning at a time (controlled through switch on subpanel)

NOTE: ALT OUT light on if voltage drops 1.5-2.0V below bus voltage or if over-voltage relay actuated

Fuel System

NOTE: Wet wing tip with 200 gal total, 194 usable (drained at 8 locations)

NOTE: Aux Pumps between fuel selector & eng-driven pump, fed from inboard leading-edge tanks

NOTE: Three bladder-type fuel cells in each wing

NOTE: Fuel pump driven off accessory drive of engine, fuel enters pump at 28-31 PSI

Landing Gear/Brakes

NOTE: 28V DC split-field, series wound motor; Drives worm gear

NOTE: Left Gear strut houses auto safety switch and gear warning horn switch (squat switch?)

NOTE: Gear up and throttle below 15” MAP

NOTE: Gear handle in UP position while on ground

NOTE: Gear up with more than 15° Flaps used

NOTE: Air-oil struts on landing gear

NOTE: Main tires: 70 PSI; Nosewheel tire: 50 PSI

Flight Controls

NOTE: Flaps: electrically driven by motor under cabin floor; trailing edge fowler flaps (single-slotted)

NOTE: Flap position transmitter on flap actuator in left wing, forward of rear spar.

NOTE: Ailerons: cables routed through bellcranks and pulleys

NOTE: Aileron Trim: on left aileron; push-pull rod attached to jackscrew-type actuator

NOTE: Elevator: connected to cables to a torque tube which is connected to a bellcrank in tail

NOTE: Elevator Trim: located on left and right elevators; push-pull rods attached to jackscrew actuators

NOTE: Rudder: bellcrank attached to bottom of rudder and cables

NOTE: Rudder Trim: actuated by push-pull rods attached to a jackscrew-type actuator

Instruments

NOTE: Manifold Pressure:     green = 15”-29.6”     red line @ 29.6”

NOTE: Tachometer:      green = 2000-2650 RPM red line @ 2700 RPM

NOTE: Fuel Flow: green = 6.9-24.3 GPH        white arc = 17.8-24.3 GPH red line @ 24.3 GPH

NOTE: Oil Temperature: yellow @ 75°          green  = 75-240°F      red line @ 240°F

NOTE: Oil Pressure:    min. red line @ 30 PSI      green = 30-100 PSI max red line @ 100

NOTE: CHT:  Indicates hottest cylinder in engine         green  = 200-460°F red line @ 460°F

NOTE: EGT:  Less than 75% power cruise displays relative scale value, not actual temps

NOTE: No static system drain due to location of plumbing and components

NOTE: Instrument air and DEICE BOOTS provided by 2 eng-driven dry pressure pumps

NOTE: Automatic check valves close if one pump fails, 2 air filters per engine

NOTE: Pitot System                      Static System
Electrical  Pressure
      Airspeed Indicator
                                   Altimeter
                                   VSI
                                                         Turn Coord
                                                                    Attitude Gyro
                                                                    DG

Weight/Balance

NOTE: Max Ramp: 5424

NOTE: MGTOW: 5400

NOTE: Max Landing: 5400

NOTE: 5400 lb CG limit:           78.0 – 86.0 (datum line is 83.1 forward of front jack point)

NOTE: 4200 or less CG limits:     74.0 – 86.0

NOTE: Nose Compartment:      300 lbs MAX

NOTE: Aft Cabin:       400 lbs MAX

NOTE: Rear Compartment:      120 lbs MAX

Emergencies

Other Limitations

NOTE: Load Factors:    Flaps up (0°) = 4.2G

NOTE: Flaps down (30°) = 2.0G

V-Speeds

NOTE: Va:        156

NOTE: Vfe:       152/122

NOTE: Vle/Vlo:   152

NOTE: Vmca:      81

NOTE: Vne:       223

NOTE: Vno:       195

NOTE: Vs:

NOTE: Vso:

NOTE: Vsse:      86

NOTE: Vy:        120

NOTE: Vx:

NOTE: Vyse:       100 (also min single-engine landing speed)

NOTE: Vxse:      96

NOTE: Emer Descent:    152

NOTE: Red Radial:            223

NOTE: White Arc:       74 – 122

NOTE: Green Arc:       84 – 195

NOTE: Blue Radial:           120

NOTE: Yellow Arc:            195 – 223

NOTE: Red Radial:            81

NOTE: White Triangle:  152

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Beechcraft/BE-55 _ BE-58 Baron/Beech_Baron_58_Study_Guide.txt

Rights: licensed

Checklist hash: e6edbc9018ef6be907ad386a66c053cc1a1fe4dafdca387415b1fc32dd21a490

Verify against the aircraft's own approved documentation before use.