Beechcraft BE-76 Duchess — TFA_Beechcraft_Duchess_Checklist_2004-02

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Preflight Inspection

1. FLIGHT DECK

[ ]  Certificates & Documents .... CHECK

[ ]  MX Requirements .... CHECK

[ ]  Control Lock .... REMOVE & STOW

[ ]  Parking Brake .... SET

[ ]  All Switches .... OFF

[ ]  Propellers .... CLEAR

[ ]  Battery Master .... ON

[ ]  Fuel Quantity .... CHECK

[ ]  Flaps .... EXTEND (verify handle)

[ ]  Battery Master .... OFF

[ ]  Trim Tabs .... SET TO ZERO

[ ]  Emergency Gear Extension Valve .... VERTICLE (Closed!)

[ ]  Hood .... CHECK IN AIRPLANE

[ ]  Fuel Drain Tool .... OBTAIN

2. LEFT WING TRAILING EDGE

[ ]  Flap .... CHECK CONDITION

[ ]  Fuel Vent .... UNOBSTRUCTED

[ ]  Aileron .... CHECK CONDITION & FREEDOM OF MOVEMENT

[ ]  Wing Tip .... CHECK

[ ]  Position & Strobe Lights .... CHECK

3. LEFT WING LEADING EDGE

[ ]  Pitot Tube .... UNOBSTRUCTED

[ ]  Landing & Taxi Lights .... CHECK

[ ]  Stall Warn Vane .... CHECK FREEDOM OF MOVEMENT

[ ]  Fuel Tank .... CHECK QUANTITY

[ ]  Tiedown & Chocks .... REMOVE

[ ]  Fuel Tank Sump .... DRAIN

[ ]  Fuel Selector .... DRAIN

[ ]  Engine Cowling .... CHECK CONDITION

[ ]  Air Intakes .... CLEAR

[ ]  Propeller .... EXAMINE

[ ]  Engine Oil .... CHECK QUANTITY

[ ]  Cowl Flap .... CHECK

[ ]  Main Gear .... CHECK

[ ]  Crossfeed Lines .... DRAIN

4. NOSE SECTION

[ ]  Nose Cowling .... CHECK

[ ]  Heater Intake .... CLEAR

[ ]  Heater Exhaust .... CLEAR

[ ]  Nose Gear .... CHECK

[ ]  Nose Gear Chock .... REMOVE

5. RIGHT WING LEADING EDGE

[ ]  Crossfeed Lines .... DRAIN

[ ]  Main Gear .... CHECK

[ ]  Engine Cowling .... CHECK CONDITION

[ ]  Air Intakes .... CLEAR

[ ]  Propeller .... EXAMINE

[ ]  Engine Oil .... CHECK QUANTITY

[ ]  Cowl Flap .... CHECK

[ ]  Fuel Tank Sump .... DRAIN

[ ]  Tiedown & Chocks .... REMOVE

[ ]  Fuel Tank .... CHECK QUANTITY

[ ]  Stall Warn Vane .... CHECK FREEDOM OF MOVEMENT

[ ]  Taxi Light .... CHECK

[ ]  Wing Tip .... CHECK

[ ]  Position & Strobe Lights .... CHECK

6. RIGHT WING TRAILING EDGE

[ ]  Aileron .... CHECK CONDITION & FREEDOM OF MOVEMENT

[ ]  Fuel Vent .... UNOBSTRUCTED

[ ]  Flap .... CHECK CONDITION

7. FUSELAGE RIGHT SIDE

[ ]  Battery Vents .... CHECK CLEAR

[ ]  Static Port .... CHECK CLEAR

[ ]  ELT .... CHECK ARMED

8. EMPENNAGE

[ ]  Control Surface & Trim .... CHECK

[ ]  Tail Cone & Position Lt .... CHECK

[ ]  Tiedown .... REMOVE

[ ]  Cabin Air Inlet .... CHECK

9. FUSELAGE LEFT SIDE

[ ]  Static Port .... CHECK CLEAR

[ ]  Cabin Air Outlet .... CHECK

[ ]  All Antennas .... CHECK

[ ]  Load Dist. .... CHECK/SECURE

[ ]  Baggage Door .... CHECK/SECURE

Speeds & Limitations

NOTE: Rotation – 71 KIAS

NOTE: 50-ft Speed – 80 KIAS

NOTE: Vx – 71 KIAS

NOTE: Vy – 85 KIAS

NOTE: Vxse – 85 KIAS

NOTE: Vyse – 85 KIAS

NOTE: Vsse – 71 KIAS

NOTE: Cruise Climb – 100 KIAS

NOTE: Final Approach Flaps Down – 76 KIAS

NOTE: Final Approach Flaps Up – 87 KIAS

NOTE: One Engine App Flaps Down – 85 KIAS

NOTE: One Engine App Flaps Up – 90 KIAS

NOTE: Best Glide – 95 KIAS

NOTE: Emergency Descent – 140 KIAS

Before Starting

[ ]  Emergency Ext Tool .... STOW

[ ]  Seats .... POSITION & LOCK

[ ]  Seatbelts/Shoulder Harness. .... FASTEN

[ ]  Parking Brake .... SET

[ ]  Avionics .... OFF

[ ]  Circuit Breakers .... IN

[ ]  Gear Handle .... DOWN

[ ]  Carburetor Heat .... OFF

[ ]  Cowl Flaps .... OPEN

[ ]  Fuel Selectors .... CHECK OPERATION, THE ON

[ ]  Light Switches .... OFF

[ ]  Battery/Alt. Switches .... ON

[ ]  Fuel Quantity Ind. .... CHECK

[ ]  Landing Gear Lights .... CHECK

Starting Engines Using External Power

[ ]  Battery Switch .... ON

[ ]  Alternators/Electrical Equip. .... OFF

[ ]  Aux Power Unit .... CONNECT

[ ]  Aux Power Unit .... SET OUTPUT 13.5 – 14.25 VOLTS

[ ]  Aux Power Unit .... ON

[ ]  Left Engine .... START

[ ]  Aux Power Unit .... OFF

[ ]  Aux Power Unit .... DISCONNECT

[ ]  Alternator Switches .... ON

Normal Starting Procedures

[ ]  Battery Switch .... ON

[ ]  Udervoltage Lights .... ILLUMINATED

[ ]  Mixture .... FULL RICH

[ ]  Propeller .... HIGH RPM

[ ]  Throttle .... ¼ TRAVEL

[ ]  Aux Fuel Pump .... ON

[ ]  Magneto Switch .... PUSH TO PRIME START POSITION (3 sec)

[ ]  Starter .... ENGAGE

[ ]  Throttle .... SET 1000 RPM

[ ]  Oil Pressure .... CHECK

[ ]  Use Above Procedure to Start Other Engine!

[ ]  Left Alt/Battery Switch .... OFF; CHECK left Alt UV light illuminates and right loadmeter picks up the load

[ ]  Left Alt/Battery Switch .... ON

[ ]  Right Alt/Battery Switch .... OFF; CHECK right Alt UV light illuminates and left loadmeter picks up the load

[ ]  Right Alt/Battery Switch .... ON

After Start

[ ]  Avionics .... ON

[ ]  Lights .... AS REQUIRED

[ ]  Annunciator Warning Lights .... PRESS-TO-TEST

[ ]  Aux Fuel Pumps .... OFF, THEN ON

[ ]  Eng. Instruments .... CHECK

[ ]  ATIS/Taxi Clearance .... OBTAIN

[ ]  Brakes .... RELEASE & CHECK

[ ]  Wing Flaps .... UP

IFR Taxi

[ ]  Nose Wheel Steering .... CHECK

[ ]  Airspeed Indicator .... 0 KIAS OR HEADWIND COMPONENT

[ ]  Attitude Indicator .... ERECT & WITH 5° IN 5 MINUTES

[ ]  Altimeter .... WITH IN 75’ OF FIELD ELEVATION

[ ]  Turn Coordinator .... IDICATES TURN AND BALL OUTSIDE

[ ]  Heading Indicator .... IDICATES TURN

[ ]  Vertical Speed Ind. .... NOTE ERROR AND USE AS 0 RATE

Before Takeoff

[ ]  Seatbelts/Shoulder Harness .... CHECK

[ ]  Parking Brake .... SET

[ ]  Radios .... CHECK/SET

[ ]  Flight Instruments .... SET

[ ]  Engine Instruments .... CHECK

[ ]  Fuel Selectors .... ON

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Wing Flaps .... CHECK OPERATION

[ ]  Trim .... SET FOR TAKEOFF

[ ]  Throttles .... 2200 RPM

[ ]  Propellers .... EXERCISE (100-200 rpm drop)

[ ]  Magnetos .... CHECK (175 rpm max drop, w/in 50 rpm differential)

[ ]  Carburetor Heat .... CHECK & OFF

[ ]  Throttles .... 1500 RPM

[ ]  Propellers .... FEATHER CHECK (500 rpm max drop) 3-4 times in cold weather

[ ]  Gyro Pressure & Loadmeters .... CHECK

[ ]  Throttles .... IDLE THEN TO 1000 RPM

[ ]  Aux Fuel Pumps .... CHECK ON

[ ]  Doors & Window .... SECURE

[ ]  Parking Brake .... RELEASE

[ ]  Engine Instruments .... CHECK

Taking the Runway

[ ]  Clearance Received .... TAKE OFF
      or POSITION & HOLD

[ ]  Approach End .... CLEAR

[ ]  Lights .... ON

[ ]  Camera .... TRANSPONDER ALT

[ ]  Action    D.G. .... ALIGN W/ RWY HEADING

[ ]  Time Off .... TAKEN

Normal Takeoff

[ ]  Elevator Control .... NUETRAL POSITION

[ ]  Mixture & Propeller Control .... VERIFY FULL FORWARD

[ ]  Brakes .... SET UNTIL FULL PWR IS OBTAINED

[ ]  Power .... SET FULL THROTTLE /2700 RPM

[ ]  Eng Instruments & ASI .... CHECK

[ ]  Rotate .... 71 KIAS

[ ]  Landing Gear .... UP AFTER POSITIVE RATE IS OBTAINED

[ ]  Airspeed .... 85 KIAS Vy

Climb

[ ]  Climb Power .... SET AFTER 500’ AGL (25”/2500 RPM)

[ ]  Airspeed .... 85 KIAS Vy, CRUISE CLIMB 100 KIAS

[ ]  Engine Instruments .... MONITOR

[ ]  Mixtures Lean .... AS REQUIRED

[ ]  Cowl Flaps .... AS REQUIRED

[ ]  Aux Fuel Pumps .... OFF AS REQUIRED

[ ]  Lights .... AS REQUIRED

Cruise

[ ]  Power .... SET (18”/2300RPM in practice area, 23”/2300RPM on x-c)

[ ]  Trim .... SET FOR LEVEL FLIGHT

[ ]  Mixtures .... LEAN AS REQUIRED

[ ]  Cowl Flaps .... AS REQUIRED

[ ]  Fuel Pumps .... OFF (one at a time)

Descent

[ ]  ATIS/Altimeter Setting .... OBTAIN

[ ]  Power .... AS REQUIRED

[ ]  Cowl Flaps .... CLOSE

[ ]  Carburetor Heat .... AS REQUIRED

[ ]  Mixtures .... ENRICH AS REQUIRED

[ ]  Lights .... AS REQUIRED

[ ]  Windshield Defroster .... AS REQUIRED

Before Landing

[ ]  Seatbelts/Shoulder Harness. .... SECURED

[ ]  Fuel Selectors .... CHECK ON

[ ]  Cowl Flaps .... AS REQUIRED

[ ]  Carburetor Heat .... CYCLE

[ ]  Mixtures .... FULL RICH

[ ]  Propellers .... FULL INCREASE

[ ]  Landing Gear .... DOWN

[ ]  Aux Fuel Pumps .... ON

[ ]  Land/Taxi Lights .... ON AS REQUIRED

[ ]  Wing Flaps .... FULL DOWN (110 KIAS Maximum)

[ ]  Airspeed .... 76 KIAS ON FINAL

Balked Landing

[ ]  Mixture/Propeller Controls .... VERIFY FULL FORWARD

[ ]  Throttles .... FULL FORWARD

[ ]  Airspeed .... 71 KIAS

[ ]  Wing Flaps .... UP

[ ]  Landing Gear .... UP

[ ]  Cowl Flaps .... OPEN

After Landing

[ ]  Land/Taxi Lights .... AS REQUIRED

[ ]  Wing Flaps .... UP

[ ]  Trim Tabs .... SET TO TAKEOFF

[ ]  Cowl Flaps .... OPEN

[ ]  Heater Switch .... BLOWER ONLY (5-minutes minimum)

[ ]  Taxi Clearance .... OBTAIN

Shutdown

[ ]  Parking Brake .... SET

[ ]  Aux Fuel Pumps .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Avionics Masters .... OFF

[ ]  Propellers .... HIGH RPM

[ ]  Throttles .... 1000 RPM

[ ]  Mixtures .... IDLE CUT-OFF

[ ]  Magnetos .... OFF

[ ]  Battery/Alt Switches .... OFF

[ ]  Controls .... LOCKED

[ ]  Hobbs Meter .... CHECK

[ ]  Airplane .... CHOCK & SECURE

Engine Failure During Ground Roll  [EMERGENCY]

[ ]  Throttles .... IDLE

[ ]  Braking .... MAXIMUM

[ ]  Fuel Selectors .... OFF

[ ]  Batt/Alt/Magnetos Switches .... OFF

Engine Failure After Lift-Off & In Flight  [EMERGENCY]

[ ]  Landing Gear & Flaps .... UP

[ ]  Inoperative Engine .... IDENTIFY

[ ]  Throttle inoperative engine .... IDLE

[ ]  Propeller inoperative engine .... FEATHER

[ ]  Operative engine .... FULL POWER

[ ]  Airspeed .... AT LEAST 80 KIAS

[ ]  After positive control of airplane is established; Secure Inoperative Engine

NOTE: If performance allows and engine instruments are normal; complete the following prior to Feather Propeller:

[ ]  Fuel Selectors .... ON

[ ]  Cowl Flaps .... OPEN ON OPERATIVE ENGINE

[ ]  Carburetor Heat .... ON BOTH ENGINES

[ ]  Aux Fuel Pumps .... ON

[ ]  Magnetos .... CHECK (try different positions)

NOTE: If above procedure did not result in a restart then:

[ ]  Propeller inoperative engine .... FEATHER

[ ]  Operative engine .... FULL POWER or as required

[ ]  Airspeed .... AT LEAST 85 KIAS

[ ]  After positive control of airplane is established; Secure Inoperative Engine

Securing Inoperative Engine  [EMERGENCY]

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Fuel Selector .... OFF

[ ]  Aux Fuel Pump .... OFF

[ ]  Magneto Switch .... OFF

[ ]  Cowl Flap .... CLOSE

[ ]  Airspeed .... ESTABLISHED 85 KIAS

[ ]  Electrical Load .... MONITOR (100% max on good alternator)

[ ]  Save Good Engine!

Precautionary Engine Shutdown  [EMERGENCY]

[ ]  Throttle .... IDLE (on inoperative engine)

[ ]  Mixture .... IDLE CUT-OFF (on inoperative engine)

[ ]  Propeller .... FEATHER

[ ]  Complete Securing Inoperative Engine Checklist

One Engine Inoperative Landing  [EMERGENCY]

ON FINAL AND LANDING ASSURED:

[ ]  Landing Gear .... DOWN

[ ]  Airspeed .... 85 KIAS

[ ]  Power .... AS REQUIRED

WHEN IT IS CERTAIN THERE IS NO POSSIBILITY OF A GO AROUND:

[ ]  Wing Flaps .... FULL DOWN

[ ]  Execute a Normal Landing

Air Start  [EMERGENCY]

[ ]  Fuel Selector .... ON

[ ]  Throttle .... SET APPROX. ¼ TRAVEL

[ ]  Aux Fuel Pump .... ON

[ ]  Magnetos .... ON

[ ]  Airspeed .... 100 KIAS

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Propeller .... FULL FORWARD until engine windmills then back to MIDRANGE, use starter if below 100 KIAS

[ ]  Mixture .... FULL RICH (if engine fails to start, mixture full lean & back to full rich when engine starts)

[ ]  Throttle .... 15”hg

[ ]  Propeller .... 2000 RPM

[ ]  Alternator Switch .... ON

[ ]  Engine Instruments .... CHECK

[ ]  Cruise Checklist – CONSULT

NOTE: ***Consult POH if Propeller Fails to Windmill***

Emergency Gear Extension  [EMERGENCY]

[ ]  Landing Gear Motor Circuit Breaker .... PULL

[ ]  Landing Gear Handle .... DOWN

[ ]  Airspeed .... 100 KIAS MAXIMUM

[ ]  Extension Valve .... OPEN (c-clockwise)

[ ]  Gear Indicator Lights .... THREE GREEN

[ ]  Extension Valve .... RETURN TO VERTICAL

WARNING: Do Not Move Gear Handle or Reset Breakers!

Gear Up Landing  [EMERGENCY]

[ ]  Cowl Flaps .... CLOSE

[ ]  Wing Flaps .... FULL DOWN

[ ]  Throttles .... IDLE

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Battery, Alternator, & Magnetos .... OFF

[ ]  Keep wings level to touchdown

[ ]  Exit airplane as soon as possible after stopping

Engine Fire On Ground  [EMERGENCY]

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Affected Engine .... CONTINUE CRANKING

[ ]  Fuel Selectors .... OFF

[ ]  Battery/Alternator Switches .... OFF

[ ]  Extinguish fire with extinguisher

Engine Fire In Flight  [EMERGENCY]

[ ]  Fuel Selector .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Propeller .... FEATHER

[ ]  Aux Fuel Pump .... OFF

[ ]  Magneto Switch .... OFF

[ ]  Alternator Switch .... OFF

Electrical Smoke or Fire  [EMERGENCY]

[ ]  Battery/Alternator Switches .... OFF

[ ]  All Electrical Switches .... OFF

[ ]  Cabin Vents and Heat .... OFF

[ ]  Fire Extinguisher .... OBTAIN & EXTINGUISH

[ ]  Cabin Vent .... OPEN

[ ]  Battery/Alternator Switches .... ON

[ ]  Essential Equipment .... ON (isolate defective equipment)

Emergency Descent  [EMERGENCY]

[ ]  Throttles .... IDLE

[ ]  Propellers .... FULL FORWARD

[ ]  Airspeed .... 140 KIAS

[ ]  Landing Gear .... DOWN

Maximum Glide  [EMERGENCY]

[ ]  Propellers .... FEATHER

[ ]  Wing Flaps .... UP

[ ]  Landing Gear .... UP

[ ]  Cowl Flaps .... CLOSED

[ ]  Airspeed .... 95 KIAS

Operation on Crossfeed  [EMERGENCY]

**LEFT ENGINE INOPERATIVE**

[ ]  Right Aux Fuel Pump .... ON

[ ]  Left Fuel Selector .... OFF

[ ]  Right Fuel Selector .... CROSSFEED

**RIGHT ENGINE INOPERATIVE**

[ ]  Left Aux Fuel Pump .... ON

[ ]  Right Fuel Selector .... OFF

[ ]  Left Fuel Selector .... CROSSFEED

Complete Loss of Electrical Power  [EMERGENCY]

[ ]  Both Alternator Switches .... OFF

[ ]  Battery Switch .... OFF

[ ]  Both BUS-ISO Circuit Breakers .... PULL

[ ]  Remove All Electrical Loads

[ ]  Both Alternators .... ON

[ ]  Minimize Electrical Loads

[ ]  Use Manual Landing Gear Extension Procedures

[ ]  Land Soon as Practical

Illumination of Alternator Out Light  [EMERGENCY]

[ ]  Load Meter .... CHECK FOR PROPER INDICATION

[ ]  No Load .... TURN OFF AFFECTED ALTERNATOR

[ ]  Reduce Load to Single Alternator Capacity

[ ]  Alternator Switch .... ON

[ ]  Re-Illumination of Alternator Light .... ALTERNATOR OFF

Illumination of Both Alternator Out Lights  [EMERGENCY]

[ ]  Load Meters .... CHECK FOR PROPER OPERATION

[ ]  Both Alternator Switches .... OFF

[ ]  Electrical Loads .... MINIMIZE (battery only available)

[ ]  Alternator Switches .... ON

[ ]  Re-Illumination of Alternator Out Lights .... ALTERNATOR OFF

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Beechcraft/BE-76 Duchess/TFA_Beechcraft_Duchess_Checklist_2004-02.txt

Rights: licensed

Checklist hash: 0ecc534aeef20236fcea83d51124446c74da6326f1a661fff23d796bf24e4d5a

Verify against the aircraft's own approved documentation before use.