Beechcraft BE-95 Travel Air — Beech95-checklist_selkirk_college

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Hot or Flooded Start" item count differs between passes (16 vs 15); verify nothing was dropped.

content_gap: Section "Pre-Takeoff" item count differs between passes (28 vs 27); verify nothing was dropped.

Pre Start

[ ]  Hobbs .... Checked

[ ]  Dispatch log .... Completed

[ ]  Logs and documents .... On board & valid

[ ]  Passenger briefing .... Completed

[ ]  Seats and belts .... Locked and secure *

[ ]  Controls .... Free and correct

[ ]  Parking brake .... Set

[ ]  Circuit breakers .... All in

[ ]  Electrics .... Off

[ ]  Alternators .... Off

[ ]  Avionics emergency .... Off

[ ]  Avionics master .... Off

[ ]  Gear selector .... Down

[ ]  Battery .... On, Gear locked

[ ]  Fuel Quantities .... Checked

[ ]  Fuel selectors .... On Aux.

[ ]  Flaps .... Up

[ ]  Cowl flaps .... Open

[ ]  Beacon .... On

Cold Engine Start

[ ]  Mixtures .... Full rich

[ ]  Propellers .... Full fine

[ ]  Throttles .... Set

[ ]  Boost pumps .... Checked & primed

[ ]  Left engine (start) .... Started

[ ]  Oil Pressure .... Normal

[ ]  Left Suction .... Normal

[ ]  Left Alternator .... On

[ ]  Right engine (start) .... Started

[ ]  Oil pressure .... Normal

[ ]  Right Suction .... Normal

[ ]  Right alternator .... On, Load split

[ ]  Engine gauges .... Normal

Hot or Flooded Start

[ ]  Mixtures .... Full rich

[ ]  Propellers .... Full fine

[ ]  Throttles .... Full forward

[ ]  Boost Pumps .... Checked & primed

[ ]  Mixtures .... Idle cut off

[ ]  Throttles .... Set

[ ]  Left engine (start) .... Started

[ ]  Oil pressure .... Normal

[ ]  Left Suction .... Normal

[ ]  Left Alternator .... On

[ ]  Right engine (start) .... Started

[ ]  Oil Pressure .... Normal

[ ]  Right Suction .... Normal

[ ]  Right alternator .... On, Load split

[ ]  Engine gauges .... Normal

Post Start

[ ]  Avionics master .... On

[ ]  PN-101 .... On and slave set

[ ]  Electrics .... On (as req=d)

[ ]  ELT .... Checked

[ ]  Instrument panel .... Panel checked

[ ]  Radios .... Radios checked

[ ]  Left fuel selector .... On crossfeed

Taxi

[ ]  Brakes .... Checked *

[ ]  Fuel crossfeed .... Checked

[ ]  Instruments .... Checked

Runup

[ ]  Parking brake .... Set

[ ]  Fuel selectors .... Quantity checked, on mains

[ ]  Mixtures .... Full rich

[ ]  Propellers .... Full fine

[ ]  Engine gauges .... Normal

[ ]  Throttles .... 2200

[ ]  Engine gauges .... Normal

[ ]  Suction .... Normal

[ ]  Propellers .... Exercised

[ ]  Throttles .... 2000

[ ]  Magnetos .... Checked

[ ]  Mixtures .... Checked

[ ]  Alternators .... Checked

[ ]  Alternate airs .... Checked

[ ]  Throttles .... 1500

[ ]  Feather .... Checked

[ ]  Throttles .... Idle checked, 1300 Set

Pre-Takeoff

[ ]  Seats & belts .... Locked & secure *

[ ]  Controls .... Free

[ ]  Trims .... Three set

[ ]  Fuel selectors .... Quantity checked, on Mains

[ ]  Alternate airs .... Off

[ ]  Flaps .... Set for takeoff

[ ]  Cowl flaps .... Open

[ ]  Circuit breakers .... All in

[ ]  Alternate static .... Checked and off

[ ]  Magnetos .... Both on both

[ ]  Alternators .... On

[ ]  Mixtures .... Full rich

[ ]  Propellers .... Full fine

[ ]  Altimeters .... _____ set

[ ]  Clock .... ______checked

[ ]  Heading indicators .... Checked & set

[ ]  Rear windows .... Closed & locked

[ ]  IFR clearance .... Clearance reviewed

[ ]  Crew briefing .... Briefing completed

[ ]  Cabin door & window .... Closed & locked *

[ ]  Boost pumps .... On

[ ]  Landing light .... On (if req=d)

[ ]  Pulse light .... On

[ ]  Pitot heat .... On (if req=d)

[ ]  Transponder .... On Alt ______

[ ]  Takeoff time .... Noted

[ ]  Heading indicator .... Checked on runway

Post Takeoff

[ ]  Gear .... Up

[ ]  Flaps .... Up

[ ]  Power .... Set

[ ]  Fuel selectors .... Quantity checked, on Aux

[ ]  Boost pumps .... Off

[ ]  Cowl flaps .... Set

[ ]  Landing light .... Off

Pre-Landing

[ ]  Seats & belts .... Locked & secure

[ ]  Parking brake .... Off

[ ]  Mixtures .... Full rich

[ ]  Propellers .... 2500

[ ]  Magnetos .... Both on both

[ ]  Electrics .... On (as req=d)

[ ]  Boost pumps .... On

[ ]  Fuel selectors .... Quantity checked, on mains

[ ]  Landing light .... On (if req=d)

[ ]  Gear .... Down & locked

Post Landing

[ ]  Flaps .... Up

[ ]  Cowl flaps .... Open

[ ]  Boost pumps .... Off

[ ]  Minimum electrics .... Set

[ ]  Transponder .... Standby

Shut Down

[ ]  Parking brake .... Set

[ ]  Throttles .... 1100

[ ]  ELT .... Checked

[ ]  Electrics .... Off

[ ]  Avionics master .... Off

[ ]  Battery .... Off

[ ]  Alternators .... Off

[ ]  Throttles .... Idle

[ ]  Live/dead mag .... Checked

[ ]  Mixtures .... Idle cut off

[ ]  Magnetos .... off

Rejected Takeoff Before Lift-off  [EMERGENCY]

[ ]  Throttles [MEMORY] .... closed

[ ]  Braking [MEMORY] .... maximum

[ ]  Stop [MEMORY] .... on remaining runway

IF THERE IS INSUFFICIENT RUNWAY TO STOP:

[ ]  Switches (master and alternators) [MEMORY] .... off

[ ]  Fuel Selectors [MEMORY] .... off

Rejected Takeoff - Airborne  [EMERGENCY]

[ ]  Throttles [MEMORY] .... as required, then off

[ ]  Braking [MEMORY] .... maximum

[ ]  Stop [MEMORY] .... on remaining runway

IF THERE IS INSUFFICIENT RUNWAY TO LAND AND STOP:

[ ]  Switches (master and alternators) [MEMORY] .... off

[ ]  Fuel Selectors [MEMORY] .... off

Engine Failure - Continuing Takeoff  [EMERGENCY]

[ ]  Control [MEMORY] .... Yaw and roll.

[ ]  Attitude [MEMORY] .... to control airspeed

[ ]  Power [MEMORY] .... full

[ ]  Drag [MEMORY] .... retract flaps and gear

[ ]  Identify [MEMORY] .... retard throttle on suspected engine

[ ]  Feather [MEMORY] .... propeller on failed engine

[ ]  Mixture [MEMORY] .... ico on failed engine

[ ]  Climb [MEMORY] .... at best S/E climb speed

[ ]  Trim [MEMORY] .... rudder, elevator, ailerons

GO TO: Complete Failed Engine Shutdown Checklist

Engine Failure ..Cruise  [EMERGENCY]

[ ]  Control [MEMORY] .... Adverse yaw and roll

[ ]  Attitude [MEMORY] .... altitude... (airspeed above Vyse)

[ ]  Power [MEMORY] .... to climb power

[ ]  Drag [MEMORY] .... confirm flaps and gear retracted

[ ]  Identify [MEMORY] .... retard throttle on suspected engine

[ ]  Check for Fire [MEMORY] .... if no fire

[ ]  Cause [MEMORY] .... check fuel pressure

[ ]  Boost pumps .... on

[ ]  Check fuel quantity

[ ]  Switch fuel tank .... as req=d

[ ]  Alternate air .... both on

[ ]  Magnetos .... on

[ ]  Test [MEMORY] .... advance throttle on affected engine.

IF ENGINE DOES NOT RE-START --- 9 TO 10

[ ]  Feather [MEMORY] .... prop control to feather detent

[ ]  Mixture [MEMORY] .... Idle cut-off

GO TO: Complete Failed Engine Shutdown Checklist

Failed Engine Shutdown Checklist  [EMERGENCY]

[ ]  Throttle .... closed on failed engine

[ ]  Prop control .... feather detent

[ ]  Mixture control .... idle cut-off

[ ]  Drag .... confirm flaps and gear as req=d

[ ]  Cowl flap .... closed

[ ]  Boost pump .... off

[ ]  Magnetos .... off (one at a time)

[ ]  Alternator .... off

[ ]  Fuel selector .... off

[ ]  Reduce electrical load

[ ]  Monitor condition of remaining engine

[ ]  Declare an emergency

[ ]  Land at nearest suitable aerodrome

Precautionary Engine Shutdown  [ABNORMAL]

[ ]  Throttle .... closed on failed engine

[ ]  Prop control .... feather detent

[ ]  Mixture control .... idle cut-off

[ ]  Drag .... confirm flaps and gear as req=d

[ ]  Cowl flap .... closed

[ ]  Boost pump .... off

[ ]  Magnetos .... off (one at a time)

[ ]  Alternator .... off

[ ]  Fuel selector .... off

[ ]  Reduce electrical load

[ ]  Monitor condition of remaining engine

[ ]  Declare an emergency

[ ]  Land at the nearest suitable aerodrome

Engine Restart In Flight  [ABNORMAL]

[ ]  Fuel Selector .... on main or aux.

[ ]  Magnetos .... on

[ ]  Boost pump .... on

[ ]  Mixture control .... idle cut-off

[ ]  Throttle .... open 2 inch

[ ]  Propeller control .... move into governing range

[ ]  Once engine is wind-milling, advance mixture to the full rich position.

[ ]  Warm up engine .... at 2000 RPM and 15" MAP

[ ]  Alternator .... on

[ ]  All cockpit switches and controls .... normal.

Engine Fire on Start-up  [EMERGENCY]

[ ]  Mixture [MEMORY] .... idle cut-off

[ ]  Throttle [MEMORY] .... full open

[ ]  Cranking [MEMORY] .... continue

IF FIRE CONTINUES LONGER THAN A FEW SECONDS

[ ]  Fuel selectors [MEMORY] .... both off

[ ]  Mixture controls [MEMORY] .... both idle cut-off

[ ]  Switches [MEMORY] .... off

[ ]  Evacuate the aircraft [MEMORY]

NOTE: Extinguish the fire with ground equipment if practical.

Engine Fire in Flight  [EMERGENCY]

[ ]  Fuel selector [MEMORY] .... off

[ ]  Mixture [MEMORY] .... idle cut-off

[ ]  Propeller [MEMORY] .... feather

[ ]  Boost pump .... off

[ ]  Magneto switch .... off

[ ]  Alternator .... off

NOTE: If fire continues, consider increasing airspeed in an attempt to extinguish it.

[ ]  Declare an emergency and indicate intentions.

[ ]  Expedite your approach to the nearest suitable landing area.

Cabin Fire  [EMERGENCY]

[ ]  Vents [MEMORY] .... close

[ ]  Extinguisher [MEMORY] .... activate if necessary.

NOTE: (Get assistance from passengers if needed).

[ ]  Ventilate [MEMORY] .... the cabin

[ ]  Land .... as soon as practicable.

[ ]  Inspect for damage.

Landing Gear Manual Extension  [ABNORMAL]

[ ]  Reduce speed .... to 120 KIAS or less

[ ]  Landing gear motor circuit breaker .... pulled

[ ]  Landing gear selector .... down

[ ]  Engage the hand- crank and turn counter-clockwise as far as possible (approx. 50 turns).

[ ]  Check mechanical indicator to ascertain that the gear is down.

[ ]  If electrical system is operative, check gear position light and warning horn.

[ ]  Disengage hand-crank.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Beechcraft/BE-95 Travel Air/Beech95-checklist_selkirk_college.txt

Rights: licensed

Checklist hash: 25e55bf3f7d230fa4704d17d6750a611163f8a11def19c71c5acbd24776790de

Verify against the aircraft's own approved documentation before use.