Beechcraft Beech 18 — david_a_seest_Beech_18_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Pre-Start Checklist

[ ]  1.  Cabin Door .... CLOSED

[ ]  2.  Controls .... FREE

[ ]  3.  Landing Gear SW. .... DOWN

[ ]  4.  Parking Brake .... ON

[ ]  5.  Bat. & Gen. SWS. .... ON

[ ]  6.  Gear Safe Light .... CHECK

[ ]  7.  Fuel Quantity .... CHECK

[ ]  8.  Fuel Selectors .... MAIN

[ ]  9.  Crossfeed .... OFF

[ ]  10.  Anti-Ice Fluid .... CHECKED

[ ]  11.  Trim .... SET > T/O

[ ]  12.  Flap Switch .... NEUTRAL

[ ]  13.  Throttles .... 1/8 OPEN

[ ]  14.  Mixtures .... RICH

[ ]  15.  Props. .... HIGH

[ ]  16.  Manifold Heat .... COLD

[ ]  17.  Oil Shutters .... HOT

[ ]  18.  Oil Shutoff Valves .... OPEN

[ ]  19.  Circuit Breakers .... CHECKED

[ ]  20.  Electrical Switchs .... OFF

[ ]  21.  Alt. Static Source .... NORMAL

[ ]  22.  Oil By-Pass Valves .... AS REQUIRED

[ ]  23.  Warning Lights .... CHECKED

Start Checklist

[ ]  1.  Fuel Boost Pumps .... ON

[ ]  2.  Magnetos .... OFF

[ ]  3.  Engine Sel. SW. .... RT or LT

[ ]  4.  Starter (2 Revs) .... ENGAGE

[ ]  5.  Magnetos. .... ON

[ ]  6.  Ignition Boost SW. .... ON

[ ]  7.  Primer .... AS REQUIRED

NOTE: (SAME PROC. FOR OTHER ENGINE)

[ ]  8.  Fuel Boost Pumps .... OFF

[ ]  9.  Warm Up Engines .... 1000-1200 RPM

After-Starting Engines

[ ]  1.  Oil Pressure .... UP

[ ]  2.  Fuel Boost Pumps .... OFF

[ ]  3.  Lights .... AS REQUIRED

[ ]  4.  Radios .... ON

[ ]  5.  Tailwheel .... UNLOCKED

[ ]  6.  Parking Brake .... UNLOCKED

Run-Up

[ ]  1.  Temps. & Press. .... GREEN ARC

[ ]  2.  Props (1500 rpm) .... EXERCISE

[ ]  3.  Manifold Heat .... CHECK

[ ]  4.  Vacuum .... CHECK

[ ]  5.  Volt/Ammeters .... CHECK

[ ]  6.  Mags. (1800 rpm) .... CHECK

Before Take-Off

[ ]  1.  Controls .... FREE

[ ]  2.  Mixture .... RICH

[ ]  3.  Props. .... HIGH RPM

[ ]  4.  Friction Locks .... SET

[ ]  5.  Oil Shutters .... AS REQUIRED

[ ]  6.  Manifold Heat .... COLD

[ ]  7.  Flaps .... UP

[ ]  8.  Fuel Selector .... MAIN TANKS

[ ]  9.  Oil By-Pass .... CLOSED

[ ]  10.  Trim .... SET

[ ]  11.  D.G.’s .... SET

[ ]  12.  Instruments .... CHECK

[ ]  13.  Navs. .... CHECK / SET

Take-Off

[ ]  1.  Time .... RECORD

[ ]  2.  Fuel Boost Pumps .... ON

[ ]  3.  Landing Lights .... ON

[ ]  4.  Inverter .... ON

[ ]  5.  Transponder .... ALT

[ ]  6.  Tail Wheel .... LOCKED

[ ]  7.  D.G.’s .... R.W. HEADING

[ ]  8.  Pitot Heat .... AS REQUIRED

[ ]  9.  Throttles .... 36.5” & 2300 RPM

[ ]  10.  Oil Pressure .... GREEN ARC

[ ]  11.  Oil Temperature .... GREEN ARC

Climb

[ ]  1.  Gear .... UP

[ ]  2.  Flaps .... UP

[ ]  3.  Manifold Pressure .... 32”

[ ]  4.  Prop. RPM .... 2150 RPM

[ ]  5.  Fuel Boost Pumps .... OFF – PX CK

[ ]  6.  Landing Lights .... OFF

[ ]  7.  Airspeed .... 120/140 MPH

[ ]  8.  Flight Plan .... OPEN

Cruise

[ ]  1.  Manifold Pressure .... 28”

[ ]  2.  Prop. RPM .... 2000 RPM

[ ]  3.  Mixture .... Lean

[ ]  4.  Oil Shutters .... 150 F.

[ ]  5.  Manifold Heat .... AS REQUIRED

[ ]  6.  Fuel Quantity .... Manage

Approach / Descent Checklist

[ ]  1.  Customs Code .... OBTAIN

[ ]  2.  Fuel Selectors .... MAIN TANKS

[ ]  3.  Mixtures .... ENRICHEN

[ ]  4.  Manifold Pressure .... 26”

[ ]  5.  Manifold Heat .... AS REQUIRED

[ ]  6.  Altimeters .... SET

[ ]  7.  Navs .... SET

[ ]  8.  Flight Plan .... CLOSED

Before Landing Checklist

[ ]  1.  Fuel Boost Pumps .... ON

[ ]  2.  Landing Lights .... ON

[ ]  3.  Props. .... 2150 RPM

[ ]  4.  Landing Gear .... DOWN/GREEN

[ ]  5.  Manifold Heat .... OFF

[ ]  6.  Heater .... CYCLE

[ ]  7.  Brakes .... CHECK

[ ]  8.  Flaps .... AS NEEDED

After Landing Checklist

[ ]  1.  Fuel Boost Pumps .... OFF

[ ]  2.  Landing Lights .... OFF

[ ]  3.  Inverter .... OFF

[ ]  4.  Transponder .... STBY

[ ]  5.  Props. .... HIGH RPM

[ ]  6.  Oil Shutters .... 150 F.

[ ]  7.  Flaps .... UP

[ ]  8.  Pitot Heat .... OFF

[ ]  9.  Taxi Lights .... AS REQUIRED

[ ]  10.  Time .... RECORD

Shutdown Checklist

[ ]  1.  Parking Brake .... SET

[ ]  2.  Radios .... OFF

[ ]  3.  Mixtures .... IDLE CUT-OFF

[ ]  4.  Magnetoes .... OFF

[ ]  5.  Battery/Gen SW’s .... OFF

[ ]  6.  Tailwheel .... LOCK

[ ]  7.  Controls .... LOCK

[ ]  8.  Tiedowns .... (3 POINTS)

[ ]  9.  Chocks .... ALL WHEELS

Airspeeds – Miles-Per-Hour

NOTE: Vne 257  Vyse 122    Vmc 86

NOTE: Vno 205  Vyxe 110    Vle 160

NOTE: Vy 135   Vlo 160     Vx 119

NOTE: Va 153   Vfe 120     Vso 75

NOTE: Vs 80    Max T\O Wt:  10,100

NOTE:     Max Lnd Wt:    9,800

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Beechcraft/Beech 18/david_a_seest_Beech_18_Checklist.txt

Rights: licensed

Checklist hash: 587a1d8d52628dc73fdfc47538a4e7319cb841d36eefd6765dabbb6eebc64ceb

Verify against the aircraft's own approved documentation before use.