Beechcraft J35 Bonanza — bonanza_j35check

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "EMERGENCY Gear Up Landing" item count differs between passes (9 vs 6); verify nothing was dropped.

Preflight

INTERIOR

[ ]  Mags .... Off

[ ]  Cowl flaps .... Open

[ ]  Fuel selector .... Left tank

[ ]  Gust lock .... Remove

[ ]  Landing gear switch .... Down

[ ]  Master switch .... On

[ ]  Fuel gages .... Check quantity

[ ]  Flaps .... Down

[ ]  Master switch .... Off

[ ]  Aircraft documents .... ARROW

[ ]  Weight and balance .... Review

[ ]  Flashlights/equipment .... Check

EXTERIOR

[ ]  Empennage .... Check security, movement, trim tab security/cable attachment free play

[ ]  Tail tiedown .... Remove

[ ]  Right static port .... Check unobstructed

[ ]  Underside .... Check condition, antennas

[ ]  Right flap .... Check for damage, security

[ ]  Right aileron .... Check for damage, security, movement

[ ]  Right wing .... Check for damage

[ ]  Right tiedown .... Remove

[ ]  Right gear .... Check strut (2 inches showing), uplock roller, brakes, and tire (30 psi)

[ ]  Right fuel tanks

PREFLIGHT (CONT’D)

[ ]  Right cowl .... Check secure

[ ]  Cowl plugs .... Remove

[ ]  Propeller and spinner .... Check condition

[ ]  Engine air inlet .... Check unobstructed

[ ]  Left cowl .... Open

[ ]  Alternator belt .... Check tension

[ ]  Oil level .... Check (9 quarts max. 8 quarts min.)

[ ]  Oil filler cap .... Check secure

[ ]  Left cowl .... Close

[ ]  Fuel selector drain .... Check sump

[ ]  Left wing .... Check for damage

[ ]  Left fuel tanks .... Check quantity, vents, sump

[ ]  Left gear .... .Check strut (2 inches showing), uplock roller, brakes, and tire (30 psi)

[ ]  Pitot tube .... Remove cover. Check for obstructions

[ ]  Stall warning switch .... Check movement, operation

[ ]  Navigation lights (night flight) .... Check

[ ]  Left tiedown .... Remove

[ ]  Left aileron .... Check for damage, security, movement

[ ]  Left flap .... Check for damage, security

[ ]  Left static port .... Check unobstructed

Starting Engine

COLD

[ ]  Brakes .... Set

[ ]  Landing gear switch .... DOWN

[ ]  Circuit breakers .... Check

[ ]  Fuel .... Desired tank (Left?, NEVER Aux)

[ ]  Prop .... Full increase

[ ]  Mixture .... Rich

[ ]  Throttle .... Open 2 turns

[ ]  Master switch .... ON

[ ]  Fuel pump .... LOW, wait until indication on fuel flow meter, then OFF

[ ]  Prop area .... “Clear!”

[ ]  Starter .... Engage

[ ]  Engine speed .... 1000 RPM

[ ]  Fuel pump .... OFF

[ ]  Oil Pressure .... must indicate within 30 seconds

[ ]  Flaps .... UP

[ ]  Flap switch .... Center

[ ]  Beacon .... ON

HOT

NOTE: Same as cold, except...

[ ]  Mixture .... Rich

[ ]  Throttle .... Open 1 turn

[ ]  Fuel pump .... LOW - peak twice, then OFF

[ ]  Throttle .... Open 3/4 of full throttle

[ ]  Prop area .... “Clear!”

[ ]  Starter .... Engage

[ ]  Fuel pump .... LOW momentarily as engine fires

[ ]  Throttle .... Reduce immediately

After Start

[ ]  Nav lights .... On, as appropriate

[ ]  Radio master .... ON

[ ]  Transponder .... STANDBY

[ ]  ATIS, Ground, Tower .... SET                   (SQL 125.9/121.6/119.0)

[ ]  Altimeter .... Set (within 40’ field elev.)

Taxi

[ ]  Brakes .... Test

[ ]  Artificial Attitude .... Check operation

[ ]  Turn Coordinator .... Check operation

[ ]  Directional Gyro .... Check operation

[ ]  Magnetic compass .... Check operation, fluid level

Before Takeoff

ENGINE RUNUP

[ ]  Brakes .... Set

[ ]  Fuel .... Desired tank (left?). NEVER aux!

[ ]  Mixture .... Rich

[ ]  Oil temperature .... Wait for 70°F

[ ]  Engine speed .... 1800 RPM

[ ]  Mags .... Test: L-Both-R-Both. Drop-off < 100 Rpm and < 50 RPM differential..

[ ]  Prop .... Cycle twice full travel. Check oil pressure/man pres/RPM drop, normal return.

[ ]  Vacuum .... Between 4 - 7 inches

[ ]  Alternator .... Charging - show discharge

INSTRUMENTS

[ ]  Engine gages .... Check

[ ]  Instruments .... Check and set

[ ]  Mags .... BOTH

[ ]  Prop .... Full increase

[ ]  Mixture .... Rich

[ ]  Elevator trim .... 3° (green/white transition)

[ ]  Aileron trim .... Neutral

[ ]  Flaps .... 0° (first line 10° short/soft field)

[ ]  Flight controls .... Free, correct, and smooth.

[ ]  Door, shout, and emergency windows .... Latched

[ ]  Seat belts .... Fastened

[ ]  Passengers .... Briefed

NAVIGATION

[ ]  Tower/Departure freq. .... Set

[ ]  Sqawk .... Set

[ ]  Departure route/Navaid .... Set

Takeoff

[ ]  Transponder .... ALT

[ ]  Mixture .... Rich

[ ]  Prop .... Full increase

[ ]  Strobes .... ON

[ ]  Landing lights .... As appropriate

[ ]  HSI .... Check heading with runway number

[ ]  Departure time .... Record

[ ]  Mixture .... Lean for takeoff, if elevation > 3000 feet

[ ]  Rotate .... 60-70 mph, VX - 75 mph Vy - 103 mph (sea level)

Post Takeoff

[ ]  Gear .... UP (when out of usable runway)

[ ]  Flaps .... Up

[ ]  Throttle/Prop .... 1000 feet AGL, - 24’’/2400 RPM

Climb

[ ]  Each cardinal 1000 feet .... Adjust manifold pressure

[ ]  Pass 3000/5000/7000 feet .... Adjust mixture

Cruse

[ ]  Throttle/Prop .... Set for cruse

[ ]  Mixture .... Lean 50°F (25° if 65% power) below peak.

[ ]  Cowl flaps .... Close

Landing - Downwind

[ ]  Gas Fuel .... Proper tank (NOT aux)

[ ]  Undercarriage Gear .... DOWN (140 mph max)

[ ]  Mixture .... Full rich (sea level) or as appropriate

[ ]  Prop .... Full increase (after power reduction)

[ ]  Switches/Seatbelts .... Landing lights/Passengers briefed and belted

[ ]  Flaps .... Set (120 mph max)

[ ]  Approach speed .... 90 mph

Landing - Final

[ ]  Prop .... Full increase

[ ]  Cowl flaps .... Open

[ ]  Gear .... DOWN indicated locked

[ ]  Speed over the fence .... 80 mph

Go Around

[ ]  Prop .... Full increase

[ ]  Throttle .... Full power

[ ]  Mixture .... Rich (sea level) or as appropriate

[ ]  Throttle .... Full power

[ ]  Flaps .... Up gradually

[ ]  Gear .... UP

[ ]  Cowl flaps .... Open

[ ]  Throttle/Prop .... 1000 feet AGL - 24’’/2400 RPM

[ ]  VX .... 75 mph Vy - 103 mph

After Landing

[ ]  Flaps .... Raised

[ ]  Transponder .... STANDBY

[ ]  Strobes .... OFF

Shutdown

[ ]  Radio master .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Throttle .... 1000 RPM

[ ]  Prop .... Full increase

[ ]  Mixture .... Idle cut-off

[ ]  Master switch .... OFF

[ ]  Mags .... OFF

[ ]  Arrival Time .... Record

Secure

[ ]  Flight time .... Record in airplane log book

[ ]  Elevator trim .... 3° (green/white transition)

[ ]  Cowl flaps .... Close

[ ]  Gust lock and bungee cords .... Install

[ ]  Instrument panel .... Cover with towel

[ ]  Cabin door .... Close and lock

[ ]  Airplane cover .... Install

[ ]  Pitot tube cover .... Install

[ ]  Cowl plugs .... Install

[ ]  Tail and wing tiedowns .... Secure

[ ]  Flight plan .... Close

EMERGENCY Gear Extension  [EMERGENCY]

[ ]  Master switch .... ON

[ ]  Gear .... DOWN

[ ]  Gear indicator .... Check green lamp lit or lower indicator in the DOWN position.

NOTE: If gear down indication still not present:

[ ]  Airspeed .... < 120 mph

[ ]  Gear circuit breaker .... Pull

[ ]  Gear crank .... approx. 50 counterclockwise turns

[ ]  Lower gear indicator .... Verify in down position

[ ]  Gear crank .... Stow

NOTE: Note: with landing gear circuit breaker “pulled”, landing gear position lights are nonfunctional.

EMERGENCY Gear Up Landing  [EMERGENCY]

NOTE: In an emergency, a gear up landing may be made:

NOTE: 171. When the surface is too soft or rough to permit a satisfactory gear down landing.

NOTE: 172. When a field is too short for a gear down landing, one that might cause more damage by hitting obstructions than a gear up landing would.

NOTE: 173. When a water landing is necessary.

[ ]  Flaps .... UP

[ ]  Throttle .... CLOSED

[ ]  Master .... OFF

[ ]  Mags .... OFF

[ ]  Fuel selector .... OFF

EMERGENCY Power Loss  [EMERGENCY]

[ ]  Best glide speed .... 103 mph

[ ]  Select suitable landing area

[ ]  Fuel selector .... Switch to tank not in use

[ ]  Throttle .... Open 1/2 inch from present setting

[ ]  Prop .... Full increase

[ ]  Fuel pressure .... Check. If none, Fuel pump - HIGH

[ ]  Mags .... Check best: left, right, both.

TO RESTART FAILED ENGINE:

[ ]  Fuel pump .... OFF

[ ]  Throttle .... Closed

[ ]  Mixture .... Idle cut-off

[ ]  Fuel selector .... OFF

[ ]  Wait several seconds

[ ]  Fuel elector .... Fullest tank

[ ]  Mixture .... Rich

[ ]  Throttle .... Open 1/4 inch

[ ]  Fuel pressure .... If zero - Fuel pump - HIGH

IF ENGINE CANNOT BE RESTARTED:

[ ]  Mixture .... Idle cut-off

[ ]  Throttle .... Closed

[ ]  Mags .... OFF

[ ]  Fuel selector .... OFF

[ ]  Prop .... Low RPM

[ ]  Best glide speed .... 90 mph

EMERGENCY Engine Fire In Flight  [EMERGENCY]

[ ]  Mixture .... Idle Cut-off

[ ]  Fuel selector .... OFF

[ ]  Mags .... OFF

[ ]  Throttle .... Closed

[ ]  Best glide speed .... 103 mph

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Beechcraft/J35 Bonanza/bonanza_j35check.txt

Rights: licensed

Checklist hash: 4930e4b59f740a649abc56dfd40fc6261d58af403d01f01552b1c3fd32df0df1

Verify against the aircraft's own approved documentation before use.