Beechcraft T-34B Mentor — T-34BChecklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Complete Engine Failure During Flight" item count differs between passes (11 vs 12); verify nothing was dropped.

content_gap: Section "Radio Failure" item count differs between passes (15 vs 26); verify nothing was dropped.

Airspeeds (KIAS)

[ ]  VS0 .... 51

[ ]  VR .... 55

[ ]  VS1 .... 62

[ ]  Short-Field Final (100% flaps) .... 65

[ ]  Normal/Soft-Field Final (100% flaps) .... 70

[ ]  VX .... 70

[ ]  VY .... 80

[ ]  Final (0% flaps) .... 80

[ ]  Base (60% flaps) .... 80

[ ]  Past Abeam (20% flaps) .... 80

[ ]  Downwind .... 90

[ ]  VMax Glide .... 90

Glide Ratio = 2.1 NM/1,000’ AGL

[ ]  Cruise Climb .... 100

[ ]  VFE .... 109

[ ]  VLO/LE .... 109

[ ]  VA .... 148

[ ]  VNO .... 152

[ ]  VNE .... 152

Crosswind

[ ]  Max Demonstrated .... 15

Preflight — Front Cockpit

[ ]  Canopy and Windshield .... CHECK

[ ]  Boost Pump .... OFF

[ ]  Fuel Shutoff Valve Handle .... OFF

[ ]  Trim Tabs .... 0(

[ ]  Throttle .... CLOSED

[ ]  Prop .... FULL INCREASE

[ ]  Mixture .... IDLE CUTOFF

[ ]  Mags .... OFF

[ ]  Landing Gear Handle .... DOWN

[ ]  Emerg Landing Gear Retract Switch .... WIRED OFF

[ ]  Accelerometer .... RESET

[ ]  Battery .... OFF

[ ]  Controls .... UNLOCKED

[ ]  Avionics .... OFF

[ ]  ELT .... ARMED

[ ]  Radio Master .... OFF

[ ]  Circuit Breakers .... IN

[ ]  Prop .... CLEAR

[ ]  Battery .... ON

[ ]  Fuel Gauges .... TRUE

[ ]  Lights .... INTERIOR / EXTERIOR

[ ]  Pitot Heat .... TEST

[ ]  Battery .... OFF

Preflight — Rear Cockpit

[ ]  Canopy .... CHECK

[ ]  Papers .... A.R.O.W.

[ ]  Boost Pump .... OFF

[ ]  Emerg Landing Gear Retract Switch .... WIRED OFF

[ ]  First Aid Kit .... SECURED

IF SOLO FLIGHT:

[ ]  Shoulder Harness, seat belt, loose equipment .... SECURED

[ ]  Canopy .... CLOSED AND LOCKED

Preflight — Fuel Samples & Engine Compartment

[ ]  Port Engine Compartment Cowl .... OPEN

[ ]  Fuel Shutoff Valve Handle .... ON

[ ]  Boost Pump .... ON

[ ]  Battery .... ON

[ ]  Engine Fuel Drain Valve .... DRAIN

[ ]  Fuel Sump .... DRAIN

[ ]  Main Fuel Strainer .... DRAIN

[ ]  Fuel Vent Standpipe .... CHECK

[ ]  Augmentor Tubes .... CHECK

[ ]  Battery .... OFF

[ ]  Boost Pump .... OFF

[ ]  Fuel Shutoff Valve Handle .... OFF

[ ]  Engine Fuel Drain Valve .... CHECK CLOSED

[ ]  Oil Level .... 9-10 QTS

[ ]  Engine & Accessories .... CHECK

[ ]  Left Engine Compartment Cowl .... LATCHED

[ ]  Right Engine Compartment Cowl .... OPEN

[ ]  Engine & Accessories .... CHECK

[ ]  Right Engine Compartment Cowl .... LATCHED

[ ]  Battery .... CHECK

[ ]  Drain Jar .... CHECK

Preflight — Left Wing

[ ]  Flap .... CHECK

[ ]  Trim Tab .... CHECK

[ ]  Static Wick .... CHECK

[ ]  Aileron .... CHECK

[ ]  Navigation Light .... CHECK

[ ]  Stall Vane .... CHECK

[ ]  Landing Light .... CHECK

[ ]  Tiedown .... DISCONNECT

[ ]  Pitot Tube .... CHECK

[ ]  Fuel Quantity .... CHECK VISUALLY

[ ]  Fuel Filler Cap .... SECURE

[ ]  Cockpit Air Intake Screen .... CHECK

[ ]  Landing Gear Doors .... CHECK

[ ]  Wheel Well .... CHECK

[ ]  Uplock Cable & Springs .... CHECK

[ ]  Landing Gear Roller .... CHECK

[ ]  Shock Strut .... CHECK

Approx 3”

[ ]  Hydraulic Fittings .... CHECK

[ ]  Brake Assembly .... CHECK

[ ]  Tire .... CHECK

35 PSI

[ ]  Chock .... REMOVE

Preflight — Nose

[ ]  Chock .... REMOVE

[ ]  Nose Tire .... CHECK

40 PSI

[ ]  Mud Scraper .... CHECK

[ ]  Grounding Wire .... CHECK

[ ]  Nose Strut .... CHECK

Approx 5”

[ ]  Shimmy Damper .... CHECK

[ ]  Nose Gear Centering Roller .... CHECK

[ ]  Uplock Mechanism .... CHECK

[ ]  Air Filter .... CLEAR

[ ]  Passing Light .... CHECK

[ ]  Engine Air Inlets .... CLEAR

[ ]  Prop/Spinner .... CHECK

Preflight — Right Wing

[ ]  Cockpit Air Intake Screen .... CHECK

[ ]  Fuel Quantity .... CHECK VISUALLY

[ ]  Fuel Filler Cap .... SECURE

[ ]  Landing Gear Doors .... CHECK

[ ]  Wheel Well .... CHECK

[ ]  Uplock Cable & Springs .... CHECK

[ ]  Landing Gear Roller .... CHECK

[ ]  Shock Strut .... CHECK

Approx 3”

[ ]  Hydraulic Fittings .... CHECK

[ ]  Brake Assembly .... CHECK

[ ]  Tire .... CHECK

35 PSI

[ ]  Chock .... REMOVE

[ ]  Tiedown .... DISCONNECT

[ ]  Landing Light .... CHECK

[ ]  Navigation Light .... CHECK

[ ]  Aileron .... CHECK

[ ]  Static Wick .... CHECK

[ ]  Trim Tab .... CHECK

[ ]  Flap .... CHECK

Preflight — Fuselage & Empennage

[ ]  Right Static Port .... CLEAR

[ ]  Antennas .... CHECK

[ ]  Left Elevator .... CHECK

[ ]  Tiedown .... DISCONNECT

[ ]  Rudder .... CHECK

[ ]  Navigation Light .... SECURE

[ ]  Static Wicks .... CHECK

[ ]  Trim Tabs .... CHECK

[ ]  Right Elevator .... CHECK

[ ]  Left Static Port .... CLEAR

[ ]  Baggage Door .... LATCHED

Prestart

[ ]  Hobbs/Tach Time .... RECORDED

[ ]  Passenger Brief .... COMPLETE

[ ]  Seat and Rudder Pedals .... ADJUST

[ ]  Harness .... FASTEN

[ ]  Inertial Reel Lock .... CHECK

[ ]  Wing Flap Lever .... OFF

[ ]  Landing Lights .... OFF

[ ]  Alternate Air .... IN

[ ]  Generator .... ON

[ ]  Cockpit Air Handles .... SET

[ ]  Landing Gear Emergency Handcrank .... KNOB UP & LOCKED

[ ]  Anti-Collision Lights .... ON

[ ]  Navigation Lights .... AS REQUIRED

[ ]  Pitot Heat .... OFF

[ ]  Circuit Breakers .... IN

Start

[ ]  Canopy .... OPEN

[ ]  Fuel Shutoff Valve Handle .... ON

[ ]  Fuel Boost Pump .... ON

[ ]  Brakes .... SET

[ ]  Battery .... ON

[ ]  Fuel Pressure .... CHECK

PRIME IF ENGINE COLD:

[ ]  Mixture .... RICH

[ ]  Throttle .... OPEN 5 SEC

[ ]  Throttle .... ¼” OPEN

[ ]  Mixture .... IDLE CUTOFF

[ ]  Prop .... CLEAR

[ ]  Mags .... BOTH

[ ]  Starter .... ENGAGE

[ ]  Mixture .... SLOWLY TO RICH

[ ]  Throttle .... 1200 RPM

[ ]  Oil Pressure .... CHECK

[ ]  Generator Warning Light .... CHECK

[ ]  Voltage .... CHECK

27.7-28.8V

[ ]  Fuel Boost Pump .... OFF

[ ]  Mixture .... LEAN

Pre-Taxi

[ ]  Radio Master .... ON

[ ]  Flaps .... CYCLED & UP

[ ]  Trim Tabs .... SET 6( R, 3( UP, 0( AILERON

[ ]  Landing Gear Warning Light .... CHECK

[ ]  Landing Gear Indicators .... DOWN

[ ]  Fuel Quantity .... NOTED

[ ]  Avionics .... ON / SET

[ ]  Transponder .... CODE / STANDBY

[ ]  Clock .... SET

[ ]  Airspeed Indicator .... CHECK

[ ]  Attitude Indicator .... SET

[ ]  Heading Indicator .... SET

[ ]  VSI .... CHECK

[ ]  ATIS/AWOS .... COPY

[ ]  Altimeter .... SET / CHECK

[ ]  Taxi Light .... AS REQUIRED

Taxi

[ ]  Brakes .... TEST

[ ]  Attitude Indicator .... CHECK

[ ]  Turn-and-Slip Indicator .... CHECK

[ ]  Heading Indicator .... CHECK

[ ]  Magnetic Compass .... CHECK

Run-Up

[ ]  Brakes .... SET

[ ]  Oil Temp .... CHECK

IF OIL TEMP < 40( C:

[ ]  Oil Bypass .... PULL

[ ]  Prop .... FULL INCREASE

[ ]  Mixture .... BEST POWER

[ ]  Throttle .... 1,800 RPM

[ ]  Prop .... CYCLE 6 TIMES (4 if warm)

[ ]  Prop Governor .... CHECK

[ ]  Prop .... FULL INCREASE

[ ]  Throttle .... 2,000 RPM

[ ]  Mags (R & L) .... CHECK

100 RPM max drop, 50 RPM max differential

[ ]  Alternate Air .... CHECK

Approx ½” MP drop

[ ]  Throttle .... 1700 RPM

[ ]  Fuel Boost Pump .... ON

15-20 PSI

[ ]  Vacuum .... CHECK

[ ]  Generator .... CHECK

27.7-28.8V & Light Out

[ ]  Oil Pressure .... CHECK

[ ]  Oil Temperature .... CHECK

[ ]  Oil Bypass .... IN

[ ]  Throttle .... CLOSED

600-750 RPM

[ ]  Throttle .... 1,000 RPM

Takeoff

[ ]  Anti-Collision Lights .... ON

[ ]  Fuel Boost Pump .... ON

[ ]  Fuel Shutoff Valve Handle .... ON

[ ]  Fuel Pressure .... NORMAL

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Caps .... LOCKED

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Trim Tabs .... SET 6( R, 3( UP, 0( AILERON

[ ]  Alternate Air .... OFF

[ ]  Generator .... OPERATING

[ ]  Instruments .... CHECKED & SET

[ ]  Mixture .... BEST POWER

[ ]  Prop .... FULL INCREASE

[ ]  Mags .... BOTH

[ ]  Flaps .... SET (0-75%)

[ ]  Harness .... LOCKED

[ ]  Canopy .... LOCKED (front may be locked open)

[ ]  Radio .... SET

[ ]  Heading Indicator .... SET

CLEARED FOR TAKEOFF

[ ]  Landing Lights .... ON

[ ]  Transponder .... ALTITUDE

[ ]  Throttle .... FULL (2375 RPM minimum)

After Takeoff

[ ]  Landing Gear .... UP

[ ]  Flaps .... UP

[ ]  Fuel Boost Pump .... OFF

[ ]  Fuel Caps .... SECURE

[ ]  Engine Gauges .... CHECK

[ ]  Landing Lights .... AS DESIRED

Airspeeds — Climb (MPH IAS)

[ ]  VR .... 55

[ ]  VX .... 70

[ ]  VY .... 80

[ ]  Cruise Climb .... 100

Cruise

[ ]  Landing Lights .... OFF

[ ]  Throttle .... SET

[ ]  Prop .... SET

[ ]  Mixture .... LEAN

[ ]  Engine Gauges .... CHECK

[ ]  Heading Indicator .... SET

Approach

[ ]  Mixture .... RICHEN AS REQUIRED

[ ]  ATIS/AWOS .... COPY

[ ]  Altimeter .... SET

[ ]  Avionics .... SET

[ ]  Heading Indicator .... SET

[ ]  Engine Gauges .... CHECK

[ ]  Brakes .... PEDAL TEST

[ ]  Harness .... FASTENED

[ ]  Landing Lights .... ON

Landing

[ ]  Fuel Boost Pump .... ON

[ ]  Alternate Air .... IN

[ ]  Mixture .... BEST POWER

[ ]  Harness .... LOCKED

[ ]  Landing Gear .... DOWN

[ ]  Flaps .... AS REQUIRED

[ ]  Prop .... FULL INCREASE

Airspeeds — Landing Pattern (MPH IAS)

[ ]  Downwind .... 90

[ ]  Past Abeam (20% flaps) .... 80

[ ]  Base (60% flaps) .... 80

[ ]  Final (0° flaps) .... 80

[ ]  Normal/Soft-Field Final (100% flaps) .... 70

[ ]  Short-Field Final (100% flaps) .... 65

After Landing

[ ]  Mixture .... LEAN

[ ]  Flaps .... UP

[ ]  Fuel Boost Pump .... OFF

[ ]  Landing Lights .... OFF

[ ]  Taxi Light .... AS REQUIRED

[ ]  Pitot Heat .... OFF

[ ]  Transponder .... STANDBY

Shutdown

[ ]  Throttle .... 1,000 RPM

[ ]  Mags .... GROUNDING CHECK

[ ]  Avionics .... OFF

[ ]  Radio Master Switch .... OFF

[ ]  Mixture .... IDLE CUTOFF

[ ]  Prop .... CHECK STOPPED

[ ]  Fuel Shutoff Valve .... OFF

[ ]  Mags .... OFF

[ ]  Lights .... OFF

[ ]  Battery .... OFF

Securing

[ ]  Hobbs/Tach Time .... RECORDED

[ ]  Flight Controls .... LOCKED

[ ]  Rudder Pedals .... FULL FORWARD

[ ]  Chocks .... INSTALL

[ ]  Tiedowns .... SECURE

[ ]  Baggage Door .... CLOSED

[ ]  Canopy .... CLOSED

Flooded Engine  [ABNORMAL]

[ ]  Mixture .... FULL LEAN

[ ]  Throttle .... FULL OPEN

[ ]  Mags .... START

WHEN ENGINE FIRES:

[ ]  Mixture .... RICH

[ ]  Throttle .... 1,200 RPM

Ground Burnout  [ABNORMAL]

[ ]  Prop .... FULL INCREASE

[ ]  Mixture .... RICH

[ ]  Throttle .... 2,000 RPM

[ ]  Mixture .... LEAN TO 50 RPM DROP BELOW BEST POWER

AFTER 1 MINUTE:

[ ]  Mixture .... RICH

[ ]  Mags .... RECHECK

Radio Failure  [ABNORMAL]

[ ]  Volume .... CHECK

[ ]  Headset Connection .... CHECK

[ ]  Audio Controls .... CHECK

[ ]  Circuit Breakers .... CHECK

[ ]  Frequency .... SWITCH

[ ]  Transponder .... 7600

NOTE: If failure occurs in VFR conditions, or VFR conditions are encountered after failure, continue flight under VFR and land as soon as practicable.

VFR LANDING AT A CONTROLLED FIELD:

[ ]  Remain outside or above Class D airspace until traffic flow determined.

[ ]  Transmit intentions “in the blind” advising tower of aircraft type, position, altitude, and intention to land.

[ ]  Enter pattern, report position “in the blind”, and watch for light signals from tower.

NOTE: If failure occurs in IFR conditions, continue flight according to the following:

[ ]  Route to Fly (in order): (AVE. F) .... A – Assigned, V – Vectored, E – Expected, F – Filed

[ ]  Altitude (highest of for the route segment being flown): (MEA) .... M – Minimum IFR Altitude, E – Expected as Advised by ATC, A – Assigned by ATC

NOTE: Descent for Approach: From enroute altitude upon reaching the IAF but not before: 1. Expect-further-clearance time (if given) 2. ETA as calculated from filed ETE

ATC Light Signals

[ ]  Steady Green — On Deck .... Cleared for Takeoff

[ ]  Steady Green — Airborne .... Cleared to Land

[ ]  Flashing Green — On Deck .... Cleared for Taxi

[ ]  Flashing Green — Airborne .... Return for Landing (to be followed by steady green)

[ ]  Steady Red — On Deck .... STOP

[ ]  Steady Red — Airborne .... Give way to other aircraft and continue circling

[ ]  Flashing Red — On Deck .... Taxi clear of runway in use

[ ]  Flashing Red — Airborne .... Airport Unsafe – DO NOT LAND

[ ]  Flashing White — On Deck .... Return to starting point on airport

[ ]  Flashing White — Airborne .... Not Applicable

[ ]  Alternating Red and Green — On Deck .... Exercise extreme caution

[ ]  Alternating Red and Green — Airborne .... Exercise extreme caution

NOTE: Acknowledge all light signals by flashing landing light and/or rocking wings.

Abort  [EMERGENCY]

[ ]  Throttle .... IDLE

[ ]  Brakes .... AS REQUIRED

Engine Failure Immediately After Takeoff  [EMERGENCY]

[ ]  Airspeed .... 90 KIAS (flaps UP)

[ ]  Gear .... AS DESIRED

[ ]  Flaps .... AS DESIRED

[ ]  Fuel Shutoff Valve Handle .... OFF

[ ]  Battery & Mags .... OFF

[ ]  Canopy .... OPEN

Complete Engine Failure During Flight  [EMERGENCY]

[ ]  Airspeed .... 90 KIAS (gear & flaps UP)

Glide Ratio = 2.1 NM/1,000’ AGL

IF AIRSTART WARRANTED:

[ ]  Fuel Boost Pump .... ON

[ ]  Fuel Shutoff Valve Handle .... ON

[ ]  Mixture .... RICH

[ ]  Alternate Air .... ON

[ ]  Prop .... FULL INCREASE

[ ]  Throttle .... FULL FORWARD

[ ]  Mags .... CHECK ALL

[ ]  Battery .... ON

WARNING: When the condition of the landing surface is in doubt, it is recommended that the landing gear remain in the up position.

Forced Landing  [EMERGENCY]

[ ]  Airspeed .... 90 KIAS (gear & flaps UP)

[ ]  Squawk .... 7700

[ ]  Emergency .... DECLARE

[ ]  ELT .... ON

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Shutoff Valve Handle .... OFF

[ ]  Prop .... LOW RPM (pull aft)

[ ]  Mags .... OFF

[ ]  Gear .... AS REQUIRED

[ ]  Flaps .... AS DESIRED

[ ]  Battery .... OFF

[ ]  Generator .... OFF

[ ]  Harness .... LOCKED

[ ]  Canopy .... OPEN

Engine Roughness  [EMERGENCY]

[ ]  Alternate Air .... ON

IF ROUGHNESS CONTINUES AFTER 1 MIN:

[ ]  Alternate Air .... OFF

[ ]  Mixture .... ADJUST FOR MAX SMOOTHNESS

[ ]  Engine Gauges .... CHECK

[ ]  Mags .... L then R then BOTH

[ ]  Mags .... SMOOTHEST SETTING

[ ]  Power .... REDUCE

[ ]  Mixture .... FULL RICH

[ ]  LAND AT NEAREST AIRPORT .... LAND AT NEAREST AIRPORT

High Oil Temperature  [EMERGENCY]

[ ]  Power .... REDUCE

[ ]  Mixture .... RICH

[ ]  Airspeed .... INCREASE

IF HIGH TEMP PERSISTS:

[ ]  Land .... LAND AT NEAREST AIRPORT

Loss of Oil Pressure  [EMERGENCY]

[ ]  Oil Temp .... CHECK

[ ]  Power .... REDUCE

IF OIL TEMP SHOWS RISE, ENGINE FAILURE IMMINENT:

[ ]  Forced Landing .... EXECUTE

IF OIL TEMP DOES NOT SHOW RISE:

[ ]  Land .... LAND AT NEAREST AIRPORT

Prop Governor Failure  [EMERGENCY]

[ ]  Throttle .... REDUCE

[ ]  CLIMB TO LOAD PROP

[ ]  Prop .... ADJUST TO RESTORE GOVERNING

[ ]  Land .... LAND AS SOON AS POSSIBLE

Engine Fire During Starting  [EMERGENCY]

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Shutoff Valve Handle .... OFF

[ ]  Throttle .... FULL FORWARD

[ ]  Starter .... CONTINUE CRANKING

IF NO START:

[ ]  Mags .... OFF

[ ]  Battery .... OFF

Engine Fire on Ground  [EMERGENCY]

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Shutoff Valve Handle .... OFF

[ ]  Throttle .... FULL FORWARD

[ ]  Mags .... OFF

[ ]  Battery .... OFF

Engine Fire in Flight  [EMERGENCY]

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Shutoff Valve Handle .... OFF

[ ]  Throttle .... CLOSED

[ ]  Mags .... OFF

[ ]  Gear .... AS DESIRED

[ ]  Battery .... OFF

[ ]  Generator .... OFF

Electrical Fire  [EMERGENCY]

[ ]  Battery .... OFF

[ ]  Generator .... OFF

Wing Fire  [EMERGENCY]

[ ]  Battery .... OFF

[ ]  Generator .... OFF

[ ]  Sideslip .... AS REQUIRED

[ ]  Forced Landing .... EXECUTE

Smoke and Fumes in Cockpit  [EMERGENCY]

[ ]  Airspeed .... REDUCE

[ ]  Canopy .... OPEN

[ ]  Cockpit Air Handles .... FULL OUT

Generator Failure  [EMERGENCY]

[ ]  Generator .... OFF

[ ]  Nonessential Electrical Equipment .... SECURE

Vacuum System Failure  [EMERGENCY]

IF VMC:

[ ]  Attitude Indicator & Heading Indicator .... COVER

[ ]  Maintain VMC

IF IMC:

[ ]  Attitude Indicator & Heading Indicator .... COVER

[ ]  Fly using Turn-and-Slip Indicator, Altimeter, and Airspeed Indicator

[ ]  Declare .... EMERGENCY

Inoperative Trim  [EMERGENCY]

[ ]  Flaps .... AS REQUIRED

[ ]  Power .... AS REQUIRED

Flap Malfunction  [EMERGENCY]

[ ]  Circuit Breaker .... CHECK / RESET

IF BREAKER POPS AGAIN:

[ ]  Flaps .... LEAVE WHERE SET

Structural Icing  [EMERGENCY]

[ ]  Pitot Heat .... ON

[ ]  Alternate Air .... ON

[ ]  Prop .... FULL INCREASE

[ ]  Mixture .... BEST POWER

[ ]  Cabin Heat .... MAX DEFROST

[ ]  Exit Icing Conditions .... EXIT ICING CONDITIONS

IF LANDING WITH STRUCTURAL ICING:

[ ]  Flaps .... LEAVE UP

Landing Gear Emergency Extension  [EMERGENCY]

[ ]  Landing Gear Circuit Breaker .... PULL

[ ]  Gear Handle .... DOWN

[ ]  Clutch Knob .... UNLOCK

[ ]  Clutch Knob .... PUSH DOWN

[ ]  Emergency Landing Gear Crank .... CRANK UNTIL CANNOT CRANK FURTHER

IF PERFORMED FOR TRAINING:

[ ]  Clutch Knob .... PUSH UP

[ ]  Clutch Knob .... LOCK

[ ]  Landing Gear Circuit Breaker .... RESET

Landing Gear Emergencies  [EMERGENCY]

NOTE: If the gear cannot be lowered successfully, proceed with the emergency procedures for the appropriate gear malfunction.

CAUTION: • If an unsafe gear indication existed and the gear have been successfully lowered, do not attempt to raise the gear. • Raising the gear after a malfunction could cause further damage.

Landing with Gear Up  [EMERGENCY]

NOTE: If the gear fails to extend, a wheels-up landing can be made on either hard or soft ground; however a hard surface is preferable. To accomplish a gear-up landing, proceed as follows:

[ ]  Flaps .... FULL  (normal approach)

[ ]  Canopy .... OPEN

[ ]  Harness .... LOCKED

AFTER TOUCHDOWN:

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Shutoff Valve Handle .... OFF

[ ]  Battery .... OFF

Landing with One Main Gear Retracted  [EMERGENCY]

NOTE: Gear up landing is preferred to a landing with one main gear retracted.

[ ]  Have gear position checked visually by another pilot or by the tower.

[ ]  If verified that one gear is not fully extended and an attempt to retract is unsuccessful, execute a normal approach with full flaps and power on to reduce landing speed, carrying the wing slightly lower on the down-and-locked side.

[ ]  Canopy .... OPEN

[ ]  Harness .... LOCKED

[ ]  Touch down smoothly on the down-and-locked gear. Hold the opposite wing up with aileron as long as possible after the nosewheel touches down.

[ ]  When wingtip strikes the ground, apply maximum opposite brake pressure.

AS SOON AS AIRCRAFT STOPS:

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Shutoff Valve Handle .... OFF

[ ]  Battery .... OFF

Landing with Nose Gear Retracted  [EMERGENCY]

NOTE: Should the nose gear fail to extend fully:

[ ]  Flaps .... FULL

[ ]  Airspeed .... 70 KIAS

[ ]  Make gentle pitching oscillations to swing nose gear into down position.

[ ]  When landing, smoothly apply full forward stick to maintain pressure on the nose gear.

NOTE: Should the nose gear fail to extend:

[ ]  Make a normal approach.

[ ]  Canopy .... OPEN

[ ]  After touching main wheels down, hold the nose up as long as possible with full nose-down elevator trim and full backstick.

BEFORE THE NOSE SETTLES ONTO THE GROUND:

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Shutoff Valve Handle .... OFF

[ ]  Battery .... OFF

Landing with a Flat Main Tire  [EMERGENCY]

[ ]  Approach .... NORMAL

[ ]  Flaps .... FULL

[ ]  Touchdown .... GOOD TIRE FIRST

[ ]  Hold airplane off flat tire as long as possible

[ ]  Maintain directional control using good-tire brake

Landing with a Flat Nose Tire  [EMERGENCY]

[ ]  Approach .... NORMAL

[ ]  Flaps .... AS REQUIRED

[ ]  Touchdown .... ON MAINS

[ ]  Hold nosewheel off ground as long as possible

[ ]  When nosewheel touches down .... FULL UP ELEVATOR

Spin Recovery  [EMERGENCY]

MEMORY AID: PARE

[ ]  Power [MEMORY] .... Idle

[ ]  Ailerons [MEMORY] .... Neutral

[ ]  Rudder [MEMORY] .... Full opposite rotation direction

[ ]  Stick [MEMORY] .... Full increase with neutral ailerons

[ ]  Rudder .... Neutral when rotation stops

[ ]  Stick .... Pull smoothly from dive

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Beechcraft/T-34B Mentor/T-34BChecklist.txt

Rights: licensed

Checklist hash: e2b36fa98822e54b62c757ede13478701996d73894038b058f97b6d23b9cfc2c

Verify against the aircraft's own approved documentation before use.