Bell 206 _ 206B _ 206L Skywagon _ Super Skywagon — aid_smith_bell_206_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "After Landing / Shutdown" item count differs between passes (11 vs 10); verify nothing was dropped.

content_gap: Section "Other Emergencies" item count differs between passes (21 vs 20); verify nothing was dropped.

Initial

[ ]  Batt .... On

[ ]  Fuel .... Check

[ ]  Fuel Boost .... Out

[ ]  CBs

[ ]  Anti-Col .... On

[ ]  Batt .... Off

Exterior

[ ]  Surfaces .... Condition

[ ]  Batt .... Connected

[ ]  Levels .... Correct

[ ]  Hatches .... Secure

[ ]  Drains .... Clear

[ ]  Tx Temp Plate .... Colour Check

[ ]  Filters .... Check

[ ]  Filler Caps .... Secure

[ ]  G'box Mounts .... Check

[ ]  TR Drive .... Free splines

[ ]  Blades .... Free & positioned

Interior

[ ]  General .... Condition

[ ]  Fire Extinguisher .... Check

[ ]  Headsets .... Sufficient

[ ]  Lifejackets .... Sufficient

[ ]  Pax Brief Cards .... Sufficient

[ ]  Co-pilot Controls .... Removed?

[ ]  Co-pilot Harness .... Secure?

[ ]  Cabin Loading .... Correct

[ ]  Doors .... Secure

Pre-Start

[ ]  Harness .... Secure

[ ]  Pedals .... Adjusted

[ ]  Controls .... Full & Free

[ ]  Throttle .... Full & Free, Closed

[ ]  Avionics .... Off

[ ]  Electrics .... Off, except Anti-Col

[ ]  Batt .... Off (Extnl Start)

[ ]  Wander Light .... Off

[ ]  Heater .... Off

[ ]  Firewall Shutoff .... Open

[ ]  Rotor Brake .... Off

[ ]  CBs .... In except gyros

[ ]  Warnings .... Test

[ ]  TOT Light .... Test (if fitted)

[ ]  Fuel Boost Pumps .... Check & On

[ ]  OAT .... Note

[ ]  Instruments .... Check

[ ]  Fuel Valve .... On & Guarded

[ ]  Hyd Switch .... On

[ ]  Ant-Ice .... Off

[ ]  Landing Light .... Off

[ ]  Throttle .... Closed

[ ]  Collective .... Down

Engine Start

[ ]  Rotors .... Clear & Untied

[ ]  Starter .... Engage & Hold

[ ]  Oil Pressure .... Rising

[ ]  TOT .... <150 ºC

[ ]  By 25 % N1 .... Rotor Turning

[ ]  58 % N1 .... Release Starter

[ ]  Throttle .... Assist through yellow

[ ]  Ts & Ps .... Check

[ ]  60-62% N1 .... Idle for 1 min

[ ]  Ext Pwr .... Out/Batt On

[ ]  Throttle .... Open to 70 % N1

[ ]  Gen .... On, Check Load

[ ]  Caution CB .... In

[ ]  Avionics & Electrics .... On

[ ]  Hyd Switch .... Off, Check Runaway

Engine Run-Up

[ ]  Hyd System .... Check

[ ]  Anti-Ice .... Check

[ ]  Throttle .... Idle, N1 61-63% in 3 Secs

[ ]  Throttle .... Fully Open

Pre Take-Off

[ ]  CBs .... In

[ ]  Fuel .... Check

[ ]  Ts & Ps .... Check

[ ]  Instruments .... Set & Aligned

[ ]  Warning Lights .... Out

[ ]  Lights .... A/R

[ ]  Radios .... A/R

[ ]  Flotation Gear .... A/R

[ ]  Doors & Harnesses .... Secure

[ ]  Frictions .... Adjust

[ ]  Throttle .... Fully Open

After Take-Off

[ ]  Hover Power .... Check Perf

[ ]  Ts & Ps .... Check

[ ]  NR .... 100%

En-Route / Cruise

[ ]  Fuel .... Sufficient

[ ]  Warning Lights .... Clear

[ ]  Ts & Ps .... Check

[ ]  DG .... Aligned

[ ]  Altimeter .... Setting

[ ]  Anti-Ice .... A/R, check OAT

After Landing / Shutdown

[ ]  Collective .... Down

[ ]  Throttle .... Idle for 2 mins

[ ]  Frictions .... On

[ ]  Avionics & Electrics .... Off

[ ]  Anti-Ice & Heater .... Off

[ ]  Throttle (2 mins) .... Close, hold

[ ]  Boost Pumps .... Off

[ ]  30-38% .... Rotor Brake

[ ]  Batt .... Off

[ ]  Rotor .... Tie Down

BELL 206 CHECKLIST

Warning Lights  [ABNORMAL]

BATT HOT (TEMP > 140 ºF)

[ ]  Batt .... Off

[ ]  Land .... ASAP

ENGINE OUT (AUDIO, N1<55%)

[ ]  Autorotation .... Establish

[ ]  Investigate Failure .... If Time

[ ]  Passengers .... Warn

[ ]  EOL .... Complete

ROTOR LOW ROM (AUDIO)

[ ]  Pitch .... Reduce

[ ]  Throttle .... Check Open

TRANS TEMP (TEMP > RED LINE)

[ ]  Ts & Ps .... Check

[ ]  Power .... Reduce

[ ]  Land .... ASAP

TRANS TEMP (PX BELOW MINIMUM)

[ ]  Pressure Gauge .... Check

[ ]  Land .... ASAP

GEN FAIL

[ ]  Gen Switch .... Reset & On

[ ]  Loadshed .... A/R < 6000 ft

[ ]  Land .... ASA Practical

FUEL FILTER (CLOGGED)

[ ]  Land .... ASA Practical

[ ]  Filter .... Clean

CHIP - ENG, TRANS OR T/R

[ ]  Land .... ASAP

BATT TEMP (TEMP > 130ºF)

[ ]  Batt .... Off until Light Out

FUEL PUMP (ONE OR BOTH INOPERATIVE)

[ ]  CBs .... Check

SINGLE FAIL:

[ ]  Altitude .... Below 6000 ft

[ ]  Land .... ASA Practical

DOUBLE FAIL:

[ ]  Land .... ASAP

Other Emergencies  [EMERGENCY]

ENGINE FIRE ON GROUND

[ ]  Throttle .... Close to Shutoff

[ ]  Starter .... Motor

[ ]  Fuel Valve .... Off

[ ]  Ignition CB .... Out

ENGINE FIRE IN FLIGHT

[ ]  Throttle .... Close to Shutoff

[ ]  Autorotation .... Establish

[ ]  Passengers .... Warn

[ ]  Fuel Valve .... Off

[ ]  Boost Pumps .... Off

[ ]  R/T .... Transmit Mayday

[ ]  EOL .... Complete

HYDRAULIC SYSTEM FAILURE

[ ]  Airspeed .... 61 - 69 kts

[ ]  Hydraulic CB .... Out

NOT RESTORED:

[ ]  Hydraulic CB .... In

[ ]  Hyd Switch .... Off

[ ]  Land .... ASA Practical

BELL 206 CHECKLIST

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Bell/206 _ 206B _ 206L Skywagon _ Super Skywagon/aid_smith_bell_206_checklist.txt

Rights: licensed

Checklist hash: e87d6f61b5b4ad4b0b7b2bee1f759212ba38a5b99a448cfc15115859bb4e27a8

Verify against the aircraft's own approved documentation before use.