Bell 206 _ 206B _ 206L Skywagon _ Super Skywagon — Eric_Triantafilo_Cessna_206_Super_Skywagon_Checklists

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "ENGINE FAILURE" item count differs between passes (16 vs 15); verify nothing was dropped.

BEFORE START

[ ]  EXTERIOR CHECK .... COMPLETE

[ ]  SEATS & SEAT BELTS .... ADJUST & LOCK

[ ]  DOOR .... LOCKED

[ ]  COWL FLAPS .... OPEN

[ ]  CIRCUIT BREAKERS .... CHECK

[ ]  MASTER .... ON

[ ]  FUEL SELECTOR .... FULLEST TANK

[ ]  RADIO SWITCHES .... OFF

START

[ ]  MIXTURE .... FULL RICH

[ ]  PROPELLER .... HIGH RPM

[ ]  THROTTLE .... FULL

[ ]  AUX FUEL PUMP .... LO & PRIME WITH HI

[ ]  THROTTLE .... ½ INCH OPEN

[ ]  STARTER .... ENGAGE

[ ]  THROTTLE .... 1000 RPM

[ ]  OIL PRESSURE .... CHECK

[ ]  AUX FUEL PUMP .... OFF

[ ]  LIGHTS .... AS REQUIERED

[ ]  TIME .... CHECK

BEFORE TAKE OFF

[ ]  INDUCTION AIR .... CLOSED

[ ]  COWL FLAPS .... OPEN

[ ]  FLIGHT CONTROLS .... CHECK

[ ]  ELEV & RUDDER TRIMS .... SET

[ ]  FLAPS .... CHECK & SET

[ ]  FLT INSTR. & RADIOS .... SET

[ ]  ENGINE INSTR. .... CHECK GREEN

[ ]  THROTTLE .... 1700 RPM

[ ]  MIXTURE .... CHECK

[ ]  MAGNETOS .... CHK (50 RPM MAX DIF.)

[ ]  PROPELLER .... CYCLE TO CHECK

[ ]  AMMETER .... CHECK

[ ]  SUCTION .... CHECK

[ ]  DOORS & WINDOW .... CLOSED & LOCKED

[ ]  LAND. & STROBE LTS .... ON

[ ]  TIME .... CHECK

AFTER LANDING

[ ]  COWL FLAPS .... OPEN

[ ]  FLAPS .... UP

[ ]  LANDING LIGHTS .... OFF

[ ]  TRANSPONDER .... STAND BY

SHUT OFF

[ ]  LIGHTS & RADIOS .... OFF

[ ]  MIXTURE .... IDLE CUT-OFF

[ ]  MAGNETOS .... OFF

[ ]  MASTER SWITCH .... OFF

[ ]  TIME .... CHECK

GENERAL SPECS

NOTE: Vx    78 MPH

NOTE: V l/d 80 MPH

NOTE: Vy    100 MPH

NOTE: Va    138 MPH

NOTE: V l/d 80 MPH

NOTE: Ground Run 380 mts.

NOTE: T.O. Dist over 50´    680 mts.

NOTE: Ground Roll      300 mts.

NOTE: L. Dist over 50´ 540 mts.

NOTE: Fuel Capacity_____________80 US GALLONS

TAKE OFF

[ ]  THROTTLE .... FULL (2700 RPM)

[ ]  ACCELERATE TO 70MPH .... ROTATE

[ ]  CLIMB SPEED .... 100 MPH

CLIMB

[ ]  FLAPS .... UP

[ ]  AIRSPEED .... 110 TO 120 MPH

[ ]  POWER .... 24,5´ X  2500 RPM

[ ]  MIXTURE .... LEAN

[ ]  COWL FLAPS .... AS REQUIRED

[ ]  LANDING LIGHTS .... OFF

CRUISE

[ ]  CRUISE POWER .... SET

[ ]  COWL FLAPS .... AS REQUIRED

[ ]  MIXTURE .... LEAN

DESCENT

[ ]  MIXTURE .... RICH

[ ]  POWER .... AS DESIRED

[ ]  COWL FLAPS .... CLOSED

BEFORE LANDING

[ ]  FUEL SELECTOR .... FULLEST TANK

[ ]  COWL FLAPS .... CLOSED

[ ]  AIRSPEED .... 90-100 MPH (FLAPS UP)

[ ]  LANDING LIGHTS .... ON

[ ]  FLAPS .... SET (BELOW 110MPH)

[ ]  MIXTURE .... RICH

[ ]  PROPELLER .... HIGH RPM

[ ]  AIRSPEED .... 80-90 MPH (FLAPS DOWN)

ENGINE FAILURE  [EMERGENCY]

[ ]  AIRSPEED .... 80 MPH

[ ]  MIXTURE .... RICH

[ ]  INDUCTION AIR .... OPEN

[ ]  PROPELLER .... HIGH RPM

[ ]  FUEL SELECTOR .... OPPOSITE TANK

[ ]  FUEL PUMP .... HI

[ ]  MAGNETOS .... BOTH

[ ]  ELECTRICAL LOAD .... REDUCE

[ ]  RESTART .... ATTEMP

NOTE: - IF RESTART IS NOT POSSIBLE, FIND A SUITABLE

NOTE: FIELD TO LAND ON AND PROCEED AS FOLLOWS -

[ ]  PASSENGERS .... BRIEF

[ ]  FLAPS .... 40º

[ ]  APPROACH SPEED .... 70 MPH

[ ]  LAND ON MAIN WHEELS FIRST, LOWER THE NOSE

[ ]  AND APPLY BRAKES HEAVILY.

MAX PERFORMANCE TAKE OFF

[ ]  FLAPS .... 20º

[ ]  BRAKES .... APPLY

[ ]  THROTTLE .... FULL (2700 RPM)

[ ]  BRAKES .... RELEASE

[ ]  ELEVATOR .... SLIGHTLY NOSE UP ATT.

[ ]  MAINTAIN Vx (78 MPH) UNTIL OBST. ARE CLEARED

[ ]  ACCELERATE TO 100 MPH THEN RETRACT FLAPS

[ ]  COWL FLAPS .... OPEN

CAUTION: DO NOT REDUCE POWER BEFORE FLAPS RETRACTION

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Bell/206 _ 206B _ 206L Skywagon _ Super Skywagon/Eric_Triantafilo_Cessna_206_Super_Skywagon_Checklists.txt

Rights: licensed

Checklist hash: c05e625abc384526364050c9d9cc70f2d429d2963684aff8299eb09d698c9971

Verify against the aircraft's own approved documentation before use.