Bellanca 7GCAA Citabria — laura_laster_7GCAA_chk_citabria

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Preflight

CABIN

[ ]  Magneto Switches .... off

[ ]  Master and Electrical Switches .... off

[ ]  Radio Switches .... off

[ ]  Flight Controls .... unlock and check

[ ]  Fuel Quantity Gauge .... check

RIGHT WING

[ ]  Fuel Sump .... drain

[ ]  Aileron .... check

[ ]  Navigation Lights .... check

[ ]  Pitot and Static Tube .... check

[ ]  Tiedown .... remove

[ ]  Fuel Quantity .... check quantity

[ ]  Fuel Caps .... secure

[ ]  Tire .... check

[ ]  Brakes .... check

NOSE

[ ]  Oil Quantity .... at least 6 quarts

[ ]  Engine Compartment .... check

[ ]  Windshield and Windows .... clean

[ ]  Propeller and Spinner .... check for nicks, security

[ ]  Fuel Sump .... drain

[ ]  Cowling .... secure

LEFT WING

[ ]  Fuel Sump .... drain

[ ]  Brakes .... check

[ ]  Tire .... check

[ ]  Fuel Quantity .... check quantity

[ ]  Fuel Caps .... secure

[ ]  Fuel Vent .... clear

[ ]  Tiedown .... remove

[ ]  Navigation Lights .... check

[ ]  Aileron .... check

EMPENNAGE

[ ]  Fuel Sump .... drain

[ ]  Antennas .... secure

[ ]  Control Surfaces, Hinges, and Attachments .... check

[ ]  Control Cables .... check

[ ]  Rudder Lock .... remove

[ ]  Tailwheel Assembly .... check

[ ]  Tire .... check

ENTIRE AIRCRAFT

[ ]  Inspection Covers .... secure

[ ]  Fabric Integrity .... check

Before Starting

[ ]  Passenger Briefing .... complete

[ ]  Seat Belts .... on

[ ]  Brakes .... set

NOTE: hold the brakes with your feet. Do not set the parking brake.

[ ]  Fuel .... on

[ ]  Mixture .... rich

[ ]  Prime as required

[ ]  Primer .... locked

[ ]  Radio and Electrical Switches .... off

[ ]  Master Switch .... on

[ ]  Beacon .... on

[ ]  Control Stick .... full aft

[ ]  Propeller Area .... clear

[ ]  Left Magneto Switch .... on

[ ]  Throttle .... open ¼ to ½ inch

[ ]  Starter .... engage

[ ]  Power .... 800 – 1000

[ ]  Right Magneto Switch .... on

[ ]  Oil Pressure .... check

[ ]  Mixture .... lean for taxi

[ ]  Altimeter .... set

[ ]  Brakes .... test

[ ]  Radio on, call for taxi

Before Takeoff

[ ]  Brakes .... set

[ ]  Mixture .... rich

[ ]  Control Stick .... full aft

[ ]  Power .... 2000 rpm

[ ]  Magnetos .... check

NOTE: Max drop 175 rpm, max difference 50 rpm

[ ]  Carb Heat .... check

[ ]  Engine Instruments .... check

[ ]  Power .... Reduce 800 – 1000

[ ]  Controls .... free and correct

[ ]  Trim .... set for takeoff

[ ]  Door and Windows .... latch

[ ]  Takeoff Performance .... review

[ ]  Engine Failure Brief .... complete

[ ]  Transponder .... on altitude

[ ]  Radio call for takeoff

[ ]  Landing Light .... on

Cruise

[ ]  Power .... 2400 rpm

[ ]  Trim .... set

[ ]  Mixture .... set

[ ]  Landing Light .... off

Before Landing

[ ]  Seat Belts .... secure

[ ]  Landing Light .... on

[ ]  Carb Heat .... check

[ ]  Mixture .... set

[ ]  Fuel .... on

After Landing

[ ]  Landing Light .... off

[ ]  Transponder .... off

[ ]  Mixture .... lean for taxi

Shutdown

[ ]  Radios and Electrical Switches (except beacon) .... off

[ ]  Mixture .... idle cutoff

[ ]  Magneto Switches .... off

[ ]  Beacon .... off

[ ]  Master Switch .... off

[ ]  Aircraft and Controls .... secure

Engine Fire During Start  [EMERGENCY]

[ ]  Starter .... crank engine

[ ]  Mixture .... idle cutoff

[ ]  Throttle .... open

[ ]  Fuel Selector .... off

[ ]  Abandon if fire continues

Engine Power Loss During Takeoff  [EMERGENCY]

[ ]  If sufficient runway remains for a normal landing, land straight ahead.

[ ]  If insufficient runway remains, maintain safe airspeed and make only shallow turns to avoid obstructions.

NOTE: If sufficient altitude has been gained to attempt a restart:

[ ]  Airspeed .... 80 mph

[ ]  Fuel Selector .... check on

[ ]  Mixture .... check rich

[ ]  Throttle .... full

[ ]  Carburetor Heat .... on

[ ]  Primer .... locked

[ ]  If power is not regained, proceed with Power Off Landing.

Engine Power Loss In Flight  [EMERGENCY]

[ ]  Fuel Selector .... check on

[ ]  Mixture .... rich

[ ]  Carburetor Heat .... on

[ ]  Engine Gauges .... check for indication

[ ]  Primer .... check locked

[ ]  Throttle .... full

NOTE: When power is restored:

[ ]  Carburetor Heat .... off

[ ]  If power is not restored, prepare for Power Off Landing.

Power Off Landing  [EMERGENCY]

[ ]  Locate suitable field.

[ ]  Establish spiral pattern 1000 ft. above field at downwind position for normal landing approach. When field can easily be reached, slow to 80 mph for shortest landing.

NOTE: When committed to landing:

[ ]  Ignition .... off

[ ]  Master Switch .... off

[ ]  Fuel Selector .... off

[ ]  Mixture .... idle cutoff

[ ]  Seat belt and harness .... tight

Fire In Flight  [EMERGENCY]

[ ]  Source of Fire .... check

ELECTRICAL FIRE (SMOKE IN CABIN):

[ ]  Master Switch .... off

[ ]  Cabin Heat .... off

[ ]  Land as soon as practicable.

ENGINE FIRE:

[ ]  Fuel Selector .... off

[ ]  Throttle .... closed

[ ]  Mixture .... idle cutoff

[ ]  Heater .... off

[ ]  Magnetos .... off

[ ]  Primer .... locked

[ ]  Proceed with Power Off Landing procedure.

Loss of Oil Pressure  [EMERGENCY]

[ ]  Land as soon as possible and investigate cause. Prepare for Power Off Landing.

High Oil Pressure  [EMERGENCY]

[ ]  Land at nearest airport and investigate the problem. Prepare for Power Off Landing.

Alternator Failure  [EMERGENCY]

[ ]  Verify failure.

[ ]  Reduce electrical load as much as possible.

[ ]  Alternator Circuit Fuses .... check

[ ]  Master Switch .... off (for 1 second) then on

NOTE: If no output:

[ ]  Master Switch .... off

[ ]  Reduce electrical load and land as soon as possible.

Spin Recovery  [EMERGENCY]

[ ]  Throttle .... idle

[ ]  Ailerons .... neutral

[ ]  Rudder .... full opposite direction of rotation

[ ]  Control Stick .... forward

[ ]  Smoothly regain level flight attitude.

Engine Roughness  [EMERGENCY]

[ ]  Carburetor Heat .... on

NOTE: If roughness continues after one minute:

[ ]  Carburetor Heat .... off

[ ]  Mixture .... adjust for max. smoothness

[ ]  Fuel Selector .... check on

[ ]  Engine Gauges .... check

[ ]  Magneto Switches .... “L” then “R” then both

NOTE: if operation is satisfactory on either one, continue on that magneto at reduced power and full “rich” mixture to first airport.

[ ]  Prepare for Power Off Landing.

Carburetor Icing  [EMERGENCY]

[ ]  Carburetor Heat .... on

[ ]  Mixture .... adjust for max. smoothness

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Bellanca/7GCAA Citabria/laura_laster_7GCAA_chk_citabria.txt

Rights: licensed

Checklist hash: d9c5a809465221ebf74867ba1697826045aa4102ddf68150c45d8be67d2b8910

Verify against the aircraft's own approved documentation before use.