Bellanca 8GCBC Scout — Sam_Dawson_8gcbc_inflight_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Turn- Time-Twist-Tap-Throttle-Talk" item count differs between passes (4 vs 1); verify nothing was dropped.

content_gap: Section "Turn-Time-Twist-Throttle-Talk" item count differs between passes (4 vs 1); verify nothing was dropped.

Final Before Take Off Check

[ ]  1. Heading .... Check aligned w/ runway

[ ]  2. Altimeter .... Field elevation

[ ]  3. Transponder .... ON

[ ]  4. Time .... Noted

[ ]  5. Strobes, lights .... All on

[ ]  6. Landing & TO lights .... On when cleared for Takeoff

Climb Check

[ ]  1. Throttle .... FULL OPEN

[ ]  2. Mixture .... Lean

[ ]  3. Airspeed .... 70-85 MPH

Cruise

[ ]  1. Level Off .... TRIM

[ ]  2. Airspeed .... ACCELERATE

[ ]  3. Power .... SET to cruise

[ ]  4. Mixture .... LEAN when below 75%

[ ]  5. Taxi/Landing Lights .... Off

In Range-Descent

[ ]  1. ATIS/AWOS/ASOS .... Received

[ ]  2. Altimeter .... Set

[ ]  4. Seat Belts .... secured

[ ]  6. Mixture .... SET

[ ]  7. Landing Light/Taxi Light .... ON

[ ]  8. Avionics .... SET

[ ]  9. Approach Briefing .... Complete

Turn-Time-Twist-Throttle-Talk

NOTE: Vx-58, Vy-80, Cruise Climb- 75-80, Va-118

NOTE: Vref-70-75, Vref short- 60, Vfe- 101,

NOTE: Vfret- 60 (all speeds MPH)

NOTE: 14 July 2007 Blackhawk Aviation, El Paso, TX

Approach Brief

[ ]  Approach .... Type, Runway

[ ]  Frequency .... Set, ID

[ ]  Final Approach Course .... CDI Set

[ ]  FAF

[ ]  DH/MDA

[ ]  Approach Lights/PAPI/VASI .... Pilot activated?

[ ]  TDZE, HAT/HAA

[ ]  MAP

[ ]  Missed Approach

[ ]  Notes

[ ]  Runway exit/taxi plan

[ ]  Terrain

[ ]  Min. Alt., MAP & Climb Out .... Complete

Before Landing

[ ]  1. Seat Belts .... Fastened

[ ]  2. Mixture .... RICH

[ ]  3. Breaks .... CHECK FIRM

[ ]  4. Approach Speed .... 70-75 MPH

GO AROUND

[ ]  1. Throttle .... Full

[ ]  2. Carburetor Heat .... Cold

[ ]  3. Wing Flaps .... 20° Immediately

[ ]  4. Climb Speed .... 62 KIAS

[ ]  5. Wing Flaps .... 10° (until obstacles clear), RETRACT (safe altitude & 60 KIAS)

After Landing

[ ]  1. Transponder .... Stby

[ ]  3. Strobes/lights .... Off as necessary

[ ]  4. Electric Fuel Pump .... OFF

[ ]  5. Taxi Clearance .... Obtain

Turn- Time-Twist-Tap-Throttle-Talk

NOTE: Vx-58, Vy-80, Cruise Climb- 75-80, Va- 118,

NOTE: Vref-70-75, Vref short- 60, Vfe-101, Vfret-

NOTE: 60 (all speeds MPH)

NOTE: 14 July 2007  Blackhawk Aviation, El Paso, Texas

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Bellanca/8GCBC Scout/Sam_Dawson_8gcbc_inflight_checklist.txt

Rights: licensed

Checklist hash: 3899b02249871e02e324fe6b6718a2dd0dae7f08eff49a58581c46b397c86d48

Verify against the aircraft's own approved documentation before use.