Bellanca 8KCAB Decathlon — Sam_Dawson_8kcab_ground_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Shutdown and Securing" item count differs between passes (14 vs 13); verify nothing was dropped.

content_gap: Section "Starting Engine" item count differs between passes (10 vs 13); verify nothing was dropped.

content_gap: Section "Taxi" item count differs between passes (3 vs 2); verify nothing was dropped.

Before Starting Engine

[ ]  Preflight Inspection .... Complete

[ ]  CRM/Passenger Briefing .... Complete

[ ]  Seats and Seat Belts .... Adjust & Lock

[ ]  Fuel Shut-Off Valve .... ON

[ ]  Brakes .... Set

[ ]  Alternate Air .... COLD

[ ]  Electrical and Radio Switches .... OFF

[ ]  Propeller Control .... FULL INCREASE

[ ]  Circuit Breakers .... Check In

Starting Engine

[ ]  Master Switch .... On

[ ]  Engine Prime (as Required) .... Mixture- RICH / Throttle- ¼-½” / Fuel Pump- ON until fuel pressure; then OFF

[ ]  Mixture .... IDLE CUT OFF

[ ]  Throttle .... ½-1” Open

[ ]  Magneto Switches .... BOTH

[ ]  Propeller Area .... CLEAR

[ ]  Starter .... ENGAGE

[ ]  Mixture .... FULL RICH

[ ]  Throttle .... 1000-2000 RPM

[ ]  Oil Pressure .... CHECK

Before Taxi

[ ]  Mixture .... Lean for taxi

[ ]  Throttle .... 1000 RPM

[ ]  Electrical and Radio Switches .... On as needed

[ ]  ATIS / AWOS .... Receive

[ ]  Clearance .... Obtain

[ ]  Avionics / Transponder .... Set / STBY

[ ]  Flight Instruments .... Check & Set

[ ]  Engine Instruments .... Check

Taxi

[ ]  Brakes & Steering .... Check

[ ]  Flight Instruments .... Check

NOTE: 11 July 2007 Blackhawk Aviation, El Paso, TX

Before Takeoff

[ ]  Seat Belts .... Secured

[ ]  Flight Controls .... Free & Correct

[ ]  Clock .... Check

[ ]  Fuel Quantity .... Check

[ ]  Flight Instruments .... Check & Set

[ ]  Radios / Avionics .... Set for Departure

[ ]  Fuel Shut off Valve .... ON

[ ]  Elevator Trim .... Check / Set

[ ]  Mixture .... Enrich

[ ]  Throttle .... 1800 RPM

[ ]  Mixture .... Set for Takeoff

[ ]  Magnetos .... Check (-175±50)

[ ]  Propeller .... Check (-300-500)

[ ]  Engine Instruments / Ammeter .... Check

[ ]  Vacuum .... Green Arc

[ ]  Throttle .... Idle / 1000 RPM

[ ]  Electrical Fuel Pump .... ON

[ ]  Cabin Doors .... Closed & Locked

[ ]  Lights (except taxi/takeoff) .... ON

Shutdown and Securing

[ ]  Avionics .... Off

[ ]  Exterior Lights (except beacon) .... Off as Required

[ ]  Throttle .... 1500 RPM

[ ]  Mixture .... Lean for 15 secs.

[ ]  Throttle .... 1000 RPM

[ ]  Mixture .... Idle Cut Off

[ ]  Magneto Switches .... Off

[ ]  Master Switch .... Off

[ ]  Fuel Shutoff Valve .... OFF

[ ]  Controls .... Secure w/ lapbelt

[ ]  Post Flight Inspection .... Accomplish

[ ]  Internal and External Covers .... Install

[ ]  Tie downs and Chocks .... Install

NOTE: 11 July Blackhawk Aviation, El Paso, TX

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Bellanca/8KCAB Decathlon/Sam_Dawson_8kcab_ground_checklist.txt

Rights: licensed

Checklist hash: c27c13bab94f2797ed4a3ce0b921baa633ba5861fb1225398334f1f1647e9a26

Verify against the aircraft's own approved documentation before use.