Bellanca 8KCAB Decathlon — Sam_Dawson_8kcab_InFlight_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "After Landing" item count differs between passes (8 vs 4); verify nothing was dropped.

content_gap: Section "In Range-Descent" item count differs between passes (9 vs 7); verify nothing was dropped.

content_gap: Section "Turn-Time-Twist-Throttle-Talk" item count differs between passes (2 vs 3); verify nothing was dropped.

Final Before Take Off Check

[ ]  Heading .... Check aligned w/ runway

[ ]  Altimeter .... Field elevation

[ ]  Transponder .... ON

[ ]  Time .... Noted

[ ]  Strobes, lights .... All on

[ ]  Landing & TO lights .... On when cleared for Takeoff

Climb Check

[ ]  Throttle .... FULL OPEN

[ ]  Propeller Control .... FULL INCREASE

[ ]  Throttle .... Full

[ ]  Mixture .... Lean

[ ]  Airspeed .... 70-85 MPH

[ ]  Electrical Fuel Pump .... OFF after safe alt.

Cruise

[ ]  Level Off .... TRIM

[ ]  Airspeed .... ACCELERATE

[ ]  Power .... SET to cruise

[ ]  Mixture .... LEAN when below 75%

[ ]  Taxi/Landing Lights .... Off

In Range-Descent

[ ]  ATIS/AWOS/ASOS .... Received

[ ]  Altimeter .... Set

[ ]  Seat Belts .... secured

[ ]  Mixture .... SET

[ ]  Landing Light/Taxi Light .... ON

[ ]  Avionics .... SET

[ ]  Approach Briefing .... Complete

Turn-Time-Twist-Throttle-Talk

NOTE: Vx-58, Vy-80, Cruise Climb- 75-80, Va-121 (normal) 132(acrobatic), Vref-70-75, Vref short- 60 (all speeds MPH)

NOTE: 11 July 2007 Blackhawk Aviation, El Paso, TX

Approach Brief

[ ]  Approach

[ ]  Frequency

[ ]  Final Approach Course

[ ]  FAF

[ ]  DH/MDA

[ ]  Approach Lights/PAPI/VASI

[ ]  TDZE, HAT/HAA

[ ]  MAP

[ ]  Missed Approach

[ ]  Notes

[ ]  Runway exit/taxi plan

[ ]  Terrain

[ ]  Min. Alt., MAP & Climb Out

Before Landing

[ ]  Seat Belts .... Fastened

[ ]  Propeller .... FULL increase

[ ]  Mixture .... RICH

[ ]  Electrical Fuel Pump .... ON

[ ]  Breaks .... CHECK FIRM

[ ]  Approach Speed .... 70-75 MPH

GO AROUND  [ABNORMAL]

[ ]  Throttle .... Full

[ ]  Carburetor Heat .... Cold

[ ]  Wing Flaps .... 20° Immediately

[ ]  Climb Speed .... 62 KIAS

[ ]  Wing Flaps .... 10° (until obstacles clear), RETRACT (safe altitude & 60 KIAS)

After Landing

[ ]  Transponder .... Stby

[ ]  Strobes/lights .... Off as necessary

[ ]  Electric Fuel Pump .... OFF

[ ]  Taxi Clearance .... Obtain

NOTE: Turn-Time-Twist-Tap-Throttle-Talk

NOTE: Vx-58, Vy-80, Cruise Climb- 75-80, Va-121 (normal) 132 (acrobat), Vref-70-75, Vref short- 65 (all speeds MPH)

NOTE: 11 July 2007 Blackhawk Aviation, El Paso, Texas

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Bellanca/8KCAB Decathlon/Sam_Dawson_8kcab_InFlight_Checklist.txt

Rights: licensed

Checklist hash: 87cd77931d7e29e140c17663d25a375e5c1fb30fd4cf5bafbb7dd240c4f3551b

Verify against the aircraft's own approved documentation before use.