BelleStar Aerostar RX8 — Kent_J_Barnes_Aerostar_RX8_8-2006_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Flight Planning Check

[ ]  Call Weather: 800-992-7433 (Surface Winds, Winds Aloft SFC>12K and Synopsis)

[ ]  Plan Flight: Course, Weight, Performance

[ ]  Determine Where To Meet: Call Crew, Advise What To Wear

[ ]  Check Fuel – Balloon, Vehicles & Fans

[ ]  Load Equipment: Burners and Pins, 2 Strikers, Other equip as needed

[ ]  Instruments (extra batteries), Radios, GPS, Pibals (Helium)

[ ]  Nitro Tanks – If temperature at take of site < 45 degrees F

Layout and Inspection Check

[ ]  Brief Crew – Duties & Positions

[ ]  Layout

[ ]  Inflation

[ ]  Prior Liftoff

[ ]  Flight Plan

[ ]  Determine Surface Wind Direction & Launch Pibal

[ ]  Check Field, Sweep Area For Obstructions

[ ]  Place Basket

[ ]  Check Fuel Supply - Test Liquid Level Gauge Valve

[ ]  Check Airworthiness, Registration, Flight Manual and Strikers

[ ]  Lay Basket Down

[ ]  Align Envelope Downwind (25’-40’ Apart)

[ ]  Connect Burner & Fuel Lines – Leak Check (Listen and Smell)

[ ]  Check Fuel Pressure for Proper Pressures on Both Tanks/Burners

[ ]  Brief Crew on Inflation Process

[ ]  Pull Throat Out Of Bag To Basket (Re-Evaluate Wind Direction)

[ ]  Connect Suspension Lines

[ ]  Run Control Lines Out and Straighten

[ ]  Lay Out Remainder of Envelope and Crown Line* (Brief Crown Person) For BelleStar “Bag Inflate” is preferred.

[ ]  Initial Fan Alignment

[ ]  Tie Off - If Required

NOTE: If in “short crew” mode, wait until fan is started to pull envelope out of bag. Remember to put fan back to side rail depth before hot inflation.

Inflation Check

[ ]  Brief Crew (Give Instructions On Fan Operation)

[ ]  Start Fan and Inflate

[ ]  Walk Around and:

[ ]  Pull Top Cap Out From Upper, Middle Cord, Align With Corresponding Tab.

[ ]  Pull Parachute Cord Slack

[ ]  Begin Tabbing

[ ]  Inspect Temp Tab – < 275 degrees F

[ ]  Inspect Internal Parachute Cords, Connections For Tangles

[ ]  Check Envelope for Center Alignment – BelleStar Blue is UP

[ ]  Pull Out Fabric On Down Side While Traveling To Throat (Be Sure To Evaluate Which Side To Pull From)

[ ]  Look In Throat From Behind Basket For Cable Alignment and Top Cap Seal

[ ]  Clear Control Line From Inside Throat and Secure

[ ]  Check Cable Connections for Safety Locks/Washers

[ ]  Adjust Fan Alignment As Necessary (Pull Out Appropriate Side(s), Direct Crown Person)

[ ]  Finish Tabbing

[ ]  Final Adjustments, Wait Until Balloon Is Cold Filled

[ ]  Signal Crown Person To Prepare For Hot Inflate

[ ]  Light Pilot and Light Both Burners

[ ]  Gimbal Burner Up To Opening, Adjust For Fan Current, Inflate

[ ]  Keep Weight On Basket When Vertical

[ ]  Tie Crown Line Off

[ ]  Pull Control Lines Down and Check For Tangles

[ ]  Check Envelope, Skirt, Valve(s) & Suspension Lines for Damage

Pre-launch Check

[ ]  Check Cables, Connections & Strikers

[ ]  Load Passengers

[ ]  Connect Instruments

[ ]  Radios and Instruments – On & Check

[ ]  Check Parachute Top and Rigging – Reset If Necessary

[ ]  Brief Passengers and Final Crew Instructions

[ ]  Welcome Onboard, My Name Is…

[ ]  At Landing, “Don’t Get Out Of The Basket Until I Instruct You To Do So”

[ ]  While In Flight, Please Don’t Touch This ….

[ ]  At Times I May Ask You To Move ….

[ ]  At Landing, Please Face Forward, Hang On In 2 Places, Bend Knees, Cameras, Purses Etc Stowed

[ ]  Are there Any Questions at This Time?

[ ]  Recheck Wind Conditions & Obstacles

[ ]  Ask Crew To Move Adjacent Equipment & To Help Weigh-Off.

[ ]  Pop Quick Release, If Appropriate.

[ ]  Take Off With Intent To Clear Obstacles.

Landing Check

[ ]  Re-Brief Passengers To Stay In Basket Until Directed Otherwise

[ ]  Face Direction of Travel

[ ]  Bend Knees – Feet Together

[ ]  Where to Hold On – 2 Places

[ ]  “Stay in Basket Until Pilot OKs to Exit!”

[ ]  Check Deflation Area for Obstacles.

[ ]  Exit Passengers When Appropriate

[ ]  Re-Evaluate Wind Direction

[ ]  Reposition Balloon In Largest Portion Of Field

[ ]  Line Up Scoop/Basket For Downwind Deflation

[ ]  Deploy Crown Line With Briefed Crewmember

[ ]  Shutoff Pilots & Fuel Valves; Cool Burners

[ ]  Get Out Of Basket – If Desired

[ ]  Rip Envelope Out And Immediately Begin Tipping Basket Over As Envelope Starts To Fall

Securing Balloon Check

[ ]  Turn Off Radios, Instruments and Stow

[ ]  Burp Envelope with EZ-Squeeze

[ ]  Disconnect Suspension Cables

[ ]  Fold Envelope Into Bag and Secure

[ ]  Disconnect Fuel Lines From Burner and Connect Filler Connection

[ ]  Remove and Stow Burner

[ ]  Turn Basket Upright – Prepare to Load

[ ]  Straps on Envelop, Fan and Basket

Emergency Checklists  [EMERGENCY]

BURNER FIRE

[ ]  Shut OFF Main Valve to Affected Burner

[ ]  Drain Fuel Line

[ ]  If Necessary to Burn – Use Tank Valve for Altitude Control

[ ]  Land As Soon As Practical

PILOT LIGHT FAILURE

[ ]  If Available – Use Other Burner

[ ]  Vent Burner – Stop then Light with Striker

[ ]  If Necessary to Burn – Use Metering Valve as Backup

[ ]  Land As Soon as Practical

BLAST VALVE LEAK

[ ]  Shut OFF Main Valve to Affected Burner

[ ]  Drain Fuel Line

[ ]  If Available – Use Other Burner

[ ]  If Necessary to Burn – Use Metering Valve as Backup

[ ]  Land As Soon as Practical

LOW FUEL

[ ]  Confirm Levels in ALL Tanks

[ ]  If Available – Run ONLY ONE Tank While Flying to Landing

[ ]  Use Fullest Tank For Final Approach & Landing

[ ]  Land As Soon As Practical

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: BelleStar/Aerostar RX8/Kent_J_Barnes_Aerostar_RX8_8-2006_Checklist.txt

Rights: licensed

Checklist hash: 82e1fa5bc776115b7b8169619ee000705a0529d7f667aa7559888e376aaff5db

Verify against the aircraft's own approved documentation before use.