Cessna 150 Commuter — aviatickyklub.cz_cessna_c150_engl_checklists

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

BEFORE STARTING ENGINE

[ ]  Exterior Preflight .... COMPLETE.

[ ]  Tach/Hobbs Meter .... RECORD

[ ]  Seats, Belts, Harnesses .... ADJUST & LOCKED.

[ ]  Fuel Shutoff Valve .... ON.

[ ]  Radios & Electrical Equipment .... OFF.

[ ]  Brakes .... TEST & SET.

STARTING ENGINE

[ ]  Mixture .... RICH.

[ ]  Carburetor Heat .... COLD.

[ ]  Master Switch .... ON.

[ ]  Prime .... AS REQUIRED.

[ ]  Throttle .... OPEN ¼ INCH.

[ ]  Propeller Area .... CLEAR.

[ ]  Ignition Switch .... START. (release when engine starts)

[ ]  Oil Pressure .... CHECK.

BEFORE TAKEOFF

[ ]  Cabin Doors .... LATCHED.

[ ]  Flight Controls .... FREE & CORRECT.

[ ]  Elevator Trim .... TAKEOFF POSITION.

[ ]  Fuel Shutoff Valve .... ON.

[ ]  Brakes .... SET.

[ ]  Throttle .... 1700 RPM.

[ ]  Magnetos .... CHECK.

[ ]  Carburetor Heat .... CHECK FOR RPM DROP.

[ ]  Engine Instruments and Ammeter .... CHECK.

[ ]  Suction Gauge .... CHECK.

[ ]  Flight Instruments and Radio .... SET.

[ ]  Throttle Friction Lock .... ADJUST.

[ ]  Wing Flaps .... AS REQUIRED.

TAKEOFF — Normal Takeoff

NORMAL TAKEOFF

[ ]  Wing Flaps .... UP (0).

[ ]  Carburetor Heat .... COLD.

[ ]  Throttle .... FULL OPEN.

[ ]  Elevator Control .... LIFT NOSE WHEEL.

[ ]  Climb Speed .... 70-80 MPH.

TAKEOFF — Maximum Performance Takeoff

MAXIMUM PERFORMANCE TAKEOFF

[ ]  Wing Flaps .... UP (0).

[ ]  Carburetor Heat .... COLD.

[ ]  Brakes .... HOLD.

[ ]  Throttle .... FULL OPEN.

[ ]  Brakes .... RELEASE.

[ ]  Elevator Control .... SLIGHTLY TAIL LOW.

[ ]  Climb Speed .... 70 MPH (with obstacles ahead).

ENROUTE CLIMB

[ ]  Airspeed .... 75 – 85 MPH.

[ ]  Throttle .... FULL OPEN.

[ ]  Mixture .... RICH. (Unless engine is rough).

CRUISE

[ ]  Power .... 2000 to 2750. (No more than 75%).

[ ]  Elevator Trim .... ADJUST.

[ ]  Mixture .... LEAN.

BEFORE LANDING

[ ]  Mixture .... RICH

[ ]  Carburetor Heat .... ON (apply before closing throttle).

[ ]  Airspeed .... 70-80 MPH (flaps up).

[ ]  Wing Flaps .... AS DESIRED (below 100 MPH)

[ ]  Airspeed .... 60-70 MPH (flaps down).

BALKED LANDING  [ABNORMAL]

[ ]  Throttle .... FULL OPEN.

[ ]  Carburetor Heat .... COLD.

[ ]  Wing Flaps .... RETRACT TO 20.

[ ]  Airspeed .... 65 MPH

[ ]  Wing Flaps .... RETRACT (slowly).

NORMAL LANDING

[ ]  Touchdown .... MAIN WHEELS FIRST.

[ ]  Landing Roll .... LOWER NOSE WHEEL GENTLY.

[ ]  Braking .... MINIMUM REQUIRED.

AFTER LANDING

[ ]  Wing Flaps .... UP.

[ ]  Carburetor Heat .... COLD.

[ ]  Unnecessary Equipment .... OFF.

SECURING AIRCRAFT

[ ]  Parking Brake .... SET

[ ]  Radios, Electrical Equipment .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF.

[ ]  Control Lock .... INSTALL.

[ ]  Tach/Hobbs Meter .... RECORD.

C150G/C150L Airspeeds (MCAS)

[ ]  Vr .... 55

[ ]  Vx .... 70

[ ]  Vy .... 78

[ ]  Va .... 109

[ ]  Vs1 .... 55

[ ]  Vso .... 48

[ ]  Vfe .... 100

[ ]  Vno .... 120

[ ]  Vne .... 162

[ ]  Glide .... 70

[ ]  XW demo .... 15

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/150 Commuter/aviatickyklub.cz_cessna_c150_engl_checklists.txt

Rights: licensed

Checklist hash: e48ebb995a414ba0415b369017448f2892b75847e1657475119b61d268f8778b

Verify against the aircraft's own approved documentation before use.