Cessna 150 Commuter — c150_abbreviated_millstone_valley_flight_school

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

BEFORE STARTING ENGINE

[ ]  Exterior Preflight .... COMPLETE

[ ]  Tach/Hobbs Meter .... RECORD

[ ]  Seats, Belts, Harnesses .... ADJUST & LOCKED

[ ]  Fuel Shutoff Valve .... ON

[ ]  Radios & Electrical Equipment .... OFF

[ ]  Brakes .... TEST & SET

STARTING ENGINE

[ ]  Mixture .... RICH

[ ]  Carburetor Heat .... COLD

[ ]  Master Switch .... ON

[ ]  Prime .... AS REQUIRED

[ ]  Throttle .... OPEN 1/4 INCH

[ ]  Propeller Area .... CLEAR

[ ]  Ignition Switch .... START

Release when engine starts.

[ ]  Oil Pressure .... CHECK

BEFORE TAKEOFF

[ ]  Cabin Doors .... LATCHED

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Elevator Trim .... TAKEOFF POSITION

[ ]  Fuel Shutoff Valve .... ON

[ ]  Brakes .... SET

[ ]  Throttle .... 1700 RPM

[ ]  Magnetos .... CHECK

[ ]  Carburetor Heat .... CHECK FOR RPM DROP

[ ]  Engine Instruments and Ammeter .... CHECK

[ ]  Suction Gauge .... CHECK

[ ]  Flight Instruments and Radio .... SET

[ ]  Throttle Friction Lock .... ADJUST

[ ]  Wing Flaps .... AS REQUIRED

TAKEOFF

NORMAL TAKEOFF

[ ]  Wing Flaps .... UP (0)

[ ]  Carburetor Heat .... COLD

[ ]  Throttle .... FULL OPEN

[ ]  Elevator Control .... LIFT NOSE WHEEL

[ ]  Climb Speed .... 70-80 MPH

MAXIMUM PERFORMANCE TAKEOFF

[ ]  Wing Flaps .... UP (0)

[ ]  Carburetor Heat .... COLD

[ ]  Brakes .... HOLD

[ ]  Throttle .... FULL OPEN

[ ]  Brakes .... RELEASE

[ ]  Elevator Control .... SLIGHTLY TAIL LOW

[ ]  Climb Speed .... 70 MPH (with obstacles ahead)

ENROUTE CLIMB

[ ]  Airspeed .... 75 - 85 MPH

[ ]  Throttle .... FULL OPEN

[ ]  Mixture .... RICH

Unless engine is rough.

CRUISE

[ ]  Power .... 2000 to 2750

No more than 75%.

[ ]  Elevator Trim .... ADJUST

[ ]  Mixture .... LEAN

BEFORE LANDING

[ ]  Mixture .... RICH

[ ]  Carburetor Heat .... ON

Apply before closing throttle.

[ ]  Airspeed .... 70-80 MPH (flaps up)

[ ]  Wing Flaps .... AS DESIRED

[ ]  Airspeed .... 60-70 MPH (flaps down)

BALKED LANDING

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... COLD

[ ]  Wing Flaps .... RETRACT TO 20

[ ]  Airspeed .... 65 MPH

[ ]  Wing Flaps .... RETRACT

Slowly.

NORMAL LANDING

[ ]  Touchdown .... MAIN WHEELS FIRST

[ ]  Landing Roll .... LOWER NOSE WHEEL GENTLY

[ ]  Braking .... MINIMUM REQUIRED

AFTER LANDING

[ ]  Wing Flaps .... UP

[ ]  Carburetor Heat .... COLD

[ ]  Unnecessary Equipment .... OFF

SECURING AIRCRAFT

[ ]  Parking Brake .... SET

[ ]  Radios, Electrical Equipment .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF

[ ]  Control Lock .... INSTALL

[ ]  Tach/Hobbs Meter .... RECORD

C150G/C150L Airspeeds (MCAS)

[ ]  Vr .... 55

[ ]  Vx .... 70

[ ]  Vy .... 78

[ ]  Va .... 109

[ ]  Vs1 .... 55

[ ]  Vso .... 48

[ ]  Vfe .... 100

[ ]  Vno .... 120

[ ]  Vne .... 162

[ ]  Glide .... 70

[ ]  XW demo .... 15

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/150 Commuter/c150_abbreviated_millstone_valley_flight_school.txt

Rights: licensed

Checklist hash: e3b84b0a05f52782736aeccbaaec6bc37fe52341a7e373b1829f1e53c02f28db

Verify against the aircraft's own approved documentation before use.