Cessna 150A — Hunter_Johnson_Cessna_150A_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Before Starting Engine

[ ]  Preflight .... COMPLETE

[ ]  Seats, Belts .... ADJUST and LOCK

[ ]  Fuel Shutoff Valve .... ON

[ ]  Radios, Elec. Equip. .... OFF

[ ]  Brakes .... TEST and SET

[ ]  Fuses .... CHECK

Starting Engine

[ ]  Mixture .... RICH

[ ]  Carb. Heat .... COLD

[ ]  Prime .... 4 SHOTS

[ ]  Throttle .... ¼ Inch Open

[ ]  Propeller Area .... CLEAR

[ ]  Master Switch .... ON

[ ]  Magnetos .... BOTH

[ ]  Starter Handle .... PULL

[ ]  Throttle .... 1,000RPM

[ ]  Oil Pressure .... CHECK

Before Takeoff

[ ]  Parking Brake .... SET

[ ]  Cabin Doors .... CLOSED and LATCHED

[ ]  Flight Controls .... FREE and CORRECT

[ ]  Flight Instruments .... SET

[ ]  Fuel Shutoff Valve .... ON

[ ]  Mixture .... RICH

[ ]  Elevator Trim .... SET FOR TAKEOFF

[ ]  Throttle .... 1700RPM

[ ]  Magnetos .... CHECK (less than 125PRM)

[ ]  Carb. Heat .... Check (for RPM drop)

[ ]  Engine Instruments .... CHECK

[ ]  Generator Light .... OFF

[ ]  Suction Gauge .... CHECK

[ ]  Radios .... SET

[ ]  Flashing Bcn, Nav. Lights .... ON (as req’d)

[ ]  Throttle Friction Lock .... ADJUST

[ ]  Brakes .... RELEASE

Takeoff

NORMAL TAKEOFF

[ ]  Wing Flaps .... 0-10 DEGREES

[ ]  Carb. Heat .... COLD

[ ]  Throttle .... FULL OPEN

[ ]  Elevator .... LIFT NOSE at 50MPH

[ ]  Climb Speed .... 70MPH

Short-Field Takeoff

[ ]  Wing Flaps .... 10 DEGREES

[ ]  Carb. Heat .... COLD

[ ]  Brakes .... APPLY

[ ]  Throttle .... FULL OPEN

[ ]  Mixture .... RICH

[ ]  Brakes .... RELEASE

[ ]  Elevator .... SLIGHTLY TAIL LOW

[ ]  Climb Speed .... 70MPH (until Clear of obstacles)

[ ]  Wing Flaps .... RETRACT (slowly, in increments)

Enroute Climb

[ ]  Airspeed .... 70-75MPH

[ ]  Throttle .... FULL OPEN

[ ]  Mixture .... RICH (below 3000ft)

Cruise

[ ]  Power .... 1900-2550PRM (no more than 75%)

[ ]  Elevator Trim .... ADJUST

[ ]  Mixture .... LEAN (above 3000ft)

Before Landing

[ ]  Seats, Belts .... ADJUST and LOCK

[ ]  Carb. Heat .... ON

Landing

NORMAL LANDING

[ ]  Airspeed .... 70-75MPH

[ ]  Wing Flaps .... AS DESIRED (under 80MPH)

[ ]  Pitch .... for 70MPH (on final)

Short-Field Landing

[ ]  Airspeed .... 70-75MPH

[ ]  Wing Flaps .... 30 Degrees (under 80MPH)

[ ]  Airspeed .... 65MPH

[ ]  Power .... REDUCE to idle

Balked Landing

[ ]  Throttle .... FULL OPEN

[ ]  Carb. Heat .... COLD

[ ]  Wing Flaps .... RETRACT to 20 Degrees

[ ]  Airspeed .... 70MPH

[ ]  Wing Flaps .... RETRACT (slowly)

After Landing

[ ]  Wing Flaps .... UP

[ ]  Carb. Heat .... COLD

Securing Airplane

[ ]  Radios, Elec. Equip. .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magneto Switch .... OFF (key out)

[ ]  Master Switch .... OFF

[ ]  Control Lock .... INSTALL

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/150A/Hunter_Johnson_Cessna_150A_checklist.txt

Rights: licensed

Checklist hash: b3dae487c4aad0f879d2660aca0f49da53e7b8e255dce6f5b2ef59028669fcd3

Verify against the aircraft's own approved documentation before use.