Cessna 152 — c152_a152_checklist_moorabbin

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

PRE START

[ ]  Loose items stowed

[ ]  Seat adjusted

[ ]  Hatches and harnesses secure

[ ]  Fuel on

[ ]  Elevator Trim through range set to t/o

[ ]  Mixture idle cut off

[ ]  Throttle closed

[ ]  Carburetor Heat off

[ ]  Switches off

[ ]  Park Brake on

[ ]  Instruments checked and set

[ ]  Avionics off

[ ]  Master on

[ ]  Circuit breakers in

[ ]  Controls full and free, correct sense

START CHECKS

[ ]  Mixture rich

[ ]  Throttle 1 cm open

[ ]  Fuel pump on, check press & off

[ ]  Prime as required

[ ]  Check “Clear Prop”

[ ]  Magnetos start

AFTER START

[ ]  Throttle set 1000 rpm

[ ]  Oil pressure green within 30 secs

[ ]  Ammeter positive charge

[ ]  Suction within limits

[ ]  Gyro instruments erection and sync

[ ]  Annunciators test & out

[ ]  Avionics on

[ ]  Beacon on

TAXI CHECKS

[ ]  Brakes checked

[ ]  Instruments turning checks

RUN-UP CHECKS

[ ]  Park brake on

[ ]  Oil temp & pressure green arc

[ ]  Lookout all clear

[ ]  Throttle set 1700 rpm

[ ]  Oil temp & pressure green arc

[ ]  Magnetos left, both, right, both

[ ]  Carb Heat on then off

[ ]  Ammeter positive charge

[ ]  Suction 4.5” – 5.4” Hg

[ ]  Slow idle smooth running

[ ]  Throttle set 1000 rpm

PRE T/O VITAL ACTIONS

[ ]  T trims set, throttle friction adjusted

[ ]  M mixture rich

[ ]  P primer – locked

[ ]  F fuel on, contents checked

[ ]  F flaps set

[ ]  I instruments checked and set

[ ]  auto pilot checked

[ ]  S switches, as required

[ ]  C controls full & free

[ ]  H hatches & harnesses secure

PRE LINE UP CHECKS

[ ]  F fuel pump on, flaps set

[ ]  I instruments, DG to runway heading

[ ]  S switches, lights as required

[ ]  T transponder to ALT

AFTER T/O CHECKS

[ ]  Flaps retracted

[ ]  Power set as required

[ ]  Oil temp & pressure green arc

Pre manoeuvre checks

[ ]  H Height

[ ]  A Airframe

[ ]  S Security

[ ]  E Engine

[ ]  L Location

[ ]  L Lookout

PRE LANDING CHECKS

[ ]  S Speed

[ ]  U Undercarriage down

[ ]  M Mixture rich

[ ]  F Fuel contents

[ ]  H Hatches & Harnesses secure

[ ]  B Brakes, check pressure and off

AFTER LANDING CHECKS

[ ]  Flaps retract

[ ]  Carburretor heat off

[ ]  Transponder STBY

[ ]  Radio to SMC

SHUTDOWN

[ ]  Park brake on

[ ]  Throttle set 1000 rpm

[ ]  Avionics off

[ ]  Beacon off

[ ]  Magnetos check for RPM drop

[ ]  Temp and pressure green arc

[ ]  Mixture idle cut off

[ ]  Throttle closed

[ ]  Magnetos off, key out

[ ]  Master off

[ ]  Park brake release if tieing down

POST FLIGHT

[ ]  Controls secure

[ ]  Hatches secure

[ ]  Tie downs secured

AEROBATIC SPEEDS (A152 only)

[ ]  Flick roll 80 KIAS

[ ]  Barrel roll 115 KIAS

[ ]  Loop 115 KIAS

[ ]  Roll of the top 130 KIAS

[ ]  Steep turn 100 KIAS

[ ]  Chandelle & lazy eight 105 KIAS

[ ]  Spin slow deceleration

“G” LIMITS (A152 only)

[ ]  Flap up +6 -3

[ ]  Flap down +3.5

RADIO FREQUENCIES

[ ]  MB GROUND 119.9 MHz

[ ]  MB TOWER (EAST) 118.1 MHz

[ ]  MB TOWER (WEST) 123.0 MHz

[ ]  MB ATIS 398 or 120.9 MHz

[ ]  EN GROUND 121.9 MHz

[ ]  EN TOWER 125.1 MHz

[ ]  EN ATIS 356 or 119.8 MHz

[ ]  ML RAS 135.7 MHz

[ ]  ML APP 124.7 MHz

[ ]  ML DEP 118.9 or 129.4 MHz

[ ]  ML TWR 120.5 MHz

[ ]  ML SMC 121.7 MHz

[ ]  ML ACD 127.2 MHz

[ ]  ML ATIS 114.1 or 132.7 MHz

ENGINE FIRE DURING START  [EMERGENCY]

[ ]  Starter crank engine

[ ]  Mixture idle cut off

[ ]  Throttle open

[ ]  Fuel selector off

[ ]  Abandon if fire continues

ENGINE FIRE IN FLIGHT  [EMERGENCY]

[ ]  Mixture idle cut-off

[ ]  Throttle closed

[ ]  Fuel Selector off

[ ]  Heater off

[ ]  Defroster off

NOTE: Proceed with POWER OFF LANDING

ENGINE FAILURE IN FLIGHT  [EMERGENCY]

INITIAL ACTIONS:

CHECK FOR FIRE

[ ]  Throttle set 1/3 position

[ ]  Carburetor Heat on

[ ]  Mixture rich

[ ]  Fuel selector check on

ASSESS WIND

SELECT FIELD

MAKE PLAN

TROUBLE CHECKS:

[ ]  Fuel on, contents checked

[ ]  Mixture through range, set rich

[ ]  Oil temps and pressure

[ ]  Switches check magnetos

[ ]  Throttle through range set 1/3 position

MAYDAY CALL

SHUTDOWN CHECKS:

[ ]  Brakes check pressure and off

[ ]  Undercarriage down

[ ]  Switches all off, fuel off

[ ]  Hatches and harnesses secure

LOSS OF OIL PRESSURE / HIGH OIL TEMPERATURE  [EMERGENCY]

[ ]  Land as soon as possible and investigate cause.

[ ]  Prepare for power off landing

C152 / A152 (CESSNA) DATA

AIRSPEEDS

[ ]  Basic Stall Speed (VS1) 40 KIAS

[ ]  Stall with Flap (VS0) 35 KIAS

[ ]  Best Climb Angle (VX) 55 KIAS

[ ]  Best Climb Rate (VY) 65 KIAS

[ ]  Cruise Climb (Normal) 80 KIAS

[ ]  Flap Limit Speed (VFE) 85 KIAS

[ ]  Threshold 60 KIAS

[ ]  Manoeuvre Speed (VA) 93 - 104 KIAS

[ ]  (A152…108 KIAS)

[ ]  Flight Planning TAS 100 KTAS

[ ]  Maximum Cruising (VNO) 111 KIAS

[ ]  Never Exceed (VNE) 149 KIAS

[ ]  Best Glide 60 KIAS

[ ]  TOSS 54 KIAS

[ ]  Base 75 KIAS

[ ]  Final 65 KIAS

[ ]  Maximum Crosswind 15 KIAS

WEIGHT LIMITATIONS

[ ]  Max Takeoff Weight 757 Kg Lb

[ ]  Empty Weight: Refer flight manual

[ ]  Max Baggage Weight

FUEL

[ ]  Type: 100 Octane Avgas (green) or 100 LL (blue)

[ ]  Quantity: Total 98 lt, Useable 93 lt (fuel flow at 75%)

[ ]  (Long range tanks: 148 / 143 Lt)

[ ]  Typical Consumption: 23 lt per hour

[ ]  Endurance with 45 min reserve intact: 4.2 hrs

POWER SETTINGS

[ ]  Max RPM 2700 RPM

[ ]  Static RPM

ENGINE

[ ]  Avco Lycoming O-335 L2C or N2C

[ ]  4 cylinder, horizontally opposed, direct drive, air cooled, normally aspirated

[ ]  Output: 110 Hp @ 2550 rpm

OIL

[ ]  Type: Aviation grade D100

[ ]  Quantity: Max 8 US quarts, Min 6 US quarts (Company reqts)

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/152/c152_a152_checklist_moorabbin.txt

Rights: licensed

Checklist hash: 6e63f619fab64419cdc6eac2aca5a1110e556352c61a330705a5d083385a7285

Verify against the aircraft's own approved documentation before use.