Cessna 152 — c152_abbreviated_checklist_millstone_valley

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

BEFORE STARTING ENGINE

[ ]  Exterior Preflight .... COMPLETE.

[ ]  Tach/Hobbs Meter .... RECORD

[ ]  Seats, Belts, Harnesses .... ADJUST & LOCKED.

[ ]  Fuel Shutoff Valve .... ON.

[ ]  Radios & Electrical Equipment .... OFF.

[ ]  Brakes .... TEST & SET.

STARTING ENGINE

[ ]  Mixture .... RICH.

[ ]  Carburetor Heat .... COLD.

[ ]  Master Switch .... ON.

[ ]  Prime .... AS REQUIRED.

[ ]  Throttle .... OPEN ¼ INCH.

[ ]  Propeller Area .... CLEAR.

[ ]  Ignition Switch .... START. (release when engine starts)

[ ]  Oil Pressure .... CHECK.

BEFORE TAKEOFF

[ ]  Cabin Doors .... LATCHED.

[ ]  Flight Controls .... FREE & CORRECT.

[ ]  Elevator Trim .... TAKEOFF POSITION.

[ ]  Fuel Shutoff Valve .... ON.

[ ]  Brakes .... SET.

[ ]  Throttle .... 1700 RPM.

[ ]  Magnetos .... CHECK. 125 MAX DROP/ 50 DIFF

[ ]  Carburetor Heat .... CHECK FOR RPM DROP.

[ ]  Engine Instruments and Ammeter .... CHECK.

[ ]  Suction Gauge .... CHECK.

[ ]  Flight Instruments and Radio .... SET.

[ ]  Throttle Friction Lock .... ADJUST.

[ ]  Wing Flaps .... AS REQUIRED.

TAKEOFF

NORMAL TAKEOFF

[ ]  Wing Flaps .... UP (0).

[ ]  Carburetor Heat .... COLD.

[ ]  Throttle .... FULL OPEN.

[ ]  Elevator Control .... LIFT NOSE WHEEL at 50.

[ ]  Climb Speed .... 70-80 KIAS.

MAXIMUM PERFORMANCE TAKEOFF

[ ]  Wing Flaps .... UP (0).

[ ]  Carburetor Heat .... COLD.

[ ]  Brakes .... HOLD.

[ ]  Throttle .... FULL OPEN.

[ ]  Brakes .... RELEASE.

[ ]  Elevator Control .... SLIGHTLY TAIL LOW.

[ ]  Climb Speed .... 70 KIAS (with obstacles ahead).

ENROUTE CLIMB.

[ ]  Airspeed .... 75 – 85 KIAS.

[ ]  Throttle .... FULL OPEN.

[ ]  Mixture .... RICH. (Unless engine is rough).

CRUISE

[ ]  Power .... 2000 to 2750. (No more than 75%).

[ ]  Elevator Trim .... ADJUST.

[ ]  Mixture .... LEAN.

BEFORE LANDING

[ ]  Mixture .... RICH

[ ]  Carburetor Heat .... ON (apply before closing throttle).

[ ]  Airspeed .... 70-80 KIAS (flaps up).

[ ]  Wing Flaps .... AS DESIRED (below 85KIAS)

[ ]  Airspeed .... 60-70 KIAS (flaps down).

BALKED LANDING

[ ]  Throttle .... FULL OPEN.

[ ]  Carburetor Heat .... COLD.

[ ]  Wing Flaps .... RETRACT TO 20.

[ ]  Airspeed .... 65 KIAS

[ ]  Wing Flaps .... RETRACT  (slowly).

NORMAL LANDING

[ ]  Touchdown .... MAIN WHEELS FIRST.

[ ]  Landing Roll .... LOWER NOSE WHEEL GENTLY.

[ ]  Braking .... MINIMUM REQUIRED.

AFTER LANDING

[ ]  Wing Flaps .... UP.

[ ]  Carburetor Heat .... COLD.

[ ]  Unnecessary Equipment .... OFF.

SECURING AIRCRAFT

[ ]  Parking Brake .... SET (Normally Not Used.)

[ ]  Radios, Electrical Equipment .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF.

[ ]  Control Lock .... INSTALL.

[ ]  Tach/Hobbs Meter .... RECORD.

C152 Airspeeds (KIAS)

[ ]  Vr .... 50

[ ]  Vx .... 55

[ ]  Vy .... 67

[ ]  Va .... 104

[ ]  Vs1 .... 40

[ ]  Vso .... 35

[ ]  Vfe .... 85

[ ]  Vno .... 111

[ ]  Vne .... 149

[ ]  Glide .... 65

[ ]  XW demo .... 12

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/152/c152_abbreviated_checklist_millstone_valley.txt

Rights: licensed

Checklist hash: 5b72c4e17c73dfbe2cb8b49b01f12f415d51bbfad4e9cb2b034ec03b0af96c61

Verify against the aircraft's own approved documentation before use.