Cessna 152 — c152_expanded_checklist_millstone_valley

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Airspeed Limitations" item count differs between passes (13 vs 12); verify nothing was dropped.

content_gap: Section "Balked Landing (Go Around)" item count differs between passes (5 vs 4); verify nothing was dropped.

content_gap: Section "Cabin Fire" item count differs between passes (5 vs 4); verify nothing was dropped.

content_gap: Section "Cold Weather Starting With or Without Pre-Heat" item count differs between passes (30 vs 25); verify nothing was dropped.

content_gap: Section "Ditching" item count differs between passes (17 vs 15); verify nothing was dropped.

content_gap: Section "Engine Failure In Flight" item count differs between passes (17 vs 16); verify nothing was dropped.

content_gap: Section "Engine Fire During Start Up" item count differs between passes (13 vs 12); verify nothing was dropped.

content_gap: Section "Engine Fire In Flight" item count differs between passes (7 vs 6); verify nothing was dropped.

content_gap: Section "Maneuvering Limits" item count differs between passes (6 vs 5); verify nothing was dropped.

content_gap: Section "Maximum Performance Takeoff (Short Field)" item count differs between passes (14 vs 12); verify nothing was dropped.

content_gap: Section "Normal Approach and Landing Procedures" item count differs between passes (20 vs 15); verify nothing was dropped.

content_gap: Section "Normal Takeoff and Climb Procedures" item count differs between passes (10 vs 9); verify nothing was dropped.

content_gap: Section "Optimum/Recommended Speeds" item count differs between passes (11 vs 10); verify nothing was dropped.

content_gap: Section "Pre-Flight Inspection Checklist" item count differs between passes (66 vs 72); verify nothing was dropped.

content_gap: Section "Precautionary Landing With Engine Power" item count differs between passes (15 vs 13); verify nothing was dropped.

content_gap: Section "Short Field Landing" item count differs between passes (22 vs 17); verify nothing was dropped.

content_gap: Section "Soft Field Landing" item count differs between passes (22 vs 17); verify nothing was dropped.

content_gap: Section "Soft Field Takeoff" item count differs between passes (14 vs 11); verify nothing was dropped.

content_gap: Section "Wing Fire" item count differs between passes (6 vs 5); verify nothing was dropped.

Airspeed Limitations

AIRSPEED LIMITATIONS

[ ]  VNE — Never Exceed Speed .... 149 KIAS / 168 MPH

NOTE: Do not exceed this speed in any operation.

[ ]  VNO — Max structural cruising speed .... 111 KIAS / 127 MPH

NOTE: Do not exceed this speed except in smooth air and then only with caution.

[ ]  VA — Maneuvering speed, 1670 Pounds .... 104 KIAS / 119 MPH

[ ]  VA — Maneuvering speed, 1500 Pounds .... 98 KIAS / 112 MPH

[ ]  VA — Maneuvering speed, 1350 Pounds .... 93 KIAS / 107 MPH

[ ]  VFE — Maximum flap extended speed .... 87 KIAS / 100 MPH

[ ]  Maximum window open speed .... 139 KIAS / 159 MPH

[ ]  VS — Stall speed (No Flaps) .... 40 KIAS / 46 MPH

[ ]  VS0 — Stall speed in landing configuration .... 35 KIAS / 40 MPH

[ ]  Demonstrated crosswind capability .... 12 KIAS / 14 MPH

Optimum/Recommended Speeds

OPTIMUM/RECOMMENDED SPEEDS

[ ]  VX — Best angle of climb .... 55 KIAS / 63 MPH

[ ]  VY — Best rate of climb .... 67 KIAS / 77 MPH

[ ]  VR — Normal rotation .... 50 KIAS / 58 MPH

[ ]  Normal climb .... 70-80 KIAS / 80-92 MPH

[ ]  Normal landing (no flaps) .... 60-70 KIAS / 69-80 MPH

[ ]  Normal landing (full flaps) .... 55-65 KIAS / 63-75 MPH

[ ]  Powered landing (no flaps) .... 65 KIAS / 75 MPH

[ ]  Powered landing (full flaps) .... 60 KIAS / 69 MPH

[ ]  Max performance approach .... 54 KIAS / 62 MPH

[ ]  Optimum glide .... 60 KIAS / 69 MPH

Maneuvering Limits

MANEUVERING LIMITS

[ ]  Chandelles .... Max indicated airspeed 95 KIAS / 109 MPH

[ ]  Lazy Eights .... Max indicated airspeed 95 KIAS / 109 MPH

[ ]  Steep Turns .... Max indicated airspeed 95 KIAS / 109 MPH

[ ]  Spins .... Use slow deceleration

[ ]  Stalls (except whip stalls) .... Use slow deceleration

Stall Speed Table (Max Gross/CG Forward)

STALL SPEED TABLE (MAX GROSS/CG FORWARD)

[ ]  Angle of Bank 00, 00 Flaps .... 40 KIAS / 46 MPH

[ ]  Angle of Bank 00, 30 Flaps .... 35 KIAS / 40 MPH

[ ]  Angle of Bank 30, 00 Flaps .... 43 KIAS / 49 MPH

[ ]  Angle of Bank 30, 30 Flaps .... 38 KIAS / 44 MPH

[ ]  Angle of Bank 45, 00 Flaps .... 48 KIAS / 55 MPH

[ ]  Angle of Bank 45, 30 Flaps .... 42 KIAS / 48 MPH

[ ]  Angle of Bank 60, 00 Flaps .... 57 KIAS / 66 MPH

[ ]  Angle of Bank 60, 30 Flaps .... 49 KIAS / 56 MPH

Engine Failure During Takeoff Run  [EMERGENCY]

[ ]  Throttle .... Idle

[ ]  Brakes .... Apply

[ ]  Flaps .... Retract

[ ]  Mixture .... Idle Cutoff

[ ]  Ignition Switch .... Off

[ ]  Master Switch .... Off

Engine Failure Immediately After Takeoff  [EMERGENCY]

1. IF ENOUGH RUNWAY REMAINING TO LAND:

[ ]  Throttle .... Idle

[ ]  Land airplane .... LAND

[ ]  Brakes .... Apply

[ ]  Flaps .... Up

[ ]  Mixture .... Idle cutoff

[ ]  Ignition Switch .... Off

[ ]  Master Switch .... Off

2. NOT ENOUGH RUNWAY TO LAND

[ ]  Airspeed .... 60 KIAS (69 MPH)

[ ]  Fly runway heading to emergency landing site .... ESTABLISH

[ ]  Mixture .... Idle cutoff

[ ]  Fuel shutoff .... Off

[ ]  Ignition switch .... Off

[ ]  Flaps .... As required

[ ]  Master switch .... Off

[ ]  Doors .... Ajar

Engine Failure In Flight  [EMERGENCY]

1. GAIN ALL THE ALTITUDE YOU CAN!

[ ]  Pull back (gently) to use the aircraft's momentum to gain altitude until airspeed falls off to the optimum glide speed .... 60 KIAS (69 MPH)

2. AIRSPEED - OPTIMUM GLIDE SPEED 60 KIAS (69 MPH)

[ ]  Trim the airplane for optimum glide speed .... TRIM

3. FIND A SUITABLE PLACE TO LAND AND FLY TO IT

NOTE: If altitude and distance to selected site permit, try to set up a normal landing pattern. If that's not possible, take what you can get. Regardless of whether or not a full pattern can be set up, make sure the approach results in a landing parallel to any furrows in the selected field.

4. IF TIME PERMITS, TRY TO CORRECT THE PROBLEM

[ ]  Fuel shut-off .... On

[ ]  Mixture .... Rich (in)

[ ]  Throttle .... 1/4 Inch

[ ]  Carburetor Heat .... On (out)

[ ]  Primer .... In and Locked

[ ]  Master Switch .... On (Both sides)

[ ]  Ignition switch .... Both magnetos — Start if propeller is stopped

5. IF STILL HAVE TIME COMMUNICATE

[ ]  Transponder .... 7700

[ ]  Comm Radio .... 121.5

Emergency Landing Without Engine Power  [EMERGENCY]

1. FLY THE AIRPLANE

[ ]  Airspeed .... 65 KIAS / 75 MPH (flaps up); 60 KIAS / 69 MPH (flaps down)

2. PREPARE AIRCRAFT FOR LANDING

[ ]  Mixture .... Idle cutoff

[ ]  Fuel Shutoff .... Off

[ ]  Ignition Switch .... Off

[ ]  Flaps .... As required (30° recommended)

[ ]  Master Switch .... Off

[ ]  Doors .... Unlatch prior to touchdown

3. LANDING

[ ]  Touchdown .... Slightly tail low

[ ]  Brakes .... Apply heavily

Precautionary Landing With Engine Power  [ABNORMAL]

1. FLY THE AIRPLANE

[ ]  Airspeed .... 60 KIAS / 69 MPH

[ ]  Flaps .... 20°

[ ]  Selected Field .... Inspect

NOTE: Fly over field noting terrain and obstructions then retract flaps upon reaching a safe altitude and airspeed.

2. PREPARE AIRPLANE FOR LANDING

[ ]  Radios and Electrical .... Off

[ ]  Flaps .... 30° (On final approach)

[ ]  Airspeed .... 55 KIAS / 64 MPH on final

[ ]  Master Switch .... Off

[ ]  Doors .... Unlatch prior to touchdown

3. LANDING

[ ]  Touchdown .... Tail slightly low

[ ]  Ignition Switch .... Off

[ ]  Brakes .... Apply heavily

Ditching  [EMERGENCY]

1. PREPARE FOR DITCHING

[ ]  Radio .... Transmit MAYDAY on 121.5

NOTE: Give location, situation and intentions. Note, if you were already communicating with ATC, report situation to controller, as opposed to using 121.5.

[ ]  Transponder .... 7700

[ ]  Heavy Baggage .... Secure or jettison

2. FLY THE AIRPLANE

[ ]  Approach — High wind / Heavy seas .... Into the wind

[ ]  Approach — Light winds / Heavy swells .... Parallel to the swells

[ ]  Flaps .... 30°

[ ]  Power .... 300 ft./min. descent, 55 KIAS (64 MPH)

[ ]  Cabin Doors .... Unlatch prior to touchdown

3. LANDING

[ ]  Touchdown .... Level attitude at 300 ft./min. descent

[ ]  Face .... Cushion with folded coat

[ ]  Evacuate .... Through doors

NOTE: If necessary, open windows to allow cabin to flood to equalize pressure so doors can be opened.

[ ]  Life Vests and Raft .... Inflate

Engine Fire During Start Up  [EMERGENCY]

[ ]  Cranking .... Continue

NOTE: Getting the engine to start will suck flames and accumulated fuel into the engine.

IF ENGINE STARTS

[ ]  Power .... 1,700 RPM for a few minutes

[ ]  Engine .... Shutdown and inspect for damage

IF ENGINE FAILS TO START

[ ]  Continue cranking .... 2 to 3 minutes

[ ]  Obtain fire extinguisher .... OBTAIN

[ ]  Master Switch .... Off

[ ]  Ignition Switch .... Off

[ ]  Fuel Shutoff .... Off

[ ]  Extinguish fire .... Extinguisher, seat cushion, blanket, etc. or dirt

[ ]  Inspect for damage and have repairs made before attempting another flight .... INSPECT

Engine Fire In Flight  [EMERGENCY]

[ ]  Mixture .... Idle cutoff

[ ]  Fuel Shutoff .... Off

[ ]  Master Switch .... Off

[ ]  Cabin Heat and Air .... Off (except overhead vents)

[ ]  Airspeed .... 85 KIAS (98 MPH)

NOTE: If that does not extinguish the fire increase airspeed to that which produces an incombustible mixture. Be aware of critical speeds; VNO (108 KIAS/124 MPH) and VNE (145 KIAS/166 MPH).

[ ]  Landing .... Forced Landing Without Power

Electrical Fire In Flight  [EMERGENCY]

1. EXTINGUISH FIRE

[ ]  Master Switch .... Off

[ ]  All other Switches .... Off

[ ]  Ignition .... On

[ ]  Vents, Cabin Heat/Air .... Closed

[ ]  Fire Extinguisher .... Activate

2. IF FIRE APPEARS TO BE OUT AND ELECTRICAL EQUIPMENT IS NEEDED

[ ]  Master Switch .... On

[ ]  Circuit Breakers .... Check for faulty circuit - do not reset

[ ]  Radio/Electrical .... On one at a time, with delay between until short circuit is localized

[ ]  Vents, Cabin Heat/Air .... Open once it is ascertained that the fire is completely extinguished

Cabin Fire  [EMERGENCY]

[ ]  Master Switch .... Off

[ ]  Vents, Cabin Heat/Air .... Closed

[ ]  Fire Extinguisher .... Activate

NOTE: After using fire extinguisher within a closed cabin ventilate the cabin.

[ ]  Landing .... As soon as possible

Wing Fire  [EMERGENCY]

[ ]  Navigation lights .... Off

[ ]  Strobe Lights .... Off

[ ]  Pitot Heat .... Off

[ ]  Attitude .... Perform side-slip to keep the flames away from the fuel tank and cabin

[ ]  Land .... ASAP

NOTE: Do not use flaps.

Pre-Flight Inspection Checklist

1. WING TOPS/FUEL TANKS

[ ]  Fuel Level .... Both Wings - Visual check

NOTE: If needed get gas (AVGAS 100) before proceeding with other fuel tank related items.

[ ]  Filler caps .... Both Wings - Secure

NOTE: Make sure vent on right wing cap is not blocked.

[ ]  Wing Tops .... Inspect for loose screws, rivets and damage

2. COCKPIT

[ ]  Control wheel lock .... Remove

[ ]  Ignition switch .... Off

[ ]  Master switch .... On (both sides)

[ ]  Fuel gauges .... Check quantity

[ ]  Flaps .... 10°

[ ]  Pitot Heat .... On - observe Ammeter drop - then off

[ ]  Strobe/Beacon .... On - visually check - off

[ ]  Master Switch .... Off

[ ]  Fuel shut-off valve .... On

[ ]  Paperwork .... CHECK

NOTE: Airworthiness certificate; Registration; Radio station license; Operating limitations (POH); Weight/loading data.

3. COCKPIT - NIGHT FLIGHTS

[ ]  Nav Lights & Strobes .... On

NOTE: Walk around plane and visually check to see that all are operating.

[ ]  Landing Light .... On

NOTE: Visually check from outside if not dark enough to see that it's on from inside the cockpit.

[ ]  Instrument Lights .... On

4. FUSELAGE - LEFT SIDE

[ ]  Radio antennas .... Check security

[ ]  Miscellaneous .... Check for loose screws/rivets, etc.

5. EMPENNAGE

[ ]  Rudder gust lock .... Remove

[ ]  Tail tie-down .... Disconnect

[ ]  Control surfaces .... Check freedom of movement, actuators, security, loose rivets, damage

6. FUSELAGE RIGHT SIDE

[ ]  Miscellaneous .... Check for loose screws/rivets, etc.

[ ]  Fuel strainer drain (belly) .... Check sample for water/dirt

7. RIGHT WING

[ ]  Flaps .... Check actuator and rollers

[ ]  Aileron .... Check freedom, hinge pins and counterweights

[ ]  Wing tip .... Check for cracks; Check navigation light

[ ]  Leading edge .... Check for dents, cracks, etc.

[ ]  Wing tie-down .... Disconnect

[ ]  Main wheel tire .... Check for proper inflation/wear; Check cotter pin in wheel nut

[ ]  Main wheel brake .... Check for fluid leaks; Check brake pads

[ ]  Fuel drain .... Check sample for water/dirt and fuel type (100LL - Blue)

8. NOSE

[ ]  Engine Oil .... Check level

NOTE: Minimum 4 qts.; 5 qts. for short flights; 6 qts. extended flights.

[ ]  Fuel bowl drain .... Check sample for water/dirt and fuel type (100LL - Blue)

[ ]  Prop/Spinner .... Check for nicks and security

[ ]  Air filter .... Check for restrictions & excessive dirt

[ ]  Landing Light .... Check condition and cleanliness

[ ]  Cowling .... Look for birds or nests inside

[ ]  Nose wheel .... Check for proper inflation/wear; Check for cotter pins in wheel nuts; Check for leaking fluids

[ ]  Nose tie-down .... Disconnect

[ ]  Static source opening .... Check not blocked

9. LEFT WING

[ ]  Wing tie-down .... Disconnect

[ ]  Leading edge .... Check for dents, cracks, etc.

[ ]  Pitot tube .... Remove cover, check for blockage

[ ]  Stall warning opening .... Check for blockage

[ ]  Fuel Tank Vent .... Check for blockage

[ ]  Wing tip .... Check for cracks; Check navigation light

[ ]  Aileron .... Check freedom, hinge pins and counterweights

[ ]  Flaps .... Check actuator and rollers

[ ]  Fuel drain .... Check sample for water/dirt and fuel type (100LL - Blue)

[ ]  Main wheel tire .... Check for proper inflation/wear; Check cotter pin in wheel nut

[ ]  Main wheel brake .... Check for fluid leaks; Check brake pads

Normal Engine Starting Checklist

1. BEFORE STARTING

[ ]  Preflight Inspection .... Completed

[ ]  Seat position .... Adjust & ensure locked

[ ]  Seat belts/harness .... Adjust and lock

NOTE: Brief passengers on use of belts/harnesses and requirements for wearing them.

[ ]  Fuel shut-off .... On

[ ]  Radios/electrical .... Off

[ ]  Brakes .... Test and set

[ ]  Circuit Breakers .... Check all in

2. STARTING ENGINE

[ ]  Mixture .... Rich (in)

[ ]  Carburetor heat .... Cold (in)

[ ]  Primer .... Prime if required; Make sure locked in

[ ]  Throttle .... 1/4 Inch

[ ]  Key .... In ignition

[ ]  Master Switch .... On (both sides)

[ ]  Propeller Area .... Call "Clear" & check prop area and behind plane

[ ]  Ignition .... Start - release on start

[ ]  Throttle .... 1,000 RPM

[ ]  Oil Pressure .... Check in green

3. BEFORE TAXIING

[ ]  Radios .... On and set to appropriate frequency. Call for radio check

[ ]  Transponder .... Standby

[ ]  Beacon/Strobe .... On

[ ]  Nav. Lights/Strobes .... On if required

[ ]  Flaps .... Full up (normal takeoff)

4. TAXIING

[ ]  Clearance .... Check for things in way of wings; Check for people ahead of and behind plane

[ ]  Flight Controls .... Set for existing wind conditions

[ ]  Brakes .... Come to full stop immediately after starting taxi roll

5. IFR INSTRUMENT CHECKS

[ ]  Turn Coordinator .... Should indicate turn in proper direction while taxiing

[ ]  Attitude Indicator .... Very little change due to turns; Slight pitch indications due to acceleration or deceleration

[ ]  Heading Indicator .... Should track headings

[ ]  Altimeter .... When set to current altimeter setting should indicate within 75 ft. of airport elevation

[ ]  VSI .... Should indicate zero. If not, note indication and use for level indication in flight

[ ]  VORs .... Check at local ground check point or against each other based on some receivable signal

Cold Weather Starting With or Without Pre-Heat

1. BEFORE STARTING

[ ]  Preflight Inspection .... Completed

[ ]  Seat position .... Adjust & ensure locked

[ ]  Seat belts/harness .... Adjust and lock

NOTE: Brief passengers on use of belts/harnesses and requirements for wearing them.

[ ]  Fuel shut-off .... On

[ ]  Radios/electrical .... Off

[ ]  Brakes .... Test and set

[ ]  Circuit Breakers .... Check all in

2. STARTING ENGINE

[ ]  Ignition .... Off (take key out and hang it up)

[ ]  Master Switch .... Off (both sides)

[ ]  Prime .... Prime engine two to four strokes (up to seven without pre-heat) while propeller is turned by hand

NOTE: With ignition switch off and throttle closed, prime the engine two to four strokes (up to seven without pre-heat) as the propeller is being turned by hand. Use heavy primer strokes for best atomization of fuel.

NOTE: If doing this by yourself, tie the plane down securely and set parking brake, in case engine starts.

NOTE: Treat propeller as if the ignition is on and engine could start. Leave primer charged and ready for a stroke.

[ ]  Propeller Area .... Clear

[ ]  Master Switch .... On

[ ]  Mixture .... Full Rich

[ ]  Throttle .... Open 1/4 inch

[ ]  Ignition Switch .... Start

[ ]  Throttle .... Pump to full open twice then return to 1/4 inch open position

[ ]  Primer .... Continue to prime until engine is running smoothly. Alternately pump throttle rapidly over first 1/4 of travel

[ ]  Oil Pressure .... In the green

NOTE: This might take a little time since the engine is cold. If pressure doesn't come up in about 30 seconds, shut the engine down.

[ ]  Carburetor Heat .... Full on after engine started. Leave on until running smoothly

[ ]  Primer .... In and locked

CAUTION: If engine does not start during the first few attempts, or if engine firing diminishes in strength, it is probable that the spark plugs have frosted over. Pre-heat must be used before another start is attempted.

CAUTION: Pumping the throttle may cause raw fuel to accumulate in the intake air duct, creating a fire hazard in the event of a back-fire. If this occurs, maintain a cranking action to suck flames into the engine. An outside attendant with a fire extinguisher is advised for cold starts without pre-heat.

3. PERFORM STEPS 3 AND 4 ON NORMAL START CHECKLIST

Before Takeoff Checklist

1. FINAL COCKPIT CHECK

[ ]  Cabin doors .... Closed and latched

[ ]  Windows .... Closed

[ ]  Flight Controls .... Free and correct

[ ]  Elevator trim .... Takeoff position

[ ]  Flight Instruments .... Check and set

NOTE: Set attitude indicator to level flight position; Set altimeter to runway altitude or locally reported altimeter setting; Set heading indicator to magnetic compass.

[ ]  Comm Radio/VOR .... Set to appropriate freqs

[ ]  Beacon/Strobe .... On

[ ]  Nav Lights/Strobes .... On if required

2. ENGINE RUN-UP

[ ]  Fuel Shutoff .... On

[ ]  Mixture .... Rich (in)

[ ]  Parking brake .... Set or hold foot brakes

[ ]  Throttle .... 1,700 RPM

[ ]  Magnetos .... Check

NOTE: RPM drop should not exceed 125 RPM on either magneto. RPM difference between magnetos should not exceed 50 RPM.

[ ]  Carburetor Heat .... On — Check for RPM drop then back to off

[ ]  Engine instruments .... Check

NOTE: Oil pressure/Temperature. Ammeter - Create electrical load with landing light. Make sure no more than needle width deflection.

[ ]  Vacuum .... Check in green

[ ]  Throttle .... Idle

[ ]  Carburetor heat .... On – Make sure engine keeps running

[ ]  Throttle .... 1,000 RPM

[ ]  Throttle friction lock .... Adjust

[ ]  Alternator switch .... Off – Check low voltage light on – Switch back to on

[ ]  Flaps .... Appropriate takeoff position

[ ]  Throttle friction lock .... Adjust

[ ]  Transponder .... Set to mode C/Altitude

Normal Takeoff and Climb Procedures

[ ]  Flaps .... Full up

[ ]  Carburetor Heat .... Cold (in)

[ ]  Elevator Trim .... Takeoff position

[ ]  Heading indicator .... Calibrate against compass

[ ]  Throttle .... Full open (in)

[ ]  Engine Instruments .... Check while starting roll

NOTE: RPM - 2600 (Top of green arc); Oil Pressure - In the green; Oil Temperature - In the green; Suction - In the green.

[ ]  Airspeed .... Building

[ ]  Elevator .... Lift nose wheel at 50 KIAS (58 MPH)

[ ]  Climb Speed .... 65 - 75 KIAS (75 - 86 MPH)

Maximum Performance Takeoff (Short Field)

[ ]  Taxi .... Maximum runway usage

NOTE: Takeoff procedure should be started using ALL available runway. Taxi to end of runway and align with centerline.

[ ]  Brakes .... Set and hold

[ ]  Flaps .... 10°

[ ]  Carburetor Heat .... Cold (in)

[ ]  Elevator Trim .... Takeoff position

[ ]  Heading indicator .... Calibrate against compass

[ ]  Throttle .... Full open (in)

[ ]  Engine Instruments .... Check before starting roll

NOTE: RPM - 2600 (Top of green arc); Oil Pressure - In the green; Oil Temperature - In the green; Suction - In the green.

[ ]  Brakes .... Release

[ ]  Airspeed .... Building

[ ]  Elevator .... Slightly tail low

[ ]  Climb Speed .... 54 KIAS (62 MPH) with obstacles ahead

Soft Field Takeoff

[ ]  Taxi .... Keep rolling to avoid bogging down

[ ]  Flaps .... 10°

NOTE: If 10° flaps are used, with obstacles ahead, leave them extended until the obstacle is cleared and at a safe altitude. The exception is in high density altitude takeoff where the climb would be marginal with the flaps at 10°.

[ ]  Carburetor Heat .... Cold (in)

[ ]  Elevator Trim .... Takeoff position

[ ]  Heading indicator .... Calibrate against compass or runway heading

[ ]  Throttle .... Full open (in)

[ ]  Engine Instruments .... Check as starting roll

NOTE: RPM - 2600 (Top of green arc); Oil Pressure - In the green; Oil Temperature - In the green; Suction - In the green.

[ ]  Airspeed .... Building

[ ]  Elevator .... Slightly tail low

NOTE: Allow the airplane to lift off as soon as possible (before reaching safe climb speed). Level off at a few feet above the ground and fly in ground effect until reaching normal climb speed.

[ ]  Climb Speed .... 54 KIAS (62 MPH) with obstacles ahead

[ ]  Flaps .... Retract at safe altitude with positive rate of climb

After Takeoff Checklist

1. CLIMBOUT

[ ]  Airspeed .... 65 - 75 Knots (75 - 86 MPH)

[ ]  Altitude .... Above 300 AGL

[ ]  Flaps .... Up (in 10° increments if more than that in use)

2. AT CRUISE ALTITUDE

[ ]  Attitude .... Level

[ ]  Airspeed .... Let build to desired cruise speed

[ ]  Throttle .... Reduce to desired cruise setting

[ ]  Heading Indicator .... Calibrate against compass

3. ABOVE 3,000 MSL

[ ]  Mixture .... Lean for maximum RPM

Enroute Climb

[ ]  Normal Airspeed .... 70 - 80 KIAS (80 - 92 MPH)

[ ]  Max Performance .... See POH Climb Table in Section 5

[ ]  Throttle .... Full Open (in)

[ ]  Carburetor Heat .... Cold (in)

[ ]  Mixture .... Rich (in) below 3,000 ft.; May be leaned above 3,000

Normal Approach and Landing Procedures

1. PRE-LANDING (DOWNWIND) CHECK

[ ]  Seat belts/Harnesses .... Adjust and lock

[ ]  Mixture .... Rich (In)

[ ]  Carburetor heat .... On (Out)

[ ]  Fuel Shut-off .... On

2. APPROACH AND LANDING

[ ]  Power .... Reduce to 1,500 to 1,700 RPM abeam approach end of runway

[ ]  Airspeed .... Let bleed off to less than 85 KIAS (95 MPH)

[ ]  Flaps .... Use as desired

NOTE: Under light (less than 10 Knots) wind conditions 10° descending on the end of the downwind leg, 20° on base, and full flaps over the threshold. In heavier winds 20° or less is good flap setting for landing. Use minimum flap setting possible for cross wind landing.

AIRSPEEDS

[ ]  Downwind through base .... 65 - 75 KIAS (75 - 86 MPH)

[ ]  Final approach .... 60 - 70 KIAS (70 - 80 MPH)

NOTE: In gusty winds add 1/2 difference between gust and average wind speed to approach speed.

TOUCHDOWN

[ ]  Touchdown .... Just above stalling speed - main wheels first

LANDING ROLL

[ ]  Lower nose wheel .... Gently

BRAKING

[ ]  Braking .... Minimum required

Short Field Landing

1. PRE-LANDING (DOWNWIND) CHECK

[ ]  Seat belts/Harnesses .... Adjust and lock

[ ]  Mixture .... Rich (In)

[ ]  Carburetor heat .... On (Out)

[ ]  Fuel Shut-off .... On

2. APPROACH AND LANDING

[ ]  Power .... Reduce to 1,500 to 1,700 RPM abeam approach end of runway

[ ]  Airspeed .... Let bleed off to less than 85 KIAS (95 MPH)

[ ]  Flaps .... Use as desired

NOTE: Under light (less than 10 Knots) wind conditions 10° descending on the end of the downwind leg, 20° on base, and full flaps on final.

AIRSPEEDS

[ ]  Downwind through base .... 65 - 75 KIAS (75 - 86 MPH)

[ ]  Final approach .... 54 KIAS (62 MPH)

NOTE: In gusty winds add 1/2 difference between gust and average wind speed to approach speed.

TOUCHDOWN

[ ]  Touchdown .... Just above stalling speed - power off - main wheels first

NOTE: Roundout must be done much faster than usual due to low airspeed.

LANDING ROLL

[ ]  Lower nose wheel .... Quickly

BRAKING

[ ]  Braking .... Maximum possible without sliding tires

[ ]  Flight Controls .... Flaps up; Hold full up elevator

Soft Field Landing

1. PRE-LANDING (DOWNWIND) CHECK

[ ]  Seat belts/Harnesses .... Adjust and lock

[ ]  Mixture .... Rich (In)

[ ]  Carburetor heat .... On (Out)

[ ]  Fuel Shut-off .... On

2. APPROACH AND LANDING

[ ]  Power .... Reduce to 1,500 to 1,700 RPM abeam approach end of runway

[ ]  Airspeed .... Let bleed off to less than 85 KIAS (95 MPH)

[ ]  Flaps .... Use as desired

NOTE: Under light (less than 10 Knots) wind conditions 10° descending on the end of the downwind leg, 20° on base, and full flaps over the threshold. In heavier winds 20° or less is good flap setting for landing. Use minimum flap setting possible for cross wind landing.

AIRSPEEDS

[ ]  Downwind through base .... 65 - 75 KIAS (75 - 86 MPH)

[ ]  Final approach .... 60 - 65 KIAS (70 - 75 MPH)

NOTE: In gusty winds add 1/2 difference between gust and average wind speed to approach speed.

TOUCHDOWN

[ ]  Touchdown .... Just above stalling speed - power off - main wheels first

LANDING ROLL

[ ]  Hold nose wheel off .... As long as possible

NOTE: Use up elevator to reduce weight on nose wheel throughout landing roll.

BRAKING

[ ]  Braking .... As required

[ ]  Flight Controls .... Full up elevator

Balked Landing (Go Around)

[ ]  Throttle .... Full Open (in)

[ ]  Carburetor Heat .... Cold (in)

[ ]  Flaps .... To 20°

NOTE: If more than 20° of flaps were in when the go-around is initiated, retract immediately to 20°. Continue to retract flaps in 10° increments only after establishing a positive rate of climb and reaching a safe altitude.

[ ]  Airspeed .... 55 KIAS (63 MPH)

Post-Landing Checklists

1. AFTER LANDING - CLEAR OF RUNWAY

[ ]  Flaps .... Full up (visual check)

[ ]  Carburetor Heat .... Off (in)

[ ]  Elevator Trim .... Takeoff position

2. IF HARD LANDING

[ ]  ELT .... Listen for on 121.5 on communications radio

3. ENGINE SHUTDOWN

[ ]  Radios/Electrical .... All off

[ ]  Throttle .... 1,000 RPM

[ ]  Mixture .... Idle cutoff

[ ]  Ignition .... Off

[ ]  Master Switch .... Off

4. SECURING THE AIRPLANE

[ ]  Parking Brake .... Set

[ ]  Control Lock .... Install

[ ]  Tiedown .... Wings and Tail

[ ]  Pitot Cover .... Install

[ ]  Double Check .... CHECK

NOTE: All electrical equipment - Off; Master Switch - Off.

5. CLOSE YOUR FLIGHT PLAN

Before Leaving Home

1. SELF CHECK

[ ]  Feeling ok .... Yes

[ ]  Under any stress .... No more than usual

[ ]  Taking any medication .... No

[ ]  Alcohol in last 12 Hrs. .... No

2. FLIGHT PLANNING/NAVIGATION EQUIPMENT

[ ]  Current Charts .... CHECK

[ ]  A/FD .... CHECK

[ ]  POH .... CHECK

[ ]  Airport Guide .... CHECK

[ ]  E6-B .... CHECK

[ ]  Plotter .... CHECK

[ ]  Calculator .... CHECK

[ ]  Timer .... CHECK

[ ]  Custom Checklists .... CHECK

[ ]  Flight Plans .... CHECK

[ ]  Weather Reports .... CHECK

[ ]  Pencils .... CHECK

[ ]  Clipboards .... CHECK

3. EMERGENCY ITEMS

[ ]  Hand Compass .... CHECK

[ ]  Knife .... CHECK

[ ]  Flashlights .... CHECK

[ ]  Batteries .... CHECK

[ ]  Bulbs .... CHECK

[ ]  Cell Phone (Charged) .... CHECK

[ ]  Spare Glasses .... CHECK

[ ]  Sun Glasses .... CHECK

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/152/c152_expanded_checklist_millstone_valley.txt

Rights: licensed

Checklist hash: d8fe8934dcfa540c9d36db2184590a6af079f14f1159c4202f8148941f47d0c7

Verify against the aircraft's own approved documentation before use.