Cessna 152 — c152

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Light Signals" item count differs between passes (7 vs 11); verify nothing was dropped.

Preflight Inspection

COCKPIT

[ ]  Aircraft docs .... Check

[ ]  Control wheel lock .... Remove

[ ]  Hobbs/Tach .... Check/Record

[ ]  Ignition .... Off

[ ]  Master Switch .... On

[ ]  Fuel quantity indicators .... check

[ ]  Flaps .... down

[ ]  Master Switch .... Off

[ ]  Fuel shutoff valve .... On

FUSELAGE AND EMPENAGE

[ ]  Rivets .... Check

[ ]  Rudder Gust Lock .... Remove

[ ]  Tail Tie-Down .... Disconnect

[ ]  Control surfaces .... free & secure

RIGHT WING

[ ]  Aileron .... free and secure

[ ]  Flaps .... secure

[ ]  Wing tie-down .... disconnect

[ ]  Main wheel tire .... inflated

[ ]  Brakes .... not leaking

[ ]  Fuel tank sump .... sample

[ ]  Fuel Quantity .... check

[ ]  Fuel Filler cap .... Secure

NOSE

[ ]  Engine oil level .... check

[ ]  Fuel strainer .... sample

[ ]  Propeller and spinner .... check

[ ]  Alternator belt .... check

[ ]  Air intake .... check

[ ]  Carburetor air filter .... check

[ ]  Landing lights .... check

[ ]  Nose wheel strut & tire .... check

[ ]  Nose-Tie down .... disconnect

[ ]  Static source opening .... check

LEFT WING

[ ]  Main wheel tire .... inflated

[ ]  Brakes .... not leaking

[ ]  Fuel tank sump .... sample

[ ]  Fuel Quantity .... check

[ ]  Fuel Filler cap .... Secure

[ ]  Wing tie-down .... disconnect

[ ]  Pitot tube .... uncover and check

[ ]  Stall warning .... check

[ ]  Fuel tank vent opening .... check

[ ]  Wing tie-down .... disconnect

[ ]  Aileron .... free and secure

[ ]  Flaps .... secure

Before starting engine

[ ]  Preflight inspection .... complete

[ ]  Seats, belts .... adjust and lock

[ ]  Fuel shutoff valve .... ON

[ ]  Radios & elect .... OFF

[ ]  Brakes .... test and set

[ ]  Circuit breakers .... IN

Starting Engine

[ ]  Mixture .... Rich (IN)

[ ]  Carb heat .... COLD (IN)

[ ]  Prime .... As required; locked

[ ]  Throttle .... Open ½ inch

[ ]  Propeller area .... CLEAR

[ ]  Master Switch .... ON

[ ]  Beacon .... ON

[ ]  Ignition .... START

[ ]  Throttle .... Adjust 1000 rpm

[ ]  Oil Pressure .... check normal

[ ]  Avionics .... ON

[ ]  Transponder .... Standby

[ ]  Flaps .... UP

Before Takeoff

[ ]  Parking brake .... SET

[ ]  Cabin doors .... CLOSE & LATCH

[ ]  Flight controls .... free & correct

[ ]  Instruments (4) .... SET

[ ]  Fuel Shutoff Valve .... ON

[ ]  Mixture .... RICH (IN)

[ ]  Trim .... set for TAKEOFF

[ ]  Throttle .... 1700 rpm

[ ]  Magnetos .... check

[ ]  Suction gage .... check

[ ]  Instruments .... check

[ ]  Ammeter .... check

[ ]  Carb heat .... ON

[ ]  Throttle .... IDLE -- chk

[ ]  Radios .... SET

[ ]  Beacon/lights .... ON

[ ]  Throttle friction lock .... ADJUST

[ ]  Brakes .... TEST

Engine failure  [EMERGENCY]

TAKEOFF

TAKEOFF

[ ]  Throttle .... IDLE

[ ]  Brakes .... apply

[ ]  Flaps .... UP

[ ]  Mixture .... IDLE cut-off

[ ]  Ignition .... OFF

[ ]  Master switch .... OFF

Engine failure  [EMERGENCY]

AFTER TAKEOFF

AFTER TAKEOFF

[ ]  Airspeed .... 60 KIAS

[ ]  Mixture .... Idle Cut-off

[ ]  Fuel shutoff valve .... OFF

[ ]  Ignition .... OFF

[ ]  Flaps .... as required

[ ]  Master switch .... OFF

Engine failure  [EMERGENCY]

DURING FLIGHT

DURING FLIGHT

[ ]  Airspeed .... 60 KIAS

[ ]  Carb Heat .... ON

[ ]  Primer .... IN & Locked

[ ]  Fuel Shutoff valve .... ON

[ ]  Mixture .... RICH

[ ]  Ignition .... BOTH (or START)

[ ]  Airspeed .... 60/65 (flaps)

[ ]  Mixture .... cutoff

[ ]  Fuel shutoff valve .... off

[ ]  Ignition .... off

[ ]  Flaps .... as required

[ ]  Mayday .... transmit 121.5

[ ]  Master switch .... off

[ ]  Doors .... unlatch

[ ]  Touchdown .... tail low

Carburetor Icing  [EMERGENCY]

[ ]  Throttle .... FULL

[ ]  Carb Heat .... ON (out)

[ ]  Mixture .... Adjust

Engine Roughness  [EMERGENCY]

[ ]  Magnetos .... check

[ ]  Mixture .... lean as necessary

Engine Fire during start  [EMERGENCY]

[ ]  Crank .... Continue

[ ]  Power .... 1700 RPM

[ ]  Engine .... Shut down and inspect

Engine Fire during flight  [EMERGENCY]

[ ]  Mixture .... Idle Cut-off

[ ]  Fuel shut off valve .... OFF

[ ]  Master Switch .... OFF

[ ]  Cabin head/air .... OFF

[ ]  Airspeed .... 85 KIAS

Electrical failure  [EMERGENCY]

[ ]  Load meter .... verify

[ ]  Alternator .... off

[ ]  Reduce load .... to minimum

[ ]  Breaker/alt .... chk & reset

[ ]  Alternator .... on

IF STILL NO POWER:

[ ]  Alternator .... off

[ ]  Reduce load and land

Electrical overload  [EMERGENCY]

[ ]  Master Switch .... OFF

[ ]  Master Switch .... ON

[ ]  Over-voltage light .... OFF

[ ]  or TERMINATE flight .... ASAP

Spin Recovery  [EMERGENCY]

[ ]  Ailerons .... NEUTRAL

[ ]  Throttle .... IDLE

[ ]  Rudder .... Full opposite

[ ]  Control wheel .... full forward

[ ]  Rudder control/wheel .... Neutral

[ ]  Pitch .... Level

Takeoff

[ ]  Flaps .... UP

[ ]  Carb heat .... COLD (IN)

[ ]  Transponder .... Altitude

[ ]  Trim .... set for TAKEOFF

[ ]  Throttle .... FULL

[ ]  Tach, oil, airspeed .... check

[ ]  Elevator .... Lift at 50 KIAS

[ ]  Climb .... 70-80 KIAS

Cruise

[ ]  Pitch .... Set

[ ]  Throttle .... As required

[ ]  Trim .... Set

[ ]  Mixture .... Adjust

Pre-landing checklist

[ ]  Fuel selector .... ON

[ ]  Mixture .... Rich

[ ]  Carb Heat .... ON

[ ]  Seatbelts .... Fastened

Approach

[ ]  Flight instruments .... Ckd & Set

[ ]  Radios .... Checked

[ ]  ATIS .... Checked

[ ]  Carb Heat .... ON

[ ]  Mixture .... Rich

After landing

[ ]  Flaps .... UP

[ ]  Carb Heat .... COLD (IN)

[ ]  Transponder .... Standby

[ ]  Landing light .... OFF

Parking

[ ]  Avionics .... OFF

[ ]  Electrical .... OFF

[ ]  Throttle .... 1000 RPM

[ ]  Mixture .... Cut-off

[ ]  Ignition switch .... OFF

[ ]  Master switch .... OFF

Securing the aircraft

[ ]  Control Lock .... Install

[ ]  Hobbs/Tach .... Record

[ ]  Door/Window .... Secure

[ ]  Tie-downs .... Secure

Short field take-off

[ ]  Take-off checklist .... complete

[ ]  Taxi .... max runway

[ ]  Brakes .... set and hold

[ ]  Flaps .... 10(

[ ]  Throttle .... FULL

[ ]  Brakes .... release

[ ]  Climb .... 54 KIAS

[ ]  Flaps .... retract when clear

[ ]  Airspeed .... 67 KIAS

Short field landing

[ ]  Pre-landing check .... complete

[ ]  Approach .... 54 KIAS

[ ]  Flaps .... 30(

[ ]  Throttle .... maintain glide

[ ]  Touchdown .... power off

[ ]  Flaps .... UP

[ ]  Elevator .... Full up

[ ]  Braking .... Heavy as required

Soft field take-off

[ ]  Take-off checklist .... complete

[ ]  Flaps .... 10(

[ ]  Taxi .... keep rolling

[ ]  Climb .... 54 KIAS

[ ]  Flaps .... retract

[ ]  Airspeed .... 67 KIAS

Soft field landing

[ ]  Pre-landing check .... complete

[ ]  Throttle .... 1500 RPM

[ ]  Flaps .... 10(

[ ]  Airspeed .... 60 KIAS

[ ]  Touchdown .... main first, softly

[ ]  Landing roll .... nose wheel up

[ ]  Elevator .... UP

[ ]  Braking .... as required

Go-around

[ ]  Throttle .... FULL

[ ]  Carb Heat .... Cold (IN)

[ ]  Flaps .... UP

[ ]  Airspeed .... 67 KIAS

Light Signals

LIGHT SIGNALS:

NOTE: Steady Green — On Ground: Takeoff; In Flight: Land

NOTE: Flashing Green — On Ground: Taxi; In Flight: Return to land

NOTE: Steady Red — On Ground: Stop; In Flight: Give way

NOTE: Flashing Red — On Ground: Clear runway; In Flight: Do not land

NOTE: Flashing white — On Ground: Return to ramp; In Flight: --

NOTE: Red/Green alternating — WARNING – USE CAUTION

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/152/c152.txt

Rights: licensed

Checklist hash: 7ca0a8f6d5cd02ca6759302b9fbd0a71ed9eead9337b98e8626784b7a5f70471

Verify against the aircraft's own approved documentation before use.