Cessna 152 — Matthew_Kudija_Cessna_152_EMERGENCY_PROCEDURES

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

ENGINE FAILURES  [EMERGENCY]

ENGINE FAILURE DURING TAKEOFF:

[ ]  Throttle .... IDLE

[ ]  Brakes .... APPLY

[ ]  Flaps .... RETRACT

[ ]  Mixture .... IDLE CUTOFF

[ ]  Ignition Switch .... OFF

[ ]  Master Switch .... OFF

ENGINE FAILURE AFTER TAKEOFF:

[ ]  Airspeed .... 60 KIAS

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Shutoff .... OFF

[ ]  Ignition Switch .... OFF

[ ]  Flaps .... AS REQUIRED

[ ]  Master Switch .... OFF

ENGINE FAILURE DURING FLIGHT:

[ ]  Airspeed .... 60 KIAS

[ ]  Carb Heat .... ON

[ ]  Primer .... IN and LOCKED

[ ]  Fuel Shutoff .... ON

[ ]  Mixture .... RICH

[ ]  Ignition Switch .... BOTH

GENERAL:

[ ]  Airspeed .... 60

[ ]  Best Field

COCKPIT CHECKS

[ ]  Carb Heat .... ON

[ ]  Mixture

[ ]  Throttle

[ ]  Fuel Shutoff

[ ]  Oil Press/Temp

[ ]  Fuel Level

FORCED LANDINGS  [EMERGENCY]

WITHOUT ENGINE POWER:

[ ]  Airspeed .... 65 (up) 60 (dn)

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Shutoff .... OFF

[ ]  Ignition Switch .... OFF

[ ]  Flaps .... FULL

[ ]  Master Switch .... OFF

[ ]  Doors .... UNLATCH

[ ]  Touchdown .... TAIL LOW

[ ]  Brakes .... APPLY HEAVILY

WITH ENGINE POWER:

[ ]  Airspeed .... 60 KIAS

[ ]  Flaps .... 20°

[ ]  Selected Field .... Fly Over

[ ]  Radio/Electrical .... OFF

[ ]  Flaps .... 30°

[ ]  Airspeed .... 55 KIAS

[ ]  Master Switch .... OFF

[ ]  Doors .... UNLATCH

[ ]  Touchdown .... Tail Low

[ ]  Ignition Switch .... OFF

[ ]  Brakes .... APPLY HEAVILY

DITCHING:

[ ]  Radio .... MAYDAY

[ ]  Objects .... Secure/Jettison

[ ]  Approach

[ ]  High Winds-Heavy Seas, Into wd

[ ]  Lt. Winds, Heavy Swells, Parallel

[ ]  Flaps .... 30°

[ ]  Power .... 300 FPM, 55 Kias

[ ]  Doors .... UNLATCH

[ ]  Touchdown .... LEVEL

[ ]  Face .... CUSION

[ ]  Airplane .... EVACUATE

[ ]  Vests/Raft .... INFLATE

FIRES  [EMERGENCY]

DURING START ON GROUND:

[ ]  Cranking .... CONTINUE

[ ]  Power .... 1700 RPM

[ ]  Shutdown and Inspect

ENGINE FIRE IN FLIGHT:

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Shutoff .... OFF

[ ]  Master Switch .... OFF

[ ]  Cabin Heat/Air .... OFF

[ ]  Airspeed .... 85 KIAS

[ ]  Forced Landing .... Execute

ELECTRICAL FIRE IN FLIGHT:

[ ]  Master Switch .... OFF

[ ]  All Switches .... OFF

[ ]  Cabin Air/Heat .... CLOSED

[ ]  Fire Ext. .... ACTIVATE

NOTE: if secure, and need electrical:

[ ]  Master Switch .... ON

[ ]  Breakers .... CHECK

[ ]  Switches .... On, one at time

[ ]  Vents .... OPEN

CABIN FIRE:

[ ]  Master Switch .... OFF

[ ]  Vents .... CLOSED

[ ]  Fire Ext. .... ACTIVATE

[ ]  LAND

WING FIRE:

[ ]  Nav. Light .... OFF

[ ]  Strobe Light .... OFF

[ ]  Pitot Heat .... OFF

[ ]  Side Slip

Flat Main Tire:  [EMERGENCY]

[ ]  Flaps .... As Desired

[ ]  Approach .... NORMAL

[ ]  Touchdown- good first

ELECTRICAL SYSTEM  [EMERGENCY]

OVER-VOLTAGE LIGHT ILLUMINATION:

[ ]  Master Switch .... OFF

[ ]  Master Switch .... ON

[ ]  Over-Voltage Light .... OFF

[ ]  Terminate Flight .... if ON

AMMETER SHOWS DISCHARGE:

[ ]  Alternator .... OFF

[ ]  Noness. Electrical .... OFF

[ ]  Flight .... TERMINATE

COMMUNICATION  [EMERGENCY]

[ ]  MAYDAY .... MAYDAY

[ ]  Name of Station

[ ]  Name of Aircraft

[ ]  Nature of Emergency

[ ]  Weather

[ ]  Intentions or Request

[ ]  Present position/ heading

[ ]  Altitude

[ ]  Fuel Remaining

[ ]  Number of people aboard

[ ]  Other Useful Info

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/152/Matthew_Kudija_Cessna_152_EMERGENCY_PROCEDURES.txt

Rights: licensed

Checklist hash: cd42458986946789919b896543245a37061ece5638a9634f303e97a07b19bbfc

Verify against the aircraft's own approved documentation before use.