Cessna 152 — Sam_Dawson_152n_ground_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Before Starting Engine

[ ]  Preflight Inspection .... Complete

[ ]  CRM/Passenger Briefing .... Complete

[ ]  Seats and Seat Belts .... Adjust & Lock

[ ]  Brakes .... Test & Set

[ ]  Fuel Selector .... ON

[ ]  Mixture .... Full Rich

[ ]  Throttle .... 1/2 inch

[ ]  Carburetor Heat .... Cold

[ ]  Avionics Master Switch .... Off

[ ]  Electrical Equipment .... Off

[ ]  Circuit Breakers .... Check In

Starting Engine

[ ]  Prime .... As Required

[ ]  Master Switch .... On

[ ]  Exterior Lights .... As Required

[ ]  Propeller Area .... Clear

[ ]  Ignition Switch .... Start

[ ]  Throttle .... 1000-1200 RPM

[ ]  Oil Pressure / Ammeter .... Check

Before Taxi

[ ]  Flaps .... Take Off Position

[ ]  Mixture .... Lean

[ ]  Throttle .... 1000 RPM

[ ]  Avionics Master (Radios) .... On

[ ]  ATIS / AWOS .... Receive

[ ]  Clearance .... Obtain

[ ]  Avionics / Transponder .... Set / STBY

[ ]  Flight Instruments .... Check & Set

[ ]  Engine Instruments .... Check

Taxi

[ ]  Brakes & Steering .... Check

[ ]  Flight Instruments .... Check

Before Takeoff

[ ]  Cabin Doors .... Closed & Locked

[ ]  Seats / Seat Belts .... Upright & Secured

[ ]  Flight Controls .... Free & Correct

[ ]  Clock .... Check

[ ]  Fuel Quantity .... Check

[ ]  Engine Instruments .... Check

[ ]  Flight Instruments .... Check & Set

[ ]  Radios / Avionics .... Set for Departure

[ ]  Fuel Selector .... ON

[ ]  Elevator Trim .... Check / Set

[ ]  Flaps .... Set for Takeoff

[ ]  Mixture .... Enrichen

[ ]  Throttle .... 1700 RPM

[ ]  Mixture .... Set for Takeoff

[ ]  Carb Heat .... Check

[ ]  Magnetos .... Check (±125/50)

[ ]  Engine Instruments / Ammeter .... Check

[ ]  Vacuum .... Green Arc

[ ]  Throttle .... Idle / 1000 RPM

[ ]  Departure Briefing .... Complete - Type T/O, Vr-, Vy-, Vx-, Emerg. Altitude

Shutdown and Securing

[ ]  Avionics Master Switch .... Off

[ ]  Exterior Lights (except beacon) .... Off

[ ]  Throttle .... 1000 RPM

[ ]  Mixture .... Idle Cut Off

[ ]  Magneto Switch .... Off

[ ]  Battery and Alternator .... Off

[ ]  Gust Lock .... Install

[ ]  Post Flight Inspection .... Accomplish

[ ]  Internal and External Covers .... Install

[ ]  Tie downs and Chocks .... Install

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/152/Sam_Dawson_152n_ground_checklist.txt

Rights: licensed

Checklist hash: 071fa46a55c41d543991d6b9ad084bbb85b43e3fa557c644e67f72257c4fe2ac

Verify against the aircraft's own approved documentation before use.