Cessna 152 — TFA_Cessna_152_Checklist_2004-01

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Before Takeoff" item count differs between passes (19 vs 20); verify nothing was dropped.

content_gap: Section "Starting Engine" item count differs between passes (14 vs 15); verify nothing was dropped.

Preflight Inspection Cockpit

[ ]  A.R.O.W. Documents .... CHECK

[ ]  Hobbs Meter .... CHECK

[ ]  Control Lock .... REMOVE

[ ]  Ignition Switch .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Master (look prop is clear) .... ON

[ ]  Fuel Quantity .... CHECK

[ ]  Lights .... CHECK

[ ]  Flaps .... 30 DEGREES

[ ]  Master Switch .... OFF

[ ]  Fuel Shutoff Valve .... ON

Preflight Inspection Fuselage & Empennage

[ ]  Rudder Gust Lock .... REMOVE

[ ]  Fuselage .... CHECK

[ ]  Tail Tie Down .... UNTIE

[ ]  Elevator .... FREE & SECURE

[ ]  Rudder .... FREE & SECURE

[ ]  VOR Antennas .... CHECK

[ ]  Trim Tab .... CHECK

Preflight Inspection Right Wing

[ ]  Flap .... SECURE

[ ]  Aileron .... FREE & SECURE

[ ]  Skin/Tip/Lights .... CHECK

[ ]  Wing Tie Down .... UNTIE

[ ]  Main Wheel Tire .... INFLATED

[ ]  Brakes .... CHECK

[ ]  Fuel Tank Sump .... SAMPLE

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Filler Cap .... SECURE

Preflight Inspection Nose

[ ]  Engine Oil .... 4-6 QUARTS

[ ]  Fuel Strainer .... SAMPLE

[ ]  Belly Quick Drain .... SAMPLE

[ ]  Propeller & Spinner .... CHECK

[ ]  Air Intakes .... FREE & OPEN

[ ]  Carburetor Air Filter .... CLEAR

[ ]  Landing Light .... SECURE

[ ]  Nose Wheel & Strut .... CHECK

[ ]  Static Port .... OPEN

Preflight Inspection Left Wing

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Filler Cap .... SECURE

[ ]  Main Wheel Tire .... INFLATED

[ ]  Brakes .... CHECK

[ ]  Pitot Tube & Drain .... FREE

[ ]  Stall Warning Opening .... CHECK

[ ]  Fuel Tank Vent .... OPEN

[ ]  Wing Tie Down .... UNTIE

[ ]  Skin/Tip/Lights .... CHECK

[ ]  Aileron .... FREE & SECURE

[ ]  Flap .... SECURE

[ ]  Fuel Tank Sump .... SAMPLE

Before Starting Engine

[ ]  Preflight .... COMPLETE

[ ]  Passenger Brief .... COMPLETE

[ ]  Seat/Seat Belts .... LOCKED

[ ]  Fuel Shutoff Valve .... ON

[ ]  Electrical Equipment .... OFF

[ ]  Circuit Breakers .... IN

[ ]  Brakes .... SET & HOLD

Starting Engine

[ ]  Mixture .... RICH

[ ]  Throttle .... OPEN 1/2 INCH

[ ]  Carburetor Heat .... COLD

[ ]  Prime .... AS REQUIRED

[ ]  Beacon .... CHECK ON

[ ]  Master Switch .... ON

[ ]  Propeller Area .... CLEAR

[ ]  Ignition Switch .... START

[ ]  Throttle .... SMOOTH IDLE

[ ]  Oil Pressure .... CHECK

[ ]  Carburetor Heat .... ON or OFF FOR WX & RNWY CONDITION

[ ]  Comm.Radio/Xpdr. .... ON/STBY

[ ]  Pitot Heat .... AS REQUIRED

[ ]  Flaps .... UP & VERIFIED

Before Takeoff

[ ]  Brakes .... SET & HOLD

[ ]  Doors & Windows .... LOCKED

[ ]  Controls .... FREE & CORRECT

[ ]  Trim .... SET FOR TAKEOFF

[ ]  Flight Instr. .... SET & CHECK

[ ]  Fuel Shutoff Valve .... ON

[ ]  Mixture .... RICH

[ ]  Throttle .... 1700 RPM

[ ]  Mags .... RT/BOTH/LEFT/BOTH

NOTE: Magnetos drop should not exceed 125 RPM or 50 RPM differential between magnetos

[ ]  Carb. Heat .... ON/CHECK/OFF

[ ]  Engine Instr. .... GREEN / NORM

[ ]  Ammeter / Alternator .... CHECK

[ ]  Suction Gauge .... CHECK

[ ]  Throttle .... SMOOTH IDLE

[ ]  Carburetor Heat .... ON or OFF FOR WX & RNWY CONDITION

[ ]  Comm./Nav. Radios .... SET

[ ]  Pitot Heat .... ON AS REQUIRED

[ ]  Throttle Lock .... TIGHTEN

Taking Runway

NOTE: Do prior to or by memory

[ ]  Clearance Received .... TAKE OFF or POSITION & HOLD

[ ]  Approach End .... CLEAR

[ ]  Lights .... ON

[ ]  Camera .... TRANSPONDER ALT

[ ]  Action .... D.G. TO RNWY HDG

[ ]  Time Off .... TAKEN

Normal Take Off

[ ]  Flaps .... 0 DEGREES

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... OFF

[ ]  Airspeed/Eng. Instr. .... CHECK

[ ]  Rotate .... 50 KIAS

[ ]  Climb Speed .... 65-75 KIAS

Soft Field Take Off

[ ]  Yoke .... FULL AFT

[ ]  Flaps .... 10 DEGREES

[ ]  Taxi .... KEEP ROLLING

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... OFF

[ ]  Airspeed/Eng. Instr. .... CHECK

[ ]  Lift Off .... SAFE AIRSPEED

[ ]  Accelerate .... Vx = 55 or Vy = 67

[ ]  Flaps (Over Obstacle Vy+) .... UP

Short Field Take Off

[ ]  Flaps .... 10 DEGREES

[ ]  Brakes .... HOLD

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... OFF

[ ]  Engine Instr. .... GREEN/NORM

[ ]  Brakes .... RELEASE

[ ]  Rotate .... 54 KIAS

[ ]  Climb Speed .... 54 KIAS

[ ]  Over Obstacle .... 67 KIAS & FLAPS UP

Enroute Climb

[ ]  Airspeed .... 70-80 KIAS

[ ]  Throttle .... FULL OPEN

[ ]  Mixture .... RICH OR AS REQ.

Cruise

[ ]  Airspeed .... ESTABLISHED

[ ]  Power 65% .... 2300 RPM

[ ]  Trim .... ADJUST

[ ]  Mixture .... LEAN AS REQUIRED

[ ]  Lights .... AS REQUIRED

[ ]  Flight Plan .... OPEN

[ ]  Company Freq. .... MONITOR

Descent

[ ]  Mixture .... RICH

[ ]  Carburetor Heat .... ON

[ ]  Throttle .... AS REQUIRED

[ ]  Landing Light .... VERIFY ON

Before Landing

[ ]  Carb. Heat .... VERIFY ON

[ ]  Gas (Fuel Valve) .... ON

[ ]  Undercarriage (Fixed) .... DOWN

[ ]  Mixture .... FULL RICH

[ ]  Primer .... IN & LOCKED

[ ]  Seatbelts / Switches .... CHECK

Normal Landings

[ ]  Airspeed .... 60-70 KIAS

[ ]  Flaps .... EXTENDED AS REQ.

[ ]  Airspeed (w/flaps) .... 55-65 KIAS

[ ]  Throttle .... AS REQ. TO RNWY

[ ]  Throttle .... TO IDLE ON RNWY

[ ]  Directional Control .... MAINTAIN

Soft Field Landing

[ ]  Airspeed .... 60-70 KIAS

[ ]  Flaps .... 30 DEGREES

[ ]  Short Final .... 54-60 KIAS

[ ]  Throttle .... AS REQUIRED

[ ]  Touchdown .... NOSE HIGH

[ ]  Nose Wheel .... HOLD OFF

[ ]  Elevator .... FULL AFT FOR TAXI

Short Field Landing

[ ]  Airspeed .... 60-70 KIAS

[ ]  Flaps .... 30 DEGREES

[ ]  Final .... MAINTAIN 54 KIAS

[ ]  Power .... REDUCE TO IDLE

[ ]  Flaps .... RETRACT

[ ]  Brakes .... APPLY HEAVILY

Go Around

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... COLD/OFF

[ ]  Flaps 20° .... IMMEDIATELY

[ ]  Airspeed .... INCREASING

[ ]  Flaps Up Increments .... 10° & 0°

After Landing

[ ]  Flaps .... UP

[ ]  Carburetor Heat .... COLD

[ ]  Transponder .... OFF

[ ]  Pitot Heat .... OFF

[ ]  Lights .... AS REQUIRED

[ ]  Taxi Clearance .... OBTAIN

Engine Shutdown & Securing Airplane

[ ]  Avionics/Avionics Master .... OFF

[ ]  Lights (except beacon) .... OFF

[ ]  Throttle .... MIN. 1000 RPM

[ ]  Mixture .... IDLE CUT OFF

[ ]  Ignition (prop stopped) .... OFF

[ ]  Master Switch .... OFF

[ ]  Control Lock .... INSTALL

[ ]  Tie Down Aircraft .... SECURELY

[ ]  Hobbs .... RECORD

[ ]  Post-flight Inspection .... DO

[ ]  Flight Plan .... CLOSE

Engine Fire During Flight  [EMERGENCY]

[ ]  Fuel Shutoff Valve [MEMORY] .... OFF

[ ]  Mixture [MEMORY] .... IDLE CUT-OFF

[ ]  Master Switch [MEMORY] .... OFF

[ ]  Cabin Heat/Air [MEMORY] .... OFF/CLOSED

[ ]  Airspeed .... 85 KIAS

NOTE: If fire is not extinguished, increase glide to an airspeed that provides an incombustible mixture

[ ]  Forced Landing .... EXECUTE

Electrical Fire In Flight  [EMERGENCY]

[ ]  Master Switch [MEMORY] .... OFF

[ ]  Switches (except Ignition) .... OFF

[ ]  Cabin Heat / Vents .... CLOSED

[ ]  Extinguisher .... ACTIVATE

[ ]  Fire Out .... VENTILATE CABIN

Engine Failure — Takeoff Run  [EMERGENCY]

[ ]  Throttle [MEMORY] .... IDLE

[ ]  Brakes .... APPLY

[ ]  Flaps .... UP

[ ]  Mixture .... IDLE CUT OFF

[ ]  Ignition Switch .... OFF

[ ]  Master Switch .... OFF

Engine Failure — After Takeoff  [EMERGENCY]

[ ]  Best Glide [MEMORY] .... 60 KIAS

[ ]  Mixture .... IDLE CUT OFF

[ ]  Fuel Shutoff Valve .... OFF

[ ]  Ignition Switch .... OFF

[ ]  Flaps .... AS REQUIRED

[ ]  Master Switch .... OFF

Engine Failure — During Flight  [EMERGENCY]

[ ]  Airspeed — Best Glide [MEMORY] .... 60 KIAS

[ ]  Best Place To Land .... TURN TO

[ ]  Carburetor Heat .... ON

[ ]  Fuel Shutoff Valve .... ON

[ ]  Mixture .... RICH

[ ]  Ignition Switch .... ON BOTH / OR START IF PROP. IS STOPPED

[ ]  Primer .... IN & LOCKED

[ ]  Trouble Shoot .... MAGS CYCLE, THROTTLE PUMP

NOTE: Unable To Re-start:

[ ]  Declare May Day .... 121.5 or ATC

[ ]  Squawk .... 7700

Forced Landing Without Engine Power  [EMERGENCY]

[ ]  Airspeed .... 65 KIAS FLAPS UP / 60 KIAS FLAP DOWN

[ ]  Fuel Shutoff Valve .... OFF

[ ]  Mixture .... IDLE CUT OFF

[ ]  Ignition Switch .... OFF

[ ]  Doors .... UNLATCH

[ ]  Flaps .... AS REQUIRED

[ ]  Master Switch .... OFF LAST

[ ]  Touchdown .... TAIL LOW

[ ]  Brakes .... APPLY HEAVILY

Precautionary Landing w/Engine Power  [EMERGENCY]

[ ]  Airspeed .... 60 KIAS

[ ]  Selected Field .... FLY OVER

NOTE: Note any obstacles

[ ]  Electrical Equipment .... OFF

[ ]  Flaps .... 30 DEGREES

[ ]  Airspeed .... 55 KIAS

[ ]  Master Switch .... OFF

[ ]  Doors .... UNLATCH

[ ]  Touchdown .... TAIL LOW

[ ]  Ignition Switch .... OFF

[ ]  Brakes .... APPLY HEAVILY

Electrical Malfunction  [ABNORMAL]

OVER VOLTAGE CONDITION

[ ]  Alternator .... OFF

[ ]  Alternator Circuit Breaker .... PULL

[ ]  Nonessential Electrical .... OFF

[ ]  Flight .... TERMINATE as soon as practical

LOW VOLTAGE CONDITION

[ ]  Avionics / Electrical .... OFF

[ ]  Alternator Circuit Breaker .... IN

[ ]  Master .... OFF BOTH SIDES

[ ]  Master .... ON

[ ]  Voltage Light .... CHECK OFF

NOTE: If low-voltage light illuminates again

[ ]  Alternator .... OFF

[ ]  Nonessential Electrical .... OFF

[ ]  Flight .... TERMINATE as soon as practical

Wing Fire  [EMERGENCY]

[ ]  Nav Lights .... OFF

[ ]  Strobe Lights .... OFF

[ ]  Pitot Heat .... OFF

[ ]  Perform .... SIDE SLIP AWAY FROM THE FUEL TANK AND CABIN

Engine Fire During Start  [EMERGENCY]

Engine fire during start

[ ]  Cranking .... CONTINUE

NOTE: IF ENGINE STARTS

[ ]  Power .... 1700 RPM/1 MIN.

[ ]  Engine .... SHUTDOWN

[ ]  Airplane .... INSPECT

NOTE: ENGINE FAILS TO START

[ ]  Cranking .... CONTINUE

[ ]  Throttle .... FULL OPEN

[ ]  Mixture .... IDLE CUT OFF

[ ]  Fuel Shutoff Valve .... OFF

[ ]  Master Switch .... OFF

[ ]  Ignition Switch .... OFF

[ ]  Damage Inspection .... DO

Operating Information

BEST GLIDE 60 KIAS

VX 55 KIAS

VY 67 KIAS

VA 93-104 KIAS

VNE 149 KIAS

VNO 111 KIAS

VFE 85 KIAS

LOCAL FREQUENCIES

NOTE: All take flight aircraft are required to monitor company frequency when in range and able to do so.

NOTE: Take Flight Alaska 129.25

NOTE: Merrill ATIS 123.7

NOTE: Merrill Ground 121.7

NOTE: Merrill Tower 126.0/127.55

NOTE: Anchorage Appch. N119.1/S126.4

NOTE: Big Lake/Willow/Wasilla 122.8

NOTE: Goose Bay 122.9

NOTE: Birchwood 123.0

NOTE: Cool/Cold Engine T.F.A. Policy: Ensure either oil temp. is in the green or moving prior to engine run ups, mag. checks and prior to flight. Pre-heating a cold engine is required before starting during winter operations.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/152/TFA_Cessna_152_Checklist_2004-01.txt

Rights: licensed

Checklist hash: a57f93c1b620ebda569583069edde938f9a2680b2151be0eea4c07d318f81ad5

Verify against the aircraft's own approved documentation before use.