Cessna 172F Skyhawk — C-172FChecklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

CABIN

[ ]  Papers .... A.R.O.W.

[ ]  Control Wheel Lock .... REMOVE

[ ]  Mags .... OFF

[ ]  Avionics .... OFF

[ ]  Circuit Breakers .... IN

[ ]  Trim .... TAKEOFF

[ ]  Fuel Selector .... CHECK EACH POSITION, SET BOTH

[ ]  Prop .... CLEAR

[ ]  Master .... ON

[ ]  Fuel Gauges .... TRUE

[ ]  Flaps .... EXTEND

[ ]  Lights .... INTERIOR / EXTERIOR

[ ]  Pitot Heat .... TEST

[ ]  Stall Indicator .... TEST

[ ]  Master .... OFF

[ ]  Fuel Strainer .... PULL (4 sec)

[ ]  Baggage Door .... LOCK

EMPENNAGE

[ ]  Left Elevator .... CHECK

[ ]  Tiedown .... DISCONNECT

[ ]  Rudder .... CHECK

[ ]  Rudder and Elevator Cables .... CHECK

[ ]  Navigation Light .... SECURE

[ ]  Trim Tab Alignment .... CHECK (within ¼" of elevator)

[ ]  Right Elevator .... CHECK

[ ]  Antennas .... CHECK

RIGHT WING

[ ]  Flap .... CHECK

[ ]  Aileron .... CHECK

[ ]  Strobe/Navigation Lights .... CHECK

[ ]  Tiedown .... DISCONNECT

[ ]  Fuel Quantity .... CHECK VISUALLY

[ ]  Fuel Filler Cap .... SECURE

[ ]  Right Fuel Tank Sump .... DRAIN

[ ]  Chock .... REMOVE

[ ]  Tire .... CHECK (24 PSI)

[ ]  Brake Assembly .... CHECK

NOSE

[ ]  Chock .... REMOVE

[ ]  Nose Strut .... 1-3" EXTENSION

[ ]  Nose Tire .... CHECK (26 PSI)

[ ]  Exhaust System .... CHECK

[ ]  Air Filter .... CLEAR

[ ]  Engine Air Inlets .... CLEAR

[ ]  Prop/Spinner .... CHECK

[ ]  Oil Level .... 7 QTS (under 3 HR flight), 8 QTS (extended flight)

[ ]  Oil Cap .... SECURE

[ ]  Engine Compartment .... CHECK

[ ]  Access Door .... SECURE

[ ]  Static Port .... CLEAR

LEFT WING

[ ]  Chock .... REMOVE

[ ]  Tire .... CHECK (24 PSI)

[ ]  Brake Assembly .... CHECK

[ ]  Left Fuel Tank Sump .... DRAIN

[ ]  Fuel Quantity .... CHECK VISUALLY

[ ]  Fuel Filler Cap .... SECURE

[ ]  Pitot Tube .... CLEAR

[ ]  Fuel Vent .... CLEAR

[ ]  Tiedown .... DISCONNECT

[ ]  Landing/Taxi Lights .... CHECK

[ ]  Strobe/Navigation Lights .... CHECK

[ ]  Aileron .... CHECK

[ ]  Flap .... CHECK

START

[ ]  Hobbs Time .... RECORDED

[ ]  Passenger Brief .... COMPLETE

[ ]  Seat Track/Back .... LOCK

[ ]  Seat Belt /Harness .... FASTEN

[ ]  Brakes .... SET

[ ]  Beacon .... ON

[ ]  Navigation Lights .... AS REQUIRED

[ ]  Mixture .... RICH

[ ]  Throttle .... OPEN 1/8"

[ ]  Carb Heat .... OFF

[ ]  Prime .... AS REQUIRED

[ ]  Prop .... CLEAR

[ ]  Master .... ON

[ ]  Mags .... START

[ ]  Throttle .... 1,000 RPM

[ ]  Ammeter .... CHECK

[ ]  Oil Pressure .... CHECK

[ ]  Mixture .... LEAN

PRE-TAXI

[ ]  Flaps .... UP

[ ]  Cabin Heat & Air .... AS DESIRED

[ ]  Avionics .... ON / SET

[ ]  Transponder .... CODE / STANDBY

[ ]  Clock .... SET

[ ]  Airspeed Indicator .... CHECK

[ ]  Attitude Indicator .... SET

[ ]  Heading Indicator .... SET

[ ]  VSI .... CHECK

[ ]  ATIS/AWOS .... COPY

[ ]  Altimeter .... SET / CHECK

[ ]  Taxi Light .... AS REQUIRED

TAXI

[ ]  Brakes .... TEST

[ ]  Attitude Indicator .... CHECK

[ ]  Turn-and-Slip Indicator .... CHECK

[ ]  Heading Indicator .... CHECK

[ ]  Magnetic Compass .... CHECK

RUN-UP

[ ]  Brakes .... SET

[ ]  Mixture .... BEST POWER

[ ]  Primer .... IN & LOCKED

[ ]  Throttle .... 1,600 RPM

[ ]  Mags (R & L) .... CHECK (125 RPM max drop, 75 RPM max differential)

[ ]  Carb Heat .... TEST

[ ]  Vacuum .... CHECK

[ ]  Ammeter .... CHECK (0 to +2 needle widths)

[ ]  Oil Pressure .... CHECK

[ ]  Oil Temperature .... CHECK

[ ]  Throttle .... IDLE CHECK

[ ]  Throttle .... 1,000 RPM

[ ]  Mixture .... LEAN

TAKEOFF

[ ]  Heading Indicator .... SET

[ ]  Avionics .... SET

[ ]  Engine Gauges .... CHECK

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Selector .... BOTH

[ ]  Carb Heat .... OFF

[ ]  Mixture .... BEST POWER

[ ]  Trim .... TAKEOFF

[ ]  Flaps .... SET (0-10°)

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Doors/Window .... LOCKED

[ ]  Takeoff Brief .... COMPLETE

TAKING RUNWAY (LIGHTS, CAMERA, ACTION)

[ ]  Strobes .... ON

[ ]  Landing Light .... ON

[ ]  Transponder .... ALTITUDE

[ ]  Mixture .... BEST POWER

AFTER TAKEOFF

[ ]  Flaps .... UP

[ ]  Engine Gauges .... CHECK

[ ]  Landing Light .... AS DESIRED

CRUISE

[ ]  Landing Light .... OFF

[ ]  Throttle .... SET

[ ]  Mixture .... LEAN

[ ]  Engine Gauges .... CHECK

[ ]  Heading Indicator .... SET

[ ]  Fuel Selector .... BALANCE AS REQUIRED

FUEL MANAGEMENT ABOVE 5,000’ MSL

[ ]  When reaching cruise altitude above 5,000’ MSL, promptly switch fuel selector from BOTH to either LEFT or RIGHT and then to opposite tank to ensure both tanks are feeding.

[ ]  During cruise, use LEFT and RIGHT tank (not BOTH) as required to manage fuel balance.

APPROACH

[ ]  Mixture .... RICHEN AS REQUIRED

[ ]  Fuel Selector .... BOTH (below 5,000’ MSL)

[ ]  Carb Heat .... AS REQUIRED

[ ]  ATIS/AWOS .... COPY

[ ]  Altimeter .... SET

[ ]  Avionics .... SET

[ ]  Heading Indicator .... SET

[ ]  Engine Gauges .... CHECK

[ ]  Brakes .... PEDAL TEST

[ ]  Seatbelt/Harness .... FASTENED

[ ]  Landing Light .... ON

LANDING

[ ]  Fuel Selector .... BOTH

[ ]  Mixture .... BEST POWER

[ ]  Carb Heat .... ON

[ ]  Flaps .... AS DESIRED

AFTER LANDING

[ ]  Carb Heat .... OFF

[ ]  Mixture .... LEAN

[ ]  Flaps .... UP

[ ]  Strobes .... OFF

[ ]  Landing Light .... OFF

[ ]  Taxi Light .... AS REQUIRED

[ ]  Pitot Heat .... OFF

[ ]  Transponder .... STANDBY

SHUTDOWN

[ ]  Avionics .... OFF

[ ]  Fuel Selector .... RIGHT

[ ]  Throttle .... 1,000 RPM

[ ]  Mags .... GROUNDING CHECK

[ ]  Mixture .... FULL LEAN

[ ]  Prop .... CHECK STOPPED

[ ]  Mags .... OFF & KEY ON DASH

[ ]  Master .... OFF

[ ]  Lights .... OFF

SECURING

[ ]  Hobbs/Tach Time .... RECORDED

[ ]  Control Wheel Lock .... INSTALL

[ ]  Chocks .... INSTALL

[ ]  Tiedowns .... SECURE

[ ]  Baggage Door .... CLOSED

[ ]  Cabin Doors/Window .... CLOSED

FLOODED ENGINE  [ABNORMAL]

[ ]  Mixture .... FULL LEAN

[ ]  Throttle .... FULL OPEN

[ ]  Mags .... START (hold for several revolutions)

[ ]  Repeat START checklist beginning on step 10 (do not reprime).

GROUND BURNOUT  [ABNORMAL]

[ ]  Mixture .... RICH

[ ]  Throttle .... 2,000 RPM

[ ]  Mixture .... LEAN TO 50 RPM DROP BELOW BEST POWER

AFTER 1 MINUTE:

[ ]  Mixture .... RICH

[ ]  Mags .... RECHECK

RADIO FAILURE  [ABNORMAL]

[ ]  Volume .... CHECK

[ ]  Headset Connection .... CHECK

[ ]  Audio Controls .... CHECK

[ ]  Circuit Breakers .... CHECK

[ ]  Speaker .... ON

[ ]  Hand-Held Microphone .... CHECK

[ ]  Frequency .... SWITCH

[ ]  If IFR (or VFR in Class B, C, D airspace): Transponder .... 7600

NOTE: If failure occurs in VFR conditions, or VFR conditions are encountered after failure, continue flight under VFR and land as soon as practicable.

VFR LANDING AT A CONTROLLED FIELD:

[ ]  Remain outside or above Class D airspace until traffic flow determined.

[ ]  Transmit intentions "in the blind" advising tower of aircraft type, position, altitude, and intention to land.

[ ]  Enter pattern, report position "in the blind", and watch for light signals from tower.

NOTE: If failure occurs in IFR conditions, continue flight according to the following:

ROUTE TO FLY (IN ORDER): (AVE. F)

[ ]  A – Assigned

[ ]  V – Vectored

[ ]  E – Expected

[ ]  F – Filed

ALTITUDE (HIGHEST OF FOR THE ROUTE SEGMENT BEING FLOWN): (MEA)

[ ]  M – Minimum IFR Altitude

[ ]  E – Expected as Advised by ATC

[ ]  A – Assigned by ATC

DESCENT FOR APPROACH:

[ ]  From enroute altitude upon reaching the IAF but not before:

[ ]  Expect-further-clearance time (if given)

[ ]  ETA as calculated from filed ETE

NOTE: ATC LIGHT SIGNALS

ABORT  [EMERGENCY]

[ ]  Throttle .... IDLE

[ ]  Brakes .... AS REQUIRED

ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF  [EMERGENCY]

[ ]  Airspeed [MEMORY] .... 80 MPH (flaps UP)

[ ]  Mixture [MEMORY] .... FULL LEAN

[ ]  Fuel Selector [MEMORY] .... OFF

[ ]  Flaps [MEMORY] .... DOWN (70 MPH glide)

[ ]  Master & Mags [MEMORY] .... OFF

COMPLETE ENGINE FAILURE DURING FLIGHT  [EMERGENCY]

[ ]  Airspeed [MEMORY] .... 80 MPH (flaps UP)   (Glide Ratio = 1.5 SM/1,000’ AGL)

IF AIRSTART WARRANTED:

[ ]  Carb Heat .... ON

[ ]  Mixture .... RICH

[ ]  Fuel Selector .... BOTH

[ ]  Primer .... IN & LOCKED

[ ]  Mags .... CHECK ALL

[ ]  Master .... ON

FORCED LANDING  [EMERGENCY]

[ ]  Airspeed [MEMORY] .... 80 MPH (flaps UP)

[ ]  Squawk [MEMORY] .... 7700

[ ]  Emergency [MEMORY] .... DECLARE

[ ]  ELT [MEMORY] .... ON

[ ]  Mixture [MEMORY] .... FULL LEAN

[ ]  Fuel Selector [MEMORY] .... OFF

[ ]  Seatbelt/Harness [MEMORY] .... FASTENED

[ ]  Flaps [MEMORY] .... AS REQUIRED (70 MPH with flaps full)

[ ]  Master & Mags [MEMORY] .... OFF

[ ]  Doors [MEMORY] .... UNLATCH

ENGINE ROUGHNESS  [EMERGENCY]

IF OVER 5,000’ MSL:

[ ]  Vapor In Fuel Tank .... EXECUTE

IF 5,000’ MSL OR BELOW, OR VAPOR IN FUEL TANK PROCEDURE DOESN’T CORRECT:

[ ]  Carb Heat .... ON

IF ROUGHNESS CONTINUES AFTER 1 MIN:

[ ]  Carb Heat .... OFF

[ ]  Mixture .... ADJUST FOR MAX SMOOTHNESS

[ ]  Fuel Selector .... BOTH

[ ]  Primer .... IN & LOCKED

[ ]  Engine Gauges .... CHECK

[ ]  Mags .... L then R then BOTH

[ ]  Mags .... SMOOTHEST SETTING

[ ]  Power .... REDUCE

[ ]  Mixture .... FULL RICH

[ ]  LAND AT NEAREST AIRPORT .... EXECUTE

VAPOR IN FUEL TANK (above 5,000 MSL)  [EMERGENCY]

DURING OPERATION ON A SINGLE TANK:

[ ]  Fuel Selector .... SWITCH TO OPPOSITE TANK

DURING OPERATION ON BOTH TANKS:

[ ]  Fuel Selector .... SWITCH TO A SINGLE TANK FOR 60 SECONDS

[ ]  Fuel Selector .... SWITCH TO OPPOSITE TANK

HIGH OIL TEMPERATURE  [EMERGENCY]

[ ]  Power .... REDUCE

[ ]  Mixture .... RICH

[ ]  Airspeed .... INCREASE

IF HIGH TEMP PERSISTS:

[ ]  LAND AT NEAREST AIRPORT .... EXECUTE

LOSS OF OIL PRESSURE  [EMERGENCY]

[ ]  Oil Temp .... CHECK

[ ]  Power .... REDUCE

IF OIL TEMP SHOWS RISE, ENGINE FAILURE IMMINENT:

[ ]  Forced Landing .... EXECUTE

IF OIL TEMP DOES NOT SHOW RISE:

[ ]  LAND AT NEAREST AIRPORT .... EXECUTE

ENGINE FIRE ON GROUND  [EMERGENCY]

[ ]  Mixture [MEMORY] .... FULL LEAN

[ ]  Fuel Selector [MEMORY] .... OFF

[ ]  Mags [MEMORY] .... OFF

[ ]  Master [MEMORY] .... OFF

ENGINE FIRE IN FLIGHT  [EMERGENCY]

[ ]  Mixture [MEMORY] .... FULL LEAN

[ ]  Fuel Selector [MEMORY] .... OFF

[ ]  Master & Mags [MEMORY] .... OFF

[ ]  Cabin Heat & Air [MEMORY] .... OFF

[ ]  Airspeed [MEMORY] .... 105 MPH

[ ]  Forced Landing [MEMORY] .... EXECUTE

ELECTRICAL FIRE  [EMERGENCY]

[ ]  Master [MEMORY] .... OFF

WING FIRE  [EMERGENCY]

[ ]  Landing/Taxi Lights .... OFF

[ ]  Navigation Lights .... OFF

[ ]  Strobe Lights .... OFF

[ ]  Pitot Heat .... OFF

[ ]  Sideslip .... AS REQUIRED

[ ]  Forced Landing .... EXECUTE

SMOKE AND FUMES IN COCKPIT  [EMERGENCY]

[ ]  Cabin Heat Knob .... IN

[ ]  Cabin Air Knob .... IN

[ ]  Upper Air Vents .... OPEN

[ ]  Pilot’s Window .... AS REQUIRED (below 100 MPH)

HIGH AMMETER READING (more than 2 needle widths positive deflection)  [EMERGENCY]

[ ]  Master .... OFF

NEGATIVE AMMETER READING  [EMERGENCY]

[ ]  Electrical Load .... REDUCE

VACUUM SYSTEM FAILURE  [EMERGENCY]

IF VMC:

[ ]  Attitude Indicator & Heading Indicator .... COVER

[ ]  Maintain VMC .... EXECUTE

IF IMC:

[ ]  Attitude Indicator & Heading Indicator .... COVER

[ ]  Fly using Turn-and-Slip Indicator, Altimeter, and Airspeed Indicator

[ ]  Declare EMERGENCY .... EXECUTE

INOPERATIVE TRIM  [EMERGENCY]

[ ]  Flaps .... AS REQUIRED

[ ]  Power .... AS REQUIRED

FLAP MALFUNCTION  [EMERGENCY]

[ ]  Circuit Breaker .... CHECK / RESET

IF BREAKER POPS AGAIN:

[ ]  Flaps .... LEAVE WHERE SET

STRUCTURAL ICING  [EMERGENCY]

[ ]  Pitot Heat .... ON

[ ]  Carb Heat .... ON

[ ]  Throttle .... MAX RPM

[ ]  Mixture .... BEST POWER

[ ]  Cabin Heat .... MAX DEFROST

[ ]  EXIT ICING CONDITIONS .... EXECUTE

IF LANDING WITH STRUCTURAL ICING:

[ ]  Flaps .... LEAVE UP

DOOR OR WINDOW OPENING IN FLIGHT  [EMERGENCY]

[ ]  FLY THE AIRPLANE [MEMORY] .... EXECUTE

IF WINDOW:

[ ]  Window .... CLOSE / LOCK

IF DOOR:

[ ]  LAND AS SOON AS PRACTICABLE .... EXECUTE

LANDING WITH A FLAT MAIN TIRE  [EMERGENCY]

[ ]  Approach .... NORMAL

[ ]  Flaps .... 30°

[ ]  Touchdown .... GOOD TIRE FIRST

[ ]  Hold airplane off flat tire as long as possible

[ ]  Maintain directional control using good-tire brake

LANDING WITH A FLAT NOSE TIRE  [EMERGENCY]

[ ]  Approach .... NORMAL

[ ]  Flaps .... AS REQUIRED

[ ]  Touchdown .... ON MAINS

[ ]  Hold nosewheel off ground as long as possible

[ ]  When nosewheel touches down .... FULL UP ELEVATOR

SPIN RECOVERY  [EMERGENCY]

[ ]  Power [MEMORY] .... Idle

[ ]  Ailerons [MEMORY] .... Neutral

[ ]  Rudder [MEMORY] .... Full opposite rotation direction

[ ]  Yoke [MEMORY] .... Full forward with neutral ailerons

[ ]  Rudder [MEMORY] .... Neutral when rotation stops

[ ]  Yoke [MEMORY] .... Pull smoothly from dive

INITIAL

[ ]  Wx & Density Alt

[ ]  Weight & Balance

[ ]  Performance Req

[ ]  Flight Plan - File

[ ]  Papers - A.R.O.W.

[ ]  Control Lock

[ ]  Fuel - Both

[ ]  Master - On

[ ]  Flaps - Extend

[ ]  Pitot Heat - Test

[ ]  Lights - Int/Ext

[ ]  Stall Indicator - Test

[ ]  Fuel Gauges - True

[ ]  Master - Off

[ ]  Fuel Strainer

EXTERIOR

[ ]  Fuel Quantity

[ ]  Fuel Quality

[ ]  Caps/Drains/Vents

[ ]  Engine/Oil

[ ]  Prop/Air Intake

[ ]  Exhaust System

[ ]  Surfaces & Controls

[ ]  Pitot & Static Ports

[ ]  Gear/Tires/Brakes

[ ]  Antennas

[ ]  Ties/Chocks

[ ]  Baggage Door

[ ]  Final Walk Around

INTERIOR

[ ]  Passenger Brief

[ ]  Hobbs Time

[ ]  Circuit Breakers

[ ]  Brakes - Pedal Test

START

[ ]  Seat Track/Back - Lock

[ ]  Avionics - Off

[ ]  Carb Heat - Off

[ ]  Beacon - On

[ ]  Nav Lights - As Req

[ ]  Mixture - Full Rich

[ ]  Throttle - Slight

[ ]  Prime - As Req

[ ]  Brakes

[ ]  Prop - Clear

[ ]  Master - On

[ ]  Mags - Start

[ ]  1,000 RPM

[ ]  Ammeter

[ ]  Oil Pressure

[ ]  Mixture - Lean

PRE-TAXI/TAXI

[ ]  Seat Belts/Harness

[ ]  Flaps - Up

[ ]  Heat/Vent

[ ]  Avionics - On/Set

[ ]  Transponder - Stby

[ ]  ATIS/AWOS

[ ]  Altimeter - Set

[ ]  Taxi Light - As Req

[ ]  Brakes - Test

[ ]  Attitude Indic - Test

[ ]  Turn & Slip - Test

[ ]  H.I./Compass - Test

RUN-UP

[ ]  Brakes - Set

[ ]  Fuel - Both

[ ]  Trim - Takeoff

[ ]  Flight Controls

[ ]  Instruments

[ ]  Mixture - Best Power

[ ]  Primer - In & Locked

[ ]  1600 RPM

[ ]  Mags (R&L) - Test

[ ]  Carb Heat - Test

[ ]  Vacuum

[ ]  Amps

[ ]  Oil Pressure

[ ]  Idle - Check Closed

[ ]  Mixture - Lean

TAKEOFF

[ ]  Flaps - 0-10°

[ ]  Carb Heat - Off

[ ]  Pitot Heat - As Req

[ ]  H.I. to Compass

[ ]  Doors/Window

[ ]  Takeoff Brief

TAKING RUNWAY

(LIGHTS, CAMERA, ACTION)

[ ]  Strobes - On

[ ]  Landing Light - On

[ ]  Transponder - Altitude

[ ]  Mixture - Best Power

CLIMB

[ ]  80-90 MPH

[ ]  Flaps - Up

[ ]  Power

[ ]  Mixture

[ ]  Instruments

[ ]  Landing Light - Off

CRUISE

[ ]  Power

[ ]  Mixture

[ ]  Instruments

[ ]  Fuel Selector - As Req

[ ]  H.I. To Compass

APPROACH

[ ]  Mixture - Richen

[ ]  Fuel - Both

[ ]  Carb Heat - As Req

[ ]  ATIS/AWOS

[ ]  Altimeter - Set

[ ]  Instruments

[ ]  H.I. To Compass

[ ]  Landing Light - On

[ ]  Seatbelt/Harness

LANDING

[ ]  Fuel - Both

[ ]  Mixture - Best Power

[ ]  Carb Heat - On

[ ]  Flaps - As Req

AFTER LANDING

[ ]  Flaps - Up

[ ]  Carb Heat - Off

[ ]  Mixture - Lean

[ ]  Strobes - Off

[ ]  Landing Light - Off

[ ]  Taxi Light - As Req

[ ]  Pitot Heat - Off

[ ]  Trim - Takeoff

[ ]  Transponder - Stby

SHUTDOWN

[ ]  Avionics - Off

[ ]  Fuel - Right

[ ]  Mags - Ground Check

[ ]  Mixture - Full Lean

[ ]  Mags - Off

[ ]  Master - Off

[ ]  Lights - Off

SECURING

[ ]  Hobbs/Tach Time

[ ]  Control Lock

[ ]  Chocks

[ ]  Tie Downs

[ ]  Baggage Door

[ ]  Cabin Doors/Window

AIRSPEEDS MPH IAS

[ ]  VS0   38

[ ]  VS1   44

[ ]  VR    60

[ ]  VX    66

[ ]  Short/Soft Final (40°) 70

[ ]  Normal Final (30°)     75

[ ]  VY    80

[ ]  Vmax Glide  80

[ ]  Final (0°)  80

[ ]  Base (20°)  80

[ ]  Past Abeam (10°) 90

[ ]  Cruise Climb     90

[ ]  VFE   100

[ ]  Downwind    100

[ ]  VA    122

[ ]  VNO   140

[ ]  VNE   174

DATA

[ ]  Engine: Continental O-300-D 145 HP

[ ]  Max T.O. Weight: 2300 #

[ ]  Max Bag Area:    120 #

[ ]  Fuel: 100LL (Blue)

[ ]  Usable Fuel:     36 Gal

[ ]  Oil Capacity:    8 Qts (Min 6)

[ ]  Electrical: 12-14 V 60 Amp

[ ]  Nose Tire:  26 PSI

[ ]  Main Tire:  24 PSI

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172F Skyhawk/C-172FChecklist.txt

Rights: licensed

Checklist hash: 24725a796bb02e1203c50a828996e1ca4e7c07c16ebfd1e761040e51c3c581ae

Verify against the aircraft's own approved documentation before use.