Cessna 172F Skyhawk — Frederic_KIEN_CESSNA_FR172J_CHECKLIST

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Avant démarrage moteur

[ ]  Frein de parking .... serre

[ ]  Pilote passages .... attaches

[ ]  Portes .... verrouillées

[ ]  Mano admission .... 30"

[ ]  Richesse (poussée) .... plein riche

[ ]  Hélice .... P.P.P.

Démarrage du moteur

[ ]  Contact général .... On

[ ]  Rotating .... On

[ ]  Essence .... Both

[ ]  Régime .... 1000 tr.

NOTE: (poussé 6 mm)

[ ]  Personne devant Start

[ ]  Pression d'huile .... Vert

[ ]  Ampèremètre .... Positif

Après démarrage

[ ]  Capot .... La demande

[ ]  Volets .... 0"

[ ]  Taxi light .... On

[ ]  Conserv. de cap .... Règle

[ ]  Altimètre .... Règle

[ ]  Avionic .... On

[ ]  Radio. S .... Règle

[ ]  Message radio

Roulage

[ ]  Frein .... desserre, essayes

Avant décollage — Point d'attente

[ ]  Frein parking .... serre

[ ]  Manche palonnier .... libre

[ ]  Pump. Aux. .... On/montée Press.off

[ ]  Instruments .... vert

[ ]  Régine .... 1800 tr / mn.

[ ]  Select. Magnet .... 150 tr / mn. Max

[ ]  Régulation hélice .... 2 fois

[ ]  Dépression .... 4 à 6

[ ]  Râlent .... 600 tr.

[ ]  Régime .... 1000 tr.

[ ]  Compensateur posi .... décollage

[ ]  Vérification Chek pré-décollage

[ ]  Portes .... Verrouillés

[ ]  Essence .... Both

[ ]  Capot .... Ouvert

[ ]  Compensateur Posi .... Décoll.

[ ]  Volet .... 10°

[ ]  Richesse .... Plein riche

[ ]  Hélice .... P.P.P.

[ ]  Rech.F. Carbu .... Plein Froid

[ ]  Taxi light .... Off

[ ]  Landing Light .... On (Nav. Light)

[ ]  Fusibles .... Bon

[ ]  Magneto .... Both

[ ]  Master .... On

[ ]  Pompe Manu .... Verrouillée

[ ]  Instru. Mot. .... Vert

[ ]  Conserv. Cap. .... Réglé

[ ]  Altimètre .... Réglé

[ ]  Message Radio

Décollage — Montée normale

[ ]  Rotation .... 65 Kts

[ ]  Montée vite. .... Mini 80 Kts

[ ]  Avant .... 300 FT

[ ]  Volets .... 0"

[ ]  P. Adm .... 25"

[ ]  Régime .... 2500 tr / mn

[ ]  Landing Light .... Off

[ ]  500 FT .... 1° virage

[ ]  Palier .... 1500 FT.

Croisière

[ ]  P. Adm .... 21"

[ ]  Régime .... 2300 t/mn.

[ ]  Richesse .... 8 à 9 psi

[ ]  Capot .... Demi

[ ]  Vitesse indic .... 110 kts

Descente du palier

[ ]  Capot .... Ferme

[ ]  Plein riche

[ ]  Régime .... 2300 t/mn

[ ]  P. Adm. .... 15"

Montée du palier

[ ]  Capot .... Ouvert

[ ]  Plein riche

[ ]  Régime .... 2500 t/mn.

[ ]  P. Adm. .... 25"

Approche intern. (prépa. Machine) vent. Arr.

[ ]  Conserv Cap .... Règle

[ ]  Altimètre .... Règle

[ ]  Linding Light .... On

[ ]  Réchauffe carbu .... Plein Chaud

[ ]  Gaz .... 18"

[ ]  Hélice .... P.P.P.

[ ]  Richesse .... Plein Riche

[ ]  Volet-De 130 Kts .... 10°

[ ]  Capot .... Ouvert

[ ]  Vitesse .... 80 Kts

Approche – finale

[ ]  Volets .... 20"

[ ]  P.Adm. à la Demande (pour) V.I .... + 70 Kts

[ ]  Chek list finale

[ ]  Plein riche

[ ]  Plein petit pas

[ ]  P Ompes

[ ]  V Olets

[ ]  A Altimètre

Arrêt Moteur

[ ]  Frein de parking .... Serre

[ ]  Avionic master / Radio .... Off

[ ]  Tous constact elect. .... Off

[ ]  Volets .... 0°

[ ]  Mixture .... Tirée

[ ]  Magnet. (clé retirée) .... OFF

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172F Skyhawk/Frederic_KIEN_CESSNA_FR172J_CHECKLIST.txt

Rights: licensed

Checklist hash: 3358759344669e2a7437da5184364205480dc04aa6e880d20f5ae6057af38ac0

Verify against the aircraft's own approved documentation before use.