Cessna 172L Skyhawk — c172l_abbreviated_checklist_millstone_valley

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

BEFORE STARTING ENGINE

[ ]  Exterior Preflight .... COMPLETE

[ ]  Tach/Hobbs Meter .... RECORD

[ ]  Seats, Belts, Harnesses .... ADJUST & LOCKED

[ ]  Fuel Shutoff Valve .... ON BOTH

[ ]  Radios & Electrical Equipment .... OFF

[ ]  Brakes .... TEST & SET

STARTING ENGINE

[ ]  Mixture .... RICH

[ ]  Carburetor Heat .... COLD

[ ]  Master Switch .... ON

[ ]  Prime .... AS REQUIRED

[ ]  Throttle .... OPEN ¼ INCH

[ ]  Propeller Area .... CLEAR

[ ]  Ignition Switch .... START (release when engine starts)

[ ]  Oil Pressure .... CHECK

BEFORE TAKEOFF

[ ]  Cabin Doors .... LATCHED

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Elevator Trim .... TAKEOFF POSITION

[ ]  Fuel Shutoff Valve .... ON BOTH

[ ]  Brakes .... SET

[ ]  Throttle .... 1700 RPM

[ ]  Magnetos .... CHECK — 125 MAX DROP / 50 MAX DIFF

[ ]  Carburetor Heat .... CHECK FOR RPM DROP

[ ]  Engine Instruments and Ammeter .... CHECK

[ ]  Suction Gauge .... CHECK

[ ]  Flight Instruments and Radio .... SET

[ ]  Throttle Friction Lock .... ADJUST

[ ]  Wing Flaps .... AS REQUIRED

TAKE-OFF

NORMAL TAKEOFF

[ ]  Wing Flaps .... UP (0)

[ ]  Carburetor Heat .... COLD

[ ]  Throttle .... FULL OPEN

[ ]  Elevator Control .... LIFT NOSE WHEEL AT 65 MPH

[ ]  Climb Speed .... 80 - 90 MPH

MAXIMUM PERFORMANCE TAKEOFF

[ ]  Wing Flaps .... 10 DEGREES

[ ]  Carburetor Heat .... COLD

[ ]  Brakes .... HOLD

[ ]  Throttle .... FULL OPEN

[ ]  Brakes .... RELEASE

[ ]  Elevator Control .... SLIGHTLY TAIL LOW

[ ]  Climb Speed .... 68 MPH (with obstacles ahead)

ENROUTE CLIMB

[ ]  Airspeed .... 80 – 100 MPH

[ ]  Throttle .... FULL OPEN

[ ]  Mixture .... RICH (Unless engine is rough)

CRUISE

[ ]  Power .... 2200 to 2500 (No more than 75%)

[ ]  Elevator Trim .... ADJUST

[ ]  Mixture .... RICH BELOW 3000

BEFORE LANDING

[ ]  Mixture .... RICH

[ ]  Carburetor Heat .... ON (apply before closing throttle)

[ ]  Airspeed .... 70-80 MPH (flaps up)

[ ]  Wing Flaps .... AS DESIRED (below 100 MPH)

[ ]  Airspeed .... 60-70 MPH (flaps down)

BALKED LANDING  [ABNORMAL]

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... COLD

[ ]  Wing Flaps .... RETRACT TO 20

[ ]  Airspeed .... 84 MPH

[ ]  Wing Flaps .... RETRACT (slowly)

NORMAL LANDING

[ ]  Touchdown .... MAIN WHEELS FIRST

[ ]  Landing Roll .... LOWER NOSE WHEEL GENTLY

[ ]  Braking .... MINIMUM REQUIRED

AFTER LANDING

[ ]  Wing Flaps .... UP

[ ]  Carburetor Heat .... COLD

[ ]  Unnecessary Equipment .... OFF

SECURING AIRCRAFT

[ ]  Parking Brake .... SET (Normally Not Used)

[ ]  Radios, Electrical Equipment .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF

[ ]  Control Lock .... INSTALL

[ ]  Tach/Hobbs Meter .... RECORD

C172L Airspeeds (MPH Indicated)

[ ]  Vr .... 65

[ ]  Vx .... 68

[ ]  Vy .... 84

[ ]  Va .... 111

[ ]  Vs .... 54

[ ]  Vso .... 47

[ ]  Vfe .... 97

[ ]  Vno .... 147

[ ]  Vne .... 184

[ ]  Best Glide .... 75

[ ]  Demonstrated X-Wind .... 15 Knots

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172L Skyhawk/c172l_abbreviated_checklist_millstone_valley.txt

Rights: licensed

Checklist hash: 058d0bb60d02960129e8ce35fc0a8d6548c5a23392a31403130c02c0bbfc4788

Verify against the aircraft's own approved documentation before use.