Cessna 172N _ 172M Skyhawk — c172n

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Light Signals" item count differs between passes (12 vs 6); verify nothing was dropped.

content_gap: Section "V-Speeds (KIAS)" item count differs between passes (16 vs 15); verify nothing was dropped.

Preflight Inspection

COCKPIT

[ ]  Aircraft docs (ARROW) .... Check

[ ]  Weight & Balance .... Check

[ ]  Parking Brake .... Set

[ ]  Control wheel lock .... Remove

[ ]  Hobbs/Tach .... Check/Remove

[ ]  Ignition .... Off

[ ]  Avionics Power Switch .... Off

[ ]  Master Switch .... On

[ ]  Fuel quantity indicators .... Check

[ ]  Pitot Heat .... On

[ ]  Avionics Master Switch .... On

[ ]  Avionics Cooling Fan .... Audible

[ ]  Avionics Master Switch .... Off

[ ]  Static Pressure Alt Src Valve .... Off

[ ]  Annunciator Panel Switch .... Test

[ ]  Annunciator's Illuminate .... Check

[ ]  Annunciator Panel Switch .... Off

[ ]  Flaps .... Extend

[ ]  Pitot Heat .... Off

[ ]  Master Switch .... Off

[ ]  Pitot Tube .... Test for Heat

[ ]  Fuel shutoff valve .... On (In)

FUSELAGE AND EMPENAGE

[ ]  Baggage Door .... Closed & Locked

[ ]  Rivets .... Check

[ ]  Rudder Gust Lock .... Remove

[ ]  Tail Tie-Down .... Disconnect

[ ]  Control surfaces .... Free & Secure

[ ]  Trim Tab .... Check Security

[ ]  Antennas .... Check Security

RIGHT WING

[ ]  Wing tie-down .... Disconnect

[ ]  Aileron .... Free and Secure

[ ]  Flaps .... Secure

[ ]  Main wheel tire .... Inflated/Cond

[ ]  Brakes .... Not Leaking

[ ]  Fuel tank sump .... Sample

[ ]  Fuel Quantity .... Check

[ ]  Fuel Filler cap .... Secure

NOSE

[ ]  Engine oil level .... Check

[ ]  Fuel strainer .... Sample

[ ]  Propeller and spinner .... Check

[ ]  Alternator belt .... Check

[ ]  Air intake .... Check

[ ]  Carburetor air filter .... Check

[ ]  Landing lights .... Check

[ ]  Nose wheel strut & tire .... Check

[ ]  Nose-Tie down .... Disconnect

[ ]  Static source opening .... Check

LEFT WING

[ ]  Wing tie-down .... Disconnect

[ ]  Aileron .... Free and Secure

[ ]  Flaps .... Secure

[ ]  Main wheel tire .... Inflated/Cond

[ ]  Brakes .... Not Leaking

[ ]  Fuel tank vent open .... Check

[ ]  Fuel tank sump .... Sample

[ ]  Fuel Quantity .... Check

[ ]  Fuel Filler cap .... Secure

[ ]  Pitot tube .... Uncover and Check

[ ]  Stall warning .... Check

[ ]  Landing/Taxi Light(s) .... Clean/Cond

Before starting engine

[ ]  Preflight inspection .... Complete

[ ]  Passenger Briefing .... Complete

[ ]  Seats, belts .... Adjust & Lock

[ ]  Doors .... Closed & Locked

[ ]  Brakes .... Test & Set

[ ]  Circuit breakers .... Check In

[ ]  Electrical Equip/Autopilot .... Off

[ ]  Avionics Power Switch .... Off

[ ]  Fuel Selector Valve .... Both

[ ]  Fuel shutoff valve .... On (In)

Starting Engine

[ ]  Throttle .... Open 1/4 inch

[ ]  Mixture .... Rich (IN)

[ ]  Carb heat .... Cold (IN)

[ ]  Prime .... As required; locked

[ ]  Aux Fuel Pump .... On

[ ]  Propeller area .... Clear

[ ]  Master Switch .... On

[ ]  Beacon .... On

[ ]  Ignition .... Start

[ ]  Throttle .... Adjust 1000 rpm

[ ]  Oil Pressure .... Check normal

[ ]  Aux Fuel Pump .... Off

[ ]  Avionics Master Switch .... On

[ ]  Radios .... On

[ ]  Transponder .... Standby

[ ]  Flaps .... Up

[ ]  Ammeter .... Check

[ ]  Heading Indicator .... Set

[ ]  ATIS/AWOS/ASOS .... Obtain

[ ]  Altimeter .... Set

[ ]  Autopilot .... Engage

[ ]  Flight Controls .... Move Against AP

[ ]  Autopilot .... Disconnect (Sound)

[ ]  Departure &Taxi Clrnce .... Contact

Before Takeoff

[ ]  Parking brake .... Set

[ ]  Cabin doors .... Closed & Locked

[ ]  Seats, belts .... Adjust & Lock

[ ]  Flight controls .... Free & Correct

[ ]  Instruments (4) .... Set

[ ]  Fuel Quantity .... Check

[ ]  Fuel Shutoff Valve .... On

[ ]  Mixture .... Rich (IN)

[ ]  Fuel Selector Valve .... Both

[ ]  Elevator Trim .... Set for TAKEOFF

[ ]  Throttle .... 1800 rpm

[ ]  Magnetos .... Check

[ ]  Suction gage .... Check

[ ]  Engine Instruments .... Check

[ ]  Ammeter .... Check

[ ]  Mixture .... Set for Density Alt

[ ]  Carb heat .... On

[ ]  Annunciator Panel .... Clear

[ ]  Throttle .... 1000 rpm

[ ]  Throttle Friction Lock .... Adjust

[ ]  Strobe Lights .... On

[ ]  Radios/Avionics .... Set

[ ]  Autopilot .... Off

[ ]  Flaps .... Set for Takeoff (00-100)

[ ]  Parking Brake .... Release

[ ]  Windows .... Closed

Takeoff

[ ]  Flaps .... Up

[ ]  Carb heat .... Cold (In)

[ ]  Transponder .... Altitude

[ ]  Trim .... set for TAKEOFF

[ ]  Throttle .... Full

[ ]  Tach, oil, airspeed .... Check

[ ]  Elevator .... Lift at 55 KIAS

[ ]  Climb .... 70-80 KIAS

Cruise

[ ]  Pitch .... Set

[ ]  Throttle .... As required

[ ]  Trim .... Set

[ ]  Mixture .... Adjust

Pre-landing checklist

[ ]  Fuel selector .... On

[ ]  Mixture .... Rich

[ ]  Carb Heat .... On

[ ]  Seatbelts .... Fastened

Approach

[ ]  Flight instruments .... Ckd & Set

[ ]  Radios .... Checked

[ ]  ATIS .... Checked

[ ]  Carb Heat .... On (Out)

[ ]  Mixture .... Rich

[ ]  Landing light .... On

[ ]  Airspeed .... 65-75 KIAS (Flaps Up)

[ ]  Airspeed .... 60-70 KIAS (Flaps Dn)

After landing

[ ]  Flaps .... Up

[ ]  Carb Heat .... Cold (In)

[ ]  Transponder .... Standby

[ ]  Landing light .... Off

Parking

[ ]  Avionics .... Off

[ ]  Electrical .... Off

[ ]  Throttle .... 1000 RPM

[ ]  Mixture .... Cut-off

[ ]  Ignition switch .... Off

[ ]  Master switch .... Off

Securing the aircraft

[ ]  Control Lock .... Install

[ ]  Hobbs/Tach .... Record

[ ]  Door/Window .... Secure

[ ]  Tie-downs .... Secure

Short field take-off

[ ]  Take-off checklist .... Complete

[ ]  Taxi .... Max runway

[ ]  Brakes .... Set and hold

[ ]  Flaps .... 10

[ ]  Throttle .... Full

[ ]  Brakes .... Release

[ ]  Climb .... 57 KIAS

[ ]  Flaps .... Retract when clear

[ ]  Airspeed .... 67 KIAS

Short field landing

[ ]  Pre-landing check .... Complete

[ ]  Approach .... 62 KIAS

[ ]  Flaps .... 30

[ ]  Throttle .... Maintain glide

[ ]  Touchdown .... Power Off

[ ]  Flaps .... Up

[ ]  Elevator .... Full up

[ ]  Braking .... Heavy as required

Soft field take-off

[ ]  Take-off checklist .... Complete

[ ]  Flaps .... 10

[ ]  Taxi .... keep rolling

[ ]  Climb .... 54 KIAS

[ ]  Flaps .... retract

[ ]  Airspeed .... 67 KIAS

Soft field landing

[ ]  Pre-landing check .... Complete

[ ]  Throttle .... 1500 RPM

[ ]  Flaps .... 10

[ ]  Airspeed .... 60 KIAS

[ ]  Touchdown .... Main first, softly

[ ]  Landing roll .... Nose wheel up

[ ]  Elevator .... Up

[ ]  Braking .... As required

Go-around

[ ]  Throttle .... Full

[ ]  Carb Heat .... Cold (In)

[ ]  Flaps .... 20

[ ]  Climb .... 55 KIAS

[ ]  Flaps .... 10

[ ]  Climb .... 60 KIAS

[ ]  Flaps .... Up

[ ]  Climb .... >60 KIAS

Engine failure - TAKEOFF  [EMERGENCY]

[ ]  Throttle [MEMORY] .... Idle

[ ]  Brakes [MEMORY] .... Apply

[ ]  Flaps [MEMORY] .... Retract

[ ]  Mixture [MEMORY] .... IDLE cut-off

[ ]  Ignition [MEMORY] .... Off

[ ]  Master switch [MEMORY] .... Off

Engine failure - AFTER TAKEOFF  [EMERGENCY]

[ ]  Airspeed .... 65 KIAS (flaps UP)

[ ]  Airspeed .... 60 KIAS (flaps DN)

[ ]  Mixture .... Idle Cut-off

[ ]  Fuel shutoff valve .... Off (Out)

[ ]  Ignition .... Off

[ ]  Flaps .... As Required

[ ]  Master switch .... Off

[ ]  Cabin Doors .... Unlatch

[ ]  Land .... Straight Ahead

Engine failure - DURING FLIGHT  [EMERGENCY]

[ ]  Airspeed .... 65 KIAS

LOOK FOR A FIELD

[ ]  Fuel shutoff valve .... On (In)

[ ]  Fuel selector valve .... Both

[ ]  Aux Fuel Pump Switch .... On

[ ]  Primer .... In & Locked

[ ]  Mixture .... Rich

[ ]  Ignition .... BOTH (or START)

[ ]  Airspeed .... 65 KIAS (flaps UP)

[ ]  Airspeed .... 60 KIAS (flaps DN)

[ ]  Mixture .... Idle Cut-off

[ ]  Fuel shutoff valve .... Off (Out)

[ ]  Ignition .... Off

[ ]  Flaps .... As Required

[ ]  Mayday .... Transmit 121.5

[ ]  Mayday .... Squawk 7700

[ ]  Master switch .... Off

[ ]  Cabin Doors .... Unlatch

[ ]  Touchdown .... Tail Low

Carburetor Icing  [EMERGENCY]

[ ]  Throttle .... Full

[ ]  Carb Heat .... On (Out)

[ ]  Mixture .... Adjust

Engine Roughness  [ABNORMAL]

[ ]  Magnetos .... Check

[ ]  Mixture .... Lean as necessary

Engine Fire during start  [EMERGENCY]

[ ]  Crank .... Continue

[ ]  Power .... 1700 RPM (2 min)

[ ]  Engine .... Shut down and inspect

Engine Fire during flight  [EMERGENCY]

[ ]  Mixture .... Idle Cut-off

[ ]  Fuel shutoff valve .... Off (Out)

[ ]  Master Switch .... Off

[ ]  Boost Pump .... Off

[ ]  Cabin Heat/Air .... Off

[ ]  Airspeed .... 100 KIAS

Electrical failure  [EMERGENCY]

[ ]  Load meter .... Verify

[ ]  Alternator .... Off

REDUCE LOAD TO MINIMUM

[ ]  Breaker/alt .... Check & Rest

[ ]  Alternator .... On

IF STILL NO POWER

[ ]  Alternator .... Off

REDUCE LOAD AND LAND

Electrical overload  [EMERGENCY]

[ ]  Master Switch .... Off

[ ]  Master Switch .... On

[ ]  Over-voltage light .... Off

[ ]  or TERMINATE flight ASAP

Spin Recovery  [EMERGENCY]

[ ]  Ailerons [MEMORY] .... NEUTRAL

[ ]  Throttle [MEMORY] .... IDLE

[ ]  Rudder [MEMORY] .... Full opposite

[ ]  Control wheel [MEMORY] .... Full forward

[ ]  Rudder control/wheel .... Neutral

[ ]  Pitch .... Level

Light Signals

NOTE: On Ground: Steady Green - Takeoff

NOTE: On Ground: Flashing Green - Taxi

NOTE: On Ground: Steady Red - Stop

NOTE: On Ground: Flashing Red - Clear runway

NOTE: On Ground: Flashing white - Return to ramp

NOTE: On Ground: Red/Green alternating - WARNING — USE CAUTION

NOTE: In Flight: Steady Green - Land

NOTE: In Flight: Flashing Green - Return to land

NOTE: In Flight: Steady Red - Give way

NOTE: In Flight: Flashing Red - Do not land

NOTE: In Flight: Flashing white - --

NOTE: In Flight: Red/Green alternating - WARNING — USE CAUTION

Comm Freq

[ ]  ATIS

[ ]  Ground

[ ]  Tower

[ ]  Club

[ ]  Fuel

Phonetic Alphabet

NOTE: Alpha

NOTE: Bravo

NOTE: Charlie

NOTE: Delta

NOTE: Echo

NOTE: Foxtrot

NOTE: Golf

NOTE: Hotel

NOTE: India

NOTE: Juliet

NOTE: Kilo

NOTE: Lima

NOTE: Mike

NOTE: November

NOTE: Oscar

NOTE: Papa

NOTE: Quebec

NOTE: Romeo

NOTE: Sierra

NOTE: Tango

NOTE: Uniform

NOTE: Victor

NOTE: Whiskey

NOTE: Xray

NOTE: Yankee

NOTE: Zulu

V-Speeds (KIAS)

[ ]  Rotate .... Vr 55

[ ]  Normal Climb .... Vclm 70-85

[ ]  Out b

[ ]  Max angle (Sea Level) .... Vx 60

[ ]  Max angle (10,000 ft) .... Vx 65

[ ]  Climb rate (Sea Level) .... Vy 79

[ ]  Climb rate (10,000 ft) .... Vy 71

[ ]  Maneuver .... Va 82-99

[ ]  Flaps .... Vfe 85

[ ]  Normal max .... Vno 127

[ ]  Never exceed .... Vne 158

[ ]  Stall (clean) .... Vs 44

[ ]  Stall (land) .... Vso 33

[ ]  Final Approach (Flaps) .... 60-70

[ ]  Final Approach (No flaps) .... 65-75

[ ]  Max Glide .... 60

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172N _ 172M Skyhawk/c172n.txt

Rights: licensed

Checklist hash: bda6a6ccf00796d94b6c2e7e7ed4df082e6798c019dcaaf08a370d3f9c29d359

Verify against the aircraft's own approved documentation before use.