Cessna 172N _ 172M Skyhawk — jochman-172chk

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

BEFORE START

[ ]  Preflight .... COMPLETE

[ ]  Seats/Belts .... ADJUST

[ ]  Fuel Valve .... Left/Right/Both

[ ]  Lights .... OFF

[ ]  Radios,Intercom .... OFF

[ ]  Transponder .... OFF or SBY

[ ]  Circuit Breakers .... CHECK

[ ]  Clock .... Wind and SET

[ ]  Altimeter .... SET

[ ]  Controls .... FREE & CORRECT

[ ]  Flaps .... CHECK

[ ]  Trim .... SET FOR TO

[ ]  Brakes .... CHECK/SET

ENGINE START

NORMAL (COLD) START

[ ]  Master .... ON

[ ]  Beacon .... ON

[ ]  Mixture .... FULL RICH

[ ]  Carb Heat .... OFF

[ ]  Throttle Open 1/2"

[ ]  Prime .... AS REQUIRED

[ ]  "CLEAR" Area .... LOOK ABOUT

[ ]  Ignition Switch .... START

[ ]  Adj Throttle .... 1000 RPM

[ ]  Oil Pressure .... CHECK

[ ]  Radios .... ON, CHECK 121.5

[ ]  Intercom .... ON

[ ]  Brakes .... OFF

[ ]  Toe Brakes .... CHECK

RUN UP

[ ]  Brakes .... Set

[ ]  Fuel Valve .... Both

[ ]  Fuel Quantity/Gauges .... CHECK

[ ]  Mixture .... Rich or As Required

[ ]  Throttle .... 1700 RPM

[ ]  Left/Right Mag Check .... CHECK

[ ]  Carb. Heat Check .... CHECK

[ ]  Vacuum Gauge Check .... CHECK

[ ]  Alternator Check .... CHECK

[ ]  Oil Pressure Check .... CHECK

[ ]  Oil Temperature Check .... CHECK

[ ]  Nav/Com Radios Set .... Check

[ ]  Transponder .... 1200

BEFORE TAKEOFF

[ ]  Flight Instruments .... CHECK

[ ]  Fuel .... Both

[ ]  Engine Gauges .... CHECK

[ ]  Carb. Heat .... OFF

[ ]  Mixture .... RICH

[ ]  Flaps .... SET

[ ]  Trim Tab .... SET

[ ]  Controls .... FREE/CORRECT

[ ]  Seat Belts/Harness .... CHECK

[ ]  Doors .... LATCHED

[ ]  Lights .... AS REQUIRED

[ ]  Strobes .... ON

[ ]  Transponder .... ON ALT

TAKEOFF

[ ]  NORMAL Liftoff 55, Climb 70-80 .... CHECK

[ ]  SHORT FIELD Liftoff 51, Climb 56 .... CHECK

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172N _ 172M Skyhawk/jochman-172chk.txt

Rights: licensed

Checklist hash: 6353f12e69ed848fc43c23874dc164dc927853c86c79c9a504d417bb6e9a158b

Verify against the aircraft's own approved documentation before use.