Cessna 172N _ 172M Skyhawk — jochman-clc172

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

ENGINE FAILURE DURING TAKEOFF  [EMERGENCY]

[ ]  Throttle .... IDLE

[ ]  Brakes .... APPLY

[ ]  Wing Flaps .... RETRACT

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Ignition Switch .... OFF

[ ]  Master Switch .... OFF

ENGINE FAILURE AFTER TAKEOFF  [EMERGENCY]

[ ]  Airspeed (flaps UP) .... 65 KIAS

[ ]  Airspeed (flaps DOWN) .... 60 KIAS

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Fuel Selector Valve .... OFF

[ ]  Ignition Switch .... OFF

[ ]  Wing Flaps .... AS REQUIRED

[ ]  Master Switch .... OFF

ENGINE FAILURE DURING FLIGHT  [EMERGENCY]

[ ]  Airspeed .... 65 KIAS

[ ]  Carburetor Heat .... ON

[ ]  Fuel Selector Valve .... BOTH

[ ]  Mixture .... RICH

[ ]  Ignition Switch .... BOTH or START

[ ]  Primer .... IN & LOCKED

EMERGENCY LANDING  [EMERGENCY]

[ ]  Airspeed (flaps UP) .... 65 KIAS

[ ]  Airspeed (flaps DOWN) .... 60 KIAS

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Fuel Selector Valve .... OFF

[ ]  Ignition Switch .... OFF

[ ]  Wing Flaps .... AS REQUIRED

[ ]  Master Switch .... OFF

[ ]  Doors .... UNLATCH PRIOR TO TOUCHDOWN

[ ]  Touchdown .... SLIGHTLY TAIL LOW

[ ]  Brakes .... APPLY HEAVILY

PRECAUTIONARY LANDING  [EMERGENCY]

[ ]  Wing Flaps .... 20(

[ ]  Airspeed .... 60 KIAS

[ ]  Select field .... SUITABLE

[ ]  Avionics Power Switch .... OFF

[ ]  Electrical Switches .... OFF

[ ]  Wing Flaps (on final approach) .... 30(

[ ]  Airspeed .... 60 KIAS

[ ]  Master Switch .... OFF

[ ]  Doors .... UNLATCH PRIOR TO TOUCHDOWN

[ ]  Touchdown .... SLIGHTLY TAIL LOW

[ ]  Ignition Switch .... OFF

[ ]  Brakes .... APPLY HEAVILY

CABIN

[ ]  Pilot’s Operating Handbook .... AVAILABLE

[ ]  Aircraft Documents .... AVAILABLE

[ ]  Control Wheel Lock .... REMOVE

[ ]  Ignition Switch .... OFF

[ ]  Avionics Power Switch .... OFF

[ ]  Master Switch .... ON

[ ]  Fuel Quanity Indicators .... CHECK QUANTITY

[ ]  Flaps .... DOWN

[ ]  Master Switch .... OFF

EMPENNAGE

[ ]  Rudder Gust Lock .... REMOVE

[ ]  Tail Tie-Down .... DISCONNECT

[ ]  Control Surfaces .... CHECK FREE

RIGHT WING

[ ]  Aileron .... CHECK FREE

[ ]  Wing Tie-Down .... DISCONNECT

[ ]  Main Wheel Tire .... CHECK

[ ]  Fuel Tank Sump .... SAMPLE

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Filler Cap .... SECURE

NOSE

[ ]  Engine Oil Level .... CHECK

[ ]  Fuel Strainer .... DRAIN

[ ]  Propeller and Spinner .... CHECK

[ ]  Landing Light .... CHECK

[ ]  Carburetor Air Filter .... CHECK

[ ]  Nose Wheel Strut and Tire .... CHECK

[ ]  Nose Tie-Down .... DISCONNECT

[ ]  Static Source Opening .... CHECK

LEFT WING

[ ]  Main Wheel Tire .... CHECK

[ ]  Fuel Tank Sump .... SAMPLE

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Filler Cap .... SECURE

[ ]  Pitot Tube Cover .... REMOVE

[ ]  Fuel Tank Vent Opening .... CHECK

[ ]  Stall Warning Opening .... CHECK

[ ]  Wing Tie-Down .... DISCONNECT

[ ]  Aileron .... CHECK FREE

BEFORE STARTING ENGINE

[ ]  Preflight Inspection .... COMPLETE

[ ]  Seats, Belts, Shoulder Harnesses .... ADJUST & LOCK

[ ]  Fuel Selector Valve .... BOTH

[ ]  Avionics Power Switch .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Brakes .... TEST & SET

[ ]  Circuit Breakers .... CHECK IN

STARTING ENGINE

[ ]  Mixture .... RICH

[ ]  Carburetor Heat .... COLD

[ ]  Master Switch .... ON

[ ]  Prime .... AS REQUIRED

[ ]  Throttle .... OPEN ¼ INCH

[ ]  Propeller Area .... CLEAR

[ ]  Ignition Switch .... START

[ ]  Throttle .... 1000 RPM

[ ]  Oil Pressure .... CHECK

[ ]  Flashing Beacon, Nav. Lights .... ON

[ ]  Avionics Power Switch .... ON

[ ]  Radios .... ON

[ ]  Transponder .... STANDBY

BEFORE TAXI

[ ]  Wing Flaps .... UP

[ ]  Parking Brake .... RELEASE

[ ]  Brakes .... TEST

[ ]  Area .... CLEAR

BEFORE TAKEOFF

[ ]  Parking Brake .... SET

[ ]  Cabin Doors and Windows .... CLOSED & LOCKED

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Flight Instruments .... SET

[ ]  Fuel Selector Valve .... BOTH

[ ]  Mixture (Below 3000’) .... RICH

[ ]  Elevator Trim .... TAKEOFF

[ ]  Rudder Trim (if installed) .... TAKEOFF

[ ]  Throttle .... 1700 RPM

[ ]  Magnetos .... CHECK

[ ]  Carburetor Heat .... CHECK

[ ]  Engine Instruments .... CHECK

[ ]  Ammeter .... CHECK

[ ]  Suction Guage .... CHECK

[ ]  Throttle .... 1000 RPM

[ ]  Radios .... SET

[ ]  Transponder .... ON-ALT

NORMAL TAKEOFF

[ ]  Wing Flaps .... 0( - 10(

[ ]  Carburetor Heat .... COLD

[ ]  Throttle .... FULL OPEN

[ ]  Elevator Control .... LIFT NOSE AT 55 KIAS

[ ]  Climb Speed .... BEST RATE

SHORT FIELD TAKEOFF

[ ]  Wing Flaps .... 10(

[ ]  Carburetor Heat .... COLD

[ ]  Brakes .... APPLY

[ ]  Throttle .... FULL OPEN

[ ]  Mixture .... LEAN above 3000’, RICH below 3000’

[ ]  Brakes .... RELEASE

[ ]  Elevator Control .... SLIGHTLY TAIL LOW

[ ]  Climb Speed (Until Obstacles Cleared) .... BEST ANGLE

ENROUTE CLIMB

[ ]  Airspeed .... BEST RATE or HIGHER

[ ]  Throttle .... FULL OPEN

[ ]  Mixture .... LEAN above 3000’, RICH below 3000’

CRUISE

[ ]  Power .... 75% or LESS

[ ]  Elevator Trim .... ADJUST

[ ]  Rudder Trim (if installed) .... ADJUST

[ ]  Mixture .... LEAN

DESCENT

[ ]  Fuel Selector Valve .... BOTH

[ ]  Mixture .... ADJUST

[ ]  Power .... AS DESIRED

[ ]  Carburetor Heat .... FULL AS REQUIRED

BEFORE LANDING

[ ]  Seats, Belts, Shoulder Harnesses .... SECURE

[ ]  Fuel Selector Valve .... BOTH

[ ]  Mixture .... RICH

[ ]  Carburetor Heat .... ON

BALKED LANDING

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... COLD

[ ]  Wing Flaps (immediately) .... 20(

[ ]  Climb Speed .... BEST ANGLE

[ ]  Wing Flaps .... 10(

[ ]  Wing Flaps (after safe alt. and 60 KIAS) .... RETRACT

AFTER LANDING

[ ]  Wing Flaps .... UP

[ ]  Carburetor Heat .... COLD

[ ]  Transponder .... STANDBY

SECURING AIRPLANE

[ ]  Parking Brake .... SET

[ ]  Avionics Power Switch .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Ignition Switch .... OFF

[ ]  Master Switch .... OFF

[ ]  Control Lock .... INSTALL

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172N _ 172M Skyhawk/jochman-clc172.txt

Rights: licensed

Checklist hash: 9f2009c3e47a825fe3e0d34d7ddc0fe806dd14a1b902be691f945bb2902e6ec0

Verify against the aircraft's own approved documentation before use.