Cessna 172N _ 172M Skyhawk — Sam_Dawson_C172N_InFlight_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Approach Brief (bold items VFR)" item count differs between passes (15 vs 14); verify nothing was dropped.

Final Before Take Off

[ ]  Heading .... Check w/ Runway

[ ]  Altimeter .... Field Elevation

[ ]  Transponder .... On

[ ]  Time .... Noted

[ ]  Exterior Lights .... On

Climb

[ ]  Fuel Selector .... Both

[ ]  Flaps .... Up

[ ]  Throttle .... Full

[ ]  Mixture .... Lean

[ ]  Airspeed .... 70-85

Cruise

[ ]  Fuel Selector .... As Required

[ ]  Elevator Trim .... Adjust

[ ]  Throttle .... 2100-2700 RPM

[ ]  Mixture .... Lean

[ ]  Landing Light .... Off

[ ]  Heading Indicator .... Align

In Range-Descent

[ ]  ATIS/AWOS/ASOS .... Received

[ ]  Altimeter .... Set

[ ]  Seats/Seat Belts .... Upright & Secured

[ ]  Fuel Selector .... Both

[ ]  Mixture .... Set

[ ]  Landing Light .... On

[ ]  Avionics .... Set

[ ]  Approach Briefing .... Complete

TURN-TIME-TWIST-THROTTLE-TALK

NOTE: Vx-65, Vy-72, Cruise Climb- 75-85, Va-85/95/105, Vref-70, Vref short- 65

Approach Brief (bold items VFR)

[ ]  -Approach .... Type, Runway, TPA

[ ]  -Frequency .... Set, ID

[ ]  Final Approach Course .... CDI Set

[ ]  FAF

[ ]  DH/MDA

[ ]  Approach Lights/PAPI/VASI .... activated?

[ ]  TDZE, HAT/HAA

[ ]  MAP

[ ]  Missed Approach

[ ]  Notes

[ ]  -Runway exit/taxi plan

[ ]  -Terrain, obstacles

[ ]  DG .... Set to MAG

[ ]  Min. Alt., MAP & Climb Out .... Complete

Before Landing

[ ]  Fuel Selector .... Both

[ ]  Mixture .... Set

[ ]  Carburetor Heat .... As Required

[ ]  Flaps .... As Desired

After Landing

[ ]  Trim .... Set for Takeoff

[ ]  Flaps .... Up

[ ]  Transponder .... Stby, 1200

[ ]  Exterior Lights .... As Required

[ ]  Carburetor Heat .... Cold

TURN-TIME-TWIST-THROTTLE-TALK

NOTE: Vx-65, Vy-72, Cruise Climb- 75-85, Va-85/95/105, Vref-70, Vref short- 65

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172N _ 172M Skyhawk/Sam_Dawson_C172N_InFlight_Checklist.txt

Rights: licensed

Checklist hash: cf64b7c88a1ba0bdd63c991f4a92b8e87aeffd992e00bd5c2c4dedf9d2980393

Verify against the aircraft's own approved documentation before use.