Cessna 172P Skyhawk — C172p-checklist_selkirk_college

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "ELECTRICAL FIRE IN FLIGHT" item count differs between passes (13 vs 12); verify nothing was dropped.

content_gap: Section "ELECTRICAL MALFUNCTION" item count differs between passes (5 vs 4); verify nothing was dropped.

content_gap: Section "ENGINE START" item count differs between passes (8 vs 7); verify nothing was dropped.

content_gap: Section "PRECAUTIONARY LANDING WITH POWER" item count differs between passes (18 vs 16); verify nothing was dropped.

PRE-START

[ ]  HOBBS .... NOTED

[ ]  PASSENGER BRIEFING .... COMPLETED (IF REQUIRED)

[ ]  DISPATCH LOG .... COMPLETED

[ ]  LOGS & DOCUMENTS .... ON BOARD & VALID

[ ]  SEATS & BELTS .... LOCKED & SECURE*

[ ]  CONTROLS .... FREE & CORRECT

[ ]  PARKING BRAKE .... SET

[ ]  CIRCUIT BREAKERS .... ALL IN

[ ]  ELECTRICS .... OFF

[ ]  RADIO MASTER .... OFF

[ ]  MASTER .... ON

[ ]  RADIO FAN .... ON

[ ]  BEACON .... ON

[ ]  LOW VOLTAGE LIGHT .... ON

[ ]  AMMETER .... CHECKED

[ ]  FUEL .... QUANTITY CHECKED, ON LEFT

[ ]  FLAPS .... UP

ENGINE START

[ ]  MIXTURE .... FULL RICH

[ ]  PRIMER .... PRIMED & LOCKED

[ ]  THROTTLE .... SET

[ ]  ENGINE (START) .... STARTED

[ ]  OIL PRESSURE .... NORMAL

[ ]  AMMETER .... CHECKED

[ ]  SUCTION .... CHECKED

NOTE: *BOTH PILOTS RESPOND

POST - START

[ ]  RADIO MASTER .... ON

[ ]  ELECTRICS .... ON (AS REQUIRED)

[ ]  ELT .... CHECKED

[ ]  INSTRUMENT PANEL .... PANEL CHECKED

[ ]  RADIOS .... RADIOS CHECKED

[ ]  FUEL SELECTOR .... QUANTITY CHECKED, ON RIGHT

TAXI

[ ]  BRAKES .... CHECKED

[ ]  INSTRUMENTS .... CHECKED

RUN - UP

[ ]  PARKING BRAKE .... SET

[ ]  THROTTLE .... 1000

[ ]  FUEL SELECTOR .... QUANTITY CHECKED, ON BOTH

[ ]  ENGINE GAUGES .... NORMAL

[ ]  THROTTLE .... 1700

[ ]  ENGINE GAUGES .... NORMAL

[ ]  SUCTION .... NORMAL

[ ]  MIXTURE .... CHECKED

[ ]  CARB HEAT .... CHECKED

[ ]  MAGNETOS .... CHECKED

[ ]  THROTTLE .... IDLE CHECKED, 1000, CARB HEAT COLD

PRE - TAKEOFF

[ ]  SEAT & BELTS .... LOCKED & SECURE*

[ ]  CONTROLS .... FREE

[ ]  TRIMS .... ____SET

[ ]  FUEL SELECTOR .... QUANTITY CHECKED, ON BOTH

[ ]  FLAPS .... SET FOR TAKEOFF

[ ]  ALTERNATE STATIC .... CHECKED & OFF

[ ]  MIXTURE .... FULL RICH

[ ]  THROTTLE TENSION .... SET

[ ]  CARB HEAT .... COLD

[ ]  CIRCUIT BREAKERS .... ALL IN

[ ]  PRIMER .... LOCKED

[ ]  ENGINE GAUGES .... NORMAL

[ ]  ALTIMETER .... ____SET

[ ]  CLOCK .... ______CHECKED

[ ]  HEADING INDICATOR .... CHECKED & SET

[ ]  IFR CLEARANCE .... CLEARANCE REVIEWED

[ ]  CREW BRIEFING .... BRIEFING COMPLETED

[ ]  DOORS & WINDOWS .... CLOSED & LOCKED*

[ ]  PITOT HEAT .... ON (IF REQUIRED)

[ ]  LANDING LIGHT .... ON (IF REQUIRED)

[ ]  STROBES .... ON (IF EQUIPPED)

[ ]  PULSE LIGHT .... ON

[ ]  TRANSPONDER .... ON ALT_______

[ ]  TIME .... NOTED

[ ]  HEADING INDICATOR .... CHECKED ON RUNWAY

POST - TAKEOFF

[ ]  FLAPS .... UP

[ ]  ENGINE GAUGES .... NORMAL

[ ]  LANDING LIGHT .... OFF

PRE - LANDING

[ ]  SEATS & BELTS .... LOCKED & SECURE*

[ ]  BRAKES .... CHECKED & OFF

[ ]  FUEL SELECTOR .... QUANTITY CHECKED, ON BOTH

[ ]  MIXTURE .... FULL RICH

[ ]  CARB HEAT .... HOT

[ ]  ELECTRICS .... ON (AS REQUIRED)

[ ]  MAGNETOS .... ON BOTH

[ ]  LANDING LIGHT .... ON (IF REQUIRED)

POST - LANDING

[ ]  FLAPS .... UP

[ ]  CARB HEAT .... COLD

[ ]  TRANSPONDER .... STANDBY

[ ]  MINIMUM ELECTRICS .... SET

SHUT - DOWN

[ ]  PARKING BRAKE .... SET

[ ]  THROTTLE .... 1000

[ ]  ELT .... CHECKED

[ ]  ELECTRICS .... OFF

[ ]  MASTER .... OFF

[ ]  THROTTLE .... IDLE

[ ]  LIVE MAG .... CHECKED

[ ]  MIXTURE .... IDLE CUT OFF

[ ]  MAGNETOS .... OFF

[ ]  FUEL SELECTOR .... ON LEFT

EMERGENCY PROCEDURES  [EMERGENCY]

NOTE: * ITEMS MUST BE PERFORMED BY MEMORY

ENGINE FAILURE DURING TAKEOFF RUN  [EMERGENCY]

[ ]  THROTTLE [MEMORY] .... IDLE

[ ]  BRAKES [MEMORY] .... APPLY

[ ]  WING FLAPS [MEMORY] .... RETRACT

[ ]  MIXTURE [MEMORY] .... IDLE CUT OFF

[ ]  IGNITION SWITCH [MEMORY] .... OFF

[ ]  MASTER SWITCH [MEMORY] .... OFF

ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF  [EMERGENCY]

[ ]  AIRSPEED [MEMORY] .... 65K (FLAPS UP) 60K (FLAPS DOWN)

[ ]  MIXTURE [MEMORY] .... IDLE CUT OFF

[ ]  FUEL SELECTOR VALVE [MEMORY] .... OFF

[ ]  IGNITION SWITCH [MEMORY] .... OFF

[ ]  WING FLAPS [MEMORY] .... AS REQUIRED

[ ]  MASTER SWITCH [MEMORY] .... OFF

ENGINE FAILURE IN FLIGHT  [EMERGENCY]

[ ]  AIRSPEED [MEMORY] .... 65KTS

[ ]  CARBURETOR HEAT [MEMORY] .... HOT

[ ]  FUEL SELECTOR [MEMORY] .... BOTH

[ ]  MIXTURE [MEMORY] .... RICH

[ ]  IGNITION SWITCH [MEMORY] .... BOTH (OR START)

[ ]  PRIMER [MEMORY] .... IN AND LOCKED

[ ]  GLIDE AIRSPEED .... 65K FLAPS UP 60K FLAPS DOWN

[ ]  RADIO .... MAYDAY CALLED

[ ]  PASSENGERS .... BRIEFED FOR ROUGH LANDING

[ ]  EVACUATION NOTICE .... ISSUED

[ ]  CABIN .... SECURED

[ ]  RESTART .... UNSUCCESSFUL

NOTE: IF RESTART IS UNSUCCESSFUL CONTINUE WITH CHECKLIST BELOW

[ ]  MIXTURE .... IDLE CUT OFF

[ ]  FUEL SELECTOR VALVE .... OFF

[ ]  IGNITION SWITCH .... OFF

[ ]  WING FLAPS .... AS REQUIRED

[ ]  MASTER SWITCH .... OFF

[ ]  DOORS .... UNLATCH BEFORE LANDING

[ ]  BRAKES .... APPLY HEAVILY

PRECAUTIONARY LANDING WITH POWER  [EMERGENCY]

NOTE: *ITEMS MUST BE PERFORMED BY MEMORY

LANDING OFF AERODROME

[ ]  WING FLAPS .... 20 DEGREES

[ ]  AIRSPEED .... 60K OR HIGHER IF REQUIRED

[ ]  RADIO CALL .... CONTACT FSS, ATC OR OTHER AIRCRAFT. REPORT POSITION, CONDITIONS AND INTENTIONS.

[ ]  PASSENGERS .... BRIEF FOR LANDING

[ ]  CABIN .... SECURE FOR LANDING

[ ]  SELECTED FIELD .... FLY OVER NOTING TERRAIN AND OBSTRUCTIONS

[ ]  OVERSHOOT .... RETRACT FLAPS UPON REACHING SAFE ALTITUDE AND AIRSPEED

[ ]  RADIO MASTER .... OFF

[ ]  ELECTRICS .... OFF

[ ]  FLAPS .... 30 ON FINAL

[ ]  AIRSPEED .... 60 KIAS

[ ]  MASTER SWITCH .... OFF

[ ]  DOORS .... UNLATCH PRIOR TO LANDING

[ ]  TOUCHDOWN .... SLIGHTLY TAIL LOW

[ ]  IGNITION SWITCH .... OFF

[ ]  BRAKES .... APPLY AS REQUIRED

ELECTRICAL FIRE IN FLIGHT  [EMERGENCY]

NOTE: * ITEMS MUST BE PERFORMED BY MEMORY

[ ]  MASTER [MEMORY] .... OFF

[ ]  RADIO MASTER [MEMORY] .... OFF

[ ]  ELECTRICAL SWITCHES [MEMORY] .... OFF

[ ]  VENTS/CABIN AIR/HEAT .... CLOSED

[ ]  FIRE EXTINGUISHER .... ACTIVATE IF REQUIRED

WARNING: AFTER DISCHARGING A FIRE EXTINGUISHER VENTILATE THE CABIN

NOTE: IF ELECTRIC POWER IS NOT REQUIRED - LEAVE MASTER OFF AND LAND AT A SUITABLE AERODROME.

[ ]  MASTER .... ON

[ ]  CIRCUIT BREAKERS .... CHECK-DO NOT RESET

[ ]  RADIO SWITCHES .... OFF

[ ]  RADIO MASTER .... ON

[ ]  RADIO AND ELECTRICAL SWITCHES .... ON ONE AT A TIME, WITH DELAY UNTIL SHORT IS LOCALIZED

CABIN FIRE  [EMERGENCY]

[ ]  MASTER [MEMORY] .... OFF

[ ]  VENTS/CABIN AIR/HEAT [MEMORY] .... CLOSED

[ ]  FIRE EXTINGUISHER .... ACTIVATE, IF REQUIRED

WARNING: AFTER DISCHARGING A FIRE EXTINGUISHER VENTILATE THE CABIN

[ ]  LAND ASAP .... INSPECT FOR DAMAGE

WING FIRE  [EMERGENCY]

[ ]  NAVIGATION LIGHTS .... OFF

[ ]  PITOT HEAT .... OFF

[ ]  STROBES .... OFF

NOTE: PERFORM A SIDESLIP TO KEEP FLAMES AWAY FROM THE FUEL TANK

[ ]  LAND .... AS SOON AS POSSIBLE

WARNING: USE FLAPS ONLY AS REQUIRED FOR LANDING

ELECTRICAL MALFUNCTION  [ABNORMAL]

NOTE: * ITEMS MUST BE PERFORMED BY MEMORY

[ ]  ALTERNATOR [MEMORY] .... OFF

[ ]  ALTERNATOR CIRCUIT BREAKER .... PULL

[ ]  NONESSENTIAL ELECTRIC EQUIPMENT .... OFF

[ ]  FLIGHT .... END AS SOON AS PRACTICAL

LOW VOLTAGE LIGHT ILLUMINATES DURING FLIGHT  [ABNORMAL]

[ ]  RADIO MASTER .... OFF

[ ]  ALTERNATOR CIRCUIT BREAKER .... CHECK IN

[ ]  MASTER .... OFF-BOTH SIDES

[ ]  MASTER .... ON-AFTER 3 SECONDS

[ ]  LOW VOLTAGE LIGHT .... CHECK OFF

[ ]  RADIO MASTER .... ON

NOTE: IF LOW-VOLTAGE LIGHT ILLUMINATES AGAIN

[ ]  ALTERNATOR .... OFF

[ ]  NONESSENTIAL ELECTRICS .... OFF

[ ]  FLIGHT .... END AS SOON AS PRACTICAL

FIRES  [EMERGENCY]

NOTE: *ITEMS MUST BE PERFORMED BY MEMORY

FIRE DURING START ON GROUND  [EMERGENCY]

[ ]  CRANKING [MEMORY] .... CONTINUE

IF ENGINE STARTS:

[ ]  POWER [MEMORY] .... 1700 RPM

[ ]  ENGINE .... SHUTDOWN AND INSPECT

IF ENGINE FAILS TO START:

[ ]  THROTTLE [MEMORY] .... FULL OPEN

[ ]  MIXTURE [MEMORY] .... IDLE CUT-OFF

[ ]  CRANKING [MEMORY] .... CONTINUE

[ ]  FIRE EXTINGUISHER .... OBTAIN

[ ]  MASTER .... OFF

[ ]  MAGNETOS .... OFF

[ ]  FUEL SELECTOR .... OFF

[ ]  PASSENGERS .... EVACUATE

[ ]  FIRE .... EXTINGUISH, IF SAFE

[ ]  FIRE DAMAGE .... INSPECT

ENGINE FIRE IN FLIGHT  [EMERGENCY]

[ ]  MIXTURE [MEMORY] .... IDLE CUT OFF

[ ]  FUEL SELECTOR VALVE [MEMORY] .... OFF

[ ]  MASTER [MEMORY] .... OFF

[ ]  CABIN HEAT AND AIR [MEMORY] .... OFF

[ ]  AIRSPEED [MEMORY] .... 100 KIAS SEE NOTE

NOTE: IF FIRE NOT EXTINGUISHED INCREASE GLIDE TO FIND AN INCOMBUSTIBLE MIXTURE

[ ]  FORCED LANDING [MEMORY] .... EXECUTE

NOTE: REFER TO ENGINE FAILURE IN FLIGHT ITEMS 7 - 19

DITCHING  [EMERGENCY]

[ ]  RADIO .... TRANSMIT MAYDAY

[ ]  TRANSPONDER .... SQUAWK 7700

[ ]  CABIN .... SECURE FOR ROUGH LANDING - JETTISON HEAVY OBJECTS

[ ]  PASSENGERS .... BRIEF FOR ROUGH LANDING

[ ]  PASSENGERS .... BRIEF ESCAPE PROCEDURE

NOTE: EVACUATE THROUGH CABIN DOORS- IF NECESSARY OPEN WINDOW AND FLOOD CABIN TO EQUALIZE PRESSURE SO DOORS CAN BE OPENED.

[ ]  APPROACH .... HIGH WINDS HEAVY SWELLS: INTO WIND; LIGHT WINDS HEAVY SWELLS: PARALLEL

[ ]  FLAPS .... 20 TO 30 DEGREES

NOTE: IF NO POWER IS AVAILABLE, APPROACH AT 65KIAS WITH FLAPS UP OR 60 KIAS WITH FLAPS 10 DEGREES

[ ]  CABIN DOOR .... UNLATCH

[ ]  TOUCHDOWN .... LEVEL ATTITUDE AT ESTABLISHED RATE OF DESCENT

[ ]  FACE .... CUSHION BEFORE TOUCHDOWN WITH FOLDED COAT

[ ]  AIRPLANE .... EVACUATE

[ ]  LIFE VESTS AND RAFT .... INFLATE

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172P Skyhawk/C172p-checklist_selkirk_college.txt

Rights: licensed

Checklist hash: dc2ec1e97f4e046632377b3d46fac0dbdcca6bd2a8c360354442aeafde497104

Verify against the aircraft's own approved documentation before use.