Cessna 172P Skyhawk — Sam_Dawson_C172XP_InFlight_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Approach Brief" item count differs between passes (15 vs 14); verify nothing was dropped.

Final Before Take Off

[ ]  Heading .... Check w/ Runway

[ ]  Altimeter .... Field Elevation

[ ]  Transponder .... On

[ ]  Time .... Noted

[ ]  Exterior Lights .... On

Climb

[ ]  Airspeed .... 85-95 KIAS

[ ]  Fuel Selector .... Both

[ ]  Flaps .... Up

[ ]  Throttle .... Full

[ ]  Mixture .... Lean

[ ]  Cowl Flaps .... OPEN as required

Cruise

[ ]  Fuel Selector .... As Required

[ ]  Throttle .... 15-25 “/2200-2600 RPM

[ ]  Elevator & Rudder Trim .... As Required

[ ]  Mixture .... Lean

[ ]  Landing Light .... Off

[ ]  Cowl Flap .... CLOSED

In Range-Descent

[ ]  ATIS/AWOS/ASOS .... Received

[ ]  Altimeter .... Set

[ ]  Seats/Seat Belts .... Upright & Secured

[ ]  Fuel Selector .... Both

[ ]  Mixture .... Set

[ ]  Landing Light .... On

[ ]  Avionics .... Set

[ ]  Approach Briefing .... Complete

Turn-Time-Twist-Throttle-Talk  [ABNORMAL]

Speed reference

NOTE: Vx-59, Vy-81, Cruise Climb- 85-95, Va-87/96/105

NOTE: 23 January 2007 ATP of El Paso, TX

Approach Brief

APPROACH BRIEF (BOXED ITEMS VFR)

[ ]  Approach .... Type, Runway, TPA

[ ]  Frequency GPS .... Set, ID

[ ]  Final Approach Course .... CDI Set

[ ]  FAF

[ ]  DH/MDA

[ ]  Approach Lights/PAPI/VASI activated?

[ ]  TDZE, HAT/HAA

[ ]  MAP

[ ]  Missed Approach

[ ]  Notes

[ ]  Runway exit/taxi plan

[ ]  Terrain, obstacles

[ ]  DG .... Set to MAG

[ ]  Min. Alt., MAP & Climb Out .... Complete

Before Landing

[ ]  Seats, Belts, Harnesses .... ADJUST & LOCK

[ ]  Fuel Selector .... Both

[ ]  Propeller .... HIGH RPM

[ ]  Mixture .... Set

[ ]  Cowl Flaps .... CLOSED

Balked Landing  [ABNORMAL]

[ ]  Power .... Full throttle/2600 RPM

[ ]  Wing Flaps .... Retract to 20°

[ ]  Airspeed .... 55 KIAS

[ ]  Wing Flaps .... Retract slowly after 65 KIAS

[ ]  Cowl Flaps .... OPEN

After Landing

[ ]  Trim .... Set for Takeoff

[ ]  Flaps .... Up

[ ]  Transponder .... Stby

[ ]  Exterior Lights .... As Required

[ ]  Carburetor Heat .... Cold

Turn-Time-Twist-Throttle-Talk

NOTE: 23 January 2007

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172P Skyhawk/Sam_Dawson_C172XP_InFlight_Checklist.txt

Rights: licensed

Checklist hash: 0867afd28854f03fa724e0748b73fd73e7aeb67d233a7d16323857c2501ead5c

Verify against the aircraft's own approved documentation before use.