Cessna 172P Skyhawk — TFA_Cessna_172P_Checklist_2004-01

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Before Takeoff" item count differs between passes (19 vs 21); verify nothing was dropped.

content_gap: Section "Cruise" item count differs between passes (8 vs 11); verify nothing was dropped.

content_gap: Section "Preflight Inspection" item count differs between passes (56 vs 58); verify nothing was dropped.

content_gap: Section "Short Field Take Off" item count differs between passes (11 vs 12); verify nothing was dropped.

V Speed Review

[ ]  Max Cross Wind .... 15 KIAS

[ ]  Vso .... 33 KIAS

[ ]  Vs1 .... 44 KIAS

[ ]  Vx .... 60 KIAS

[ ]  Best Glide .... 65 KIAS

[ ]  Vy .... 76 KIAS

[ ]  Va .... 82-99 KIAS

[ ]  Vfe 20° & + .... 85 KIAS

[ ]  Vfe 10° .... 110 KIAS

[ ]  Vno .... 127 KIAS

[ ]  Vne .... 158 KIAS

Preflight Inspection

CABIN

[ ]  A.R.O.W.& Mtc. Docs. .... CHECK

[ ]  Hobbs Meter .... CHECK

[ ]  Control Lock .... REMOVE

[ ]  Ignition Switch .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Master (look prop is clear) .... ON

[ ]  Avionics Power .... ON/FAN/OFF

[ ]  Fuel Quantity Indicators .... CHECK

[ ]  Lights & Pitot Heat .... CHECK

[ ]  Flaps .... 30°

[ ]  Master Switch .... OFF

[ ]  Alternate Static .... OFF

[ ]  Fuel Selector Valve .... BOTH

[ ]  Baggage Door .... CHECK

EMPENNAGE

[ ]  Rudder Gust Lock .... REMOVE

[ ]  Fuselage .... CHECK

[ ]  Tail Tie Down .... UNTIE

[ ]  Elevator .... FREE & SECURE

[ ]  Rudder .... FREE & SECURE

[ ]  VOR Antennas .... CHECK

[ ]  Trim Tab .... CHECK

RIGHT WING

[ ]  Flap .... SECURE

[ ]  Aileron .... FREE & SECURE

[ ]  Skin/Tip/Lights .... CHECK

[ ]  Wing Tie Down .... UNTIE

[ ]  Main Wheel Tire .... INFLATED

[ ]  Brakes .... CHECK

[ ]  Fuel Tank Sump .... SAMPLE

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Filler Cap .... CHECK SECURE

NOSE

[ ]  Engine Oil .... 5-7 Qts. Min/Max (N63022 6-8 Qts. Min/Max)

[ ]  Fuel Strainer .... SAMPLE

[ ]  Belly Quick Drain .... SAMPLE (N52950 No Belly Quick Drain Avail.)

[ ]  Propeller & Spinner .... CHECK

[ ]  Air Intakes .... FREE & OPEN

[ ]  Carburetor Air Filter .... CLEAR

[ ]  Landing Light .... SECURE

[ ]  Nose Wheel & Strut .... CHECK

[ ]  Static Port .... OPEN

LEFT WING

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Filler Cap .... CHECK SECURE

[ ]  Main Wheel Tire .... INFLATED

[ ]  Brakes .... CHECK

[ ]  Fuel Tank Sump .... SAMPLE

[ ]  Pitot Tube & Drain .... FREE

[ ]  Stall Warning Opening .... CHECK

[ ]  Fuel Tank Vent .... OPEN

[ ]  Wing Tie Down .... UNTIE

[ ]  Aileron .... FREE & SECURE

[ ]  Flap .... SECURE

[ ]  Skin/Tip/Lights .... CHECK

Before Starting Engine

[ ]  Preflight .... COMPLETE

[ ]  Passenger Brief .... COMPLETE

[ ]  Seat/Seat Belts .... LOCKED

[ ]  Fuel Selector Valve .... BOTH

[ ]  Avionics Power .... OFF

[ ]  Circuit Breakers .... IN

[ ]  Electrical Equipment .... OFF

[ ]  Brakes .... SET & HOLD

Starting Engine

[ ]  Mixture .... RICH

[ ]  Throttle .... OPEN ¼ INCH

[ ]  Carburetor Heat .... COLD

[ ]  Prime .... AS REQ

[ ]  Beacon .... ON

[ ]  Master Switch .... ON

[ ]  Propeller Area .... CLEAR

[ ]  Ignition Switch .... START

[ ]  Throttle .... SMOOTH IDLE

[ ]  Oil Pressure .... CHECK

[ ]  Carburetor Heat .... AS REQ

[ ]  Flaps .... UP & VERIFIED

[ ]  Pitot Heat .... AS REQ

[ ]  Avionics Master / Avionics .... ON

Before Takeoff

[ ]  Brakes .... SET & HOLD

[ ]  Doors & Windows .... LOCKED

[ ]  Controls .... FREE & CORRECT

[ ]  Trim .... SET FOR TAKEOFF

[ ]  Flight Instr. .... SET & CHECK

[ ]  Fuel Selector Valve .... BOTH

[ ]  Mixture .... RICH

[ ]  Throttle .... 1700 RPM

[ ]  Mags .... RT/BOTH/LEFT/BOTH; Max 125 Drop & 50 Diff.

[ ]  Carb. Heat .... ON/CHECK/OFF

[ ]  Engine Instr. .... GREEN/NORM

[ ]  Ammeter/Alternator .... CHECK

[ ]  Suction Gauge .... CHECK

[ ]  Throttle .... SMOOTH IDLE

[ ]  Carburetor Heat .... AS REQ (FOR WX & RNWY COND.)

[ ]  Comm./Nav. Radios .... SET

[ ]  Strobe Lights (if installed) .... ON

[ ]  Throttle Lock .... ADJUST

[ ]  Flaps .... SET FOR TAKEOFF

Taking Runway

NOTE: MEMORY ITEMS – CALCAT

[ ]  Clearance .... RECEIVED T.O. or P.& H.

[ ]  Approach End .... CLEAR

[ ]  Lights .... ON

[ ]  Camera .... TRANSPONDER ALT

[ ]  Action .... D.G. TO RNWY HDG

[ ]  Time .... OFF TAKE

Normal Takeoff

[ ]  Flaps .... 0° - 10°

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... OFF/COLD

[ ]  Airspeed .... ALIVE

[ ]  Eng. Instr. .... GREEN/NORM.

[ ]  Lift Nose Wheel .... 55 KIAS

[ ]  Climb Speed .... 70 – 80 KIAS

Soft Field Take Off

[ ]  Elevator Control .... FULL AFT

[ ]  Flaps .... 10°

[ ]  Taxi .... KEEP ROLLING

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... OFF/COLD

[ ]  Airspeed .... ALIVE

[ ]  Eng. Instr. .... GREEN/NORM.

[ ]  Lift Off .... AS SOON AS PRACTICAL

[ ]  Accelerate In Ground Effect .... TO Vx OR Vy

[ ]  Flaps (Over Obstacle & Vy+) .... UP

Short Field Take Off

[ ]  Flaps .... 10°

[ ]  Brakes .... HOLD

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... OFF/COLD

[ ]  Mixture .... RICH (above 3000’ Lean Max RPMs)

[ ]  Engine Instr. .... GREEN/NORM

[ ]  Brakes .... RELEASE

[ ]  Airspeed .... ALIVE

[ ]  Lift Nose .... APPROX. 55 KIAS

[ ]  Climb Speed .... 56 KIAS

[ ]  Over Obstacle .... ACCELERATE TO Vy & THEN FLAPS UP

Enroute Climb

[ ]  Airspeed .... 70 – 85 KIAS

[ ]  Throttle .... FULL OPEN

[ ]  Mixture .... RICH OR AS REQ.

Cruise

[ ]  Attitude .... ESTABLISH

[ ]  Power .... ADJUST 65% 2400 RPM (upon reaching desired airspeed)

[ ]  Trim .... ADJUST

[ ]  Carb Heat .... AS REQ (ON WHENEVER IN RAIN, SNOW, OR VISIBLE MOISTURE)

[ ]  Mixture .... LEAN AS REQ (50° Less Peak EGT If Installed)

[ ]  Lights .... AS REQ

[ ]  Flight Plan .... OPEN

[ ]  Co. Frequency .... MONITOR 129.25 WHENEVER ABLE

Descent

[ ]  Landing Light .... ON

[ ]  Mixture .... RICH

[ ]  Carburetor Heat .... ON/HOT

[ ]  Throttle .... AS REQ

Before Landing

NOTE: ALOUD ALL TRAINING FLIGHTS SOLO OR DUAL SEVERAL TIMES

[ ]  Carb. Heat .... VERIFIED ON

[ ]  Gas (Fuel Valve) .... BOTH

[ ]  Undercarriage (Fixed) .... DOWN

[ ]  Mixture .... FULL RICH

[ ]  Primer .... IN & LOCKED

[ ]  Seatbelts / Switches .... CHECK

Normal Landings

[ ]  Airspeed .... 65-75 KIAS

[ ]  Flaps .... AS REQ

[ ]  Airspeed (with flaps) .... 60-70 KIAS

[ ]  Throttle .... AS REQ

[ ]  Braking .... MIN REQ

Soft Field Landing

[ ]  Airspeed .... 65-75 KIAS

[ ]  Flaps .... 30°

[ ]  Short Final .... 61 KIAS

[ ]  Touchdown .... NOSE HIGH

[ ]  Throttle .... IF/OR AS REQ

[ ]  Nose Wheel .... HOLD OFF

[ ]  Elevator .... FULL AFT FOR TAXI

[ ]  Throttle .... AS REQ/RNWY COND.

Short Field Landing

[ ]  Airspeed .... 65 - 75 KIAS

[ ]  Flaps .... 30°

[ ]  Airspeed .... MAINTAIN 61 KIAS (Until Flare)

[ ]  Clearing Obstacle .... THROTTLE REDUCE TO IDLE AT TOUCHDOWN

[ ]  Flaps .... RETRACT

[ ]  Brakes .... APPLY HEAVILY

Go Around

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... COLD/OFF

[ ]  Flaps 20° .... IMMEDIATELY

[ ]  Airspeed .... INCREASING

[ ]  Flaps .... RETRACT SLOWLY

After Landing

[ ]  Flaps .... UP

[ ]  Carburetor Heat .... COLD

[ ]  Transponder .... OFF/1200

[ ]  Pitot Heat .... OFF

[ ]  Lights .... AS REQ

[ ]  Taxi Clearance .... OBTAIN

Securing Airplane

[ ]  Avionics/Avionics Master .... OFF

[ ]  Lights/Switches (Ex. Beacon) .... OFF

[ ]  Throttle .... MIN. 1000 RPM

[ ]  Mixture .... IDLE CUT OFF

[ ]  Ignition (verify prop stopped) .... OFF

[ ]  Master Switch .... OFF

[ ]  Control Lock .... INSTALL

[ ]  Hobbs .... RECORD

[ ]  Tie Down Aircraft .... SECURELY

[ ]  Post-flight Inspection .... DO

[ ]  Flight Plan .... CLOSE WITH FSS

Emergency Checklist — Fires  [EMERGENCY]

ENGINE FIRE DURING START

[ ]  Cranking .... CONTINUE

IF ENGINE STARTS

[ ]  Power .... 1700 RPM/1 MIN.

[ ]  Engine .... SHUTDOWN/INSPECT

ENGINE FAILS TO START

[ ]  Throttle .... FULL OPEN

[ ]  Mixture .... IDLE CUT OFF

[ ]  Cranking .... CONTINUE

[ ]  Fuel Shutoff Valve .... OFF

[ ]  Master Switch .... OFF

[ ]  Ignition Switch .... OFF

[ ]  Fire Extinguisher .... OBTAIN/EXT.

[ ]  Fire Damage .... INSPECT

ENGINE FIRE DURING FLIGHT

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Fuel Shutoff Valve .... OFF

[ ]  Master Switch .... OFF

[ ]  Cabin Heat & Air .... OFF (except overhead vents)

[ ]  Airspeed .... 100 KIAS

NOTE: If fire is not extinguished, increase airspeed to an incombustible mixture.

[ ]  Forced Landing .... EXECUTE

WING FIRE

[ ]  Nav/Strobe/Landing Lights .... OFF

[ ]  Pitot Heat .... OFF

[ ]  Perform .... SIDE SLIP AWAY FROM THE FUEL TANK AND CABIN

ELECTRICAL FIRE IN FLIGHT

[ ]  Master Switch .... OFF

[ ]  Switches (except Ignition) .... OFF

[ ]  Vents/Cabin Air/Heat .... CLOSED

[ ]  Extinguisher .... ACTIVATE

[ ]  Fire Out .... VENTILATE CABIN

NOTE: IF FIRE OUT & ELECTRIC IS NECESSARY FOR CONTINUED FLIGHT (i.e. I.F.R.& I.M.C.) THEN:

[ ]  Master .... ON

[ ]  Circuit Breakers .... CHK/NO RESET

[ ]  All Radio Switches .... OFF

[ ]  Avionics Power Switch .... ON

[ ]  Radio/Elect. Switches .... ON ONE AT A TIME, WITH DELAY UNTIL FAULT FOUND

Engine Failure Takeoff Run  [EMERGENCY]

[ ]  Throttle .... IDLE

[ ]  Brakes .... APPLY

[ ]  Flaps .... UP

[ ]  Mixture .... IDLE CUT OFF

[ ]  Ignition Switch .... OFF

[ ]  Master Switch .... OFF

Engine Failure After Takeoff  [EMERGENCY]

[ ]  Best Glide .... 60 KIAS

[ ]  Fuel Shutoff Valve .... OFF

[ ]  Mixture .... IDLE CUT OFF

[ ]  Ignition Switch .... OFF

[ ]  Flaps .... AS REQ

[ ]  Master Switch .... OFF

Engine Failure During Flight  [EMERGENCY]

[ ]  Airspeed Best Glide .... 65 KIAS

[ ]  Best Place To Land .... TURN TO

[ ]  Carburetor Heat .... ON

[ ]  Fuel Shutoff Valve .... BOTH

[ ]  Mixture .... RICH

[ ]  Ignition Switch .... CHECK BOTH/OR START IF PROP IS STOPPED

[ ]  Primer .... IN & LOCKED

[ ]  Trouble Shoot .... MAGS CYCLE THROTTLE PUMP

UNABLE TO RE-START:

[ ]  Declare May Day .... ATC or 121.5

[ ]  Squawk .... 7700

Forced Landing Without Engine Power  [EMERGENCY]

[ ]  Airspeed .... 65 KIAS FLAPS UP; 60 KIAS FLAP DOWN

[ ]  Fuel Shutoff Valve .... OFF

[ ]  Mixture .... IDLE CUT OFF

[ ]  Ignition Switch .... OFF

[ ]  Doors .... UNLATCH

[ ]  Flaps .... AS REQ

[ ]  Master Switch .... OFF LAST

[ ]  Touchdown .... TAIL LOW

[ ]  Brakes .... APPLY HEAVILY

Precautionary Landing With Engine Power  [EMERGENCY]

[ ]  Airspeed .... 60 KIAS

[ ]  Electrical Equipment .... OFF

[ ]  Flaps .... 20°

[ ]  Master Switch .... OFF

[ ]  Doors .... UNLATCH

[ ]  Touchdown .... TAIL LOW

[ ]  Ignition Switch .... OFF

[ ]  Brakes .... APPLY HEAVILY

Electrical Malfunction  [EMERGENCY]

OVER VOLTAGE CONDITION

[ ]  Alternator .... OFF

[ ]  Alternator Circuit Breaker .... PULL

[ ]  Nonessential electrical equip .... OFF

[ ]  Flight .... TERMINATE AS SOON AS PRACTICAL

LOW VOLTAGE CONDITION

[ ]  Avionics / Electrical .... OFF

[ ]  Alternator Circuit Breaker .... IN

[ ]  Master Switch .... OFF

[ ]  Master Switch .... ON

[ ]  Low Voltage Light .... CHECK OFF

[ ]  Avionics Power Switch .... ON

NOTE: If low voltage light illuminates again:

[ ]  Alternator .... OFF

[ ]  Nonessential elect. equip .... OFF

[ ]  Flight .... TERMINATE AS SOON AS PRACTICAL

Cold & Cool Weather Ops

NOTE: A preheated engine is required before starting during cool, cold, or winter operations. Ensure oil temp. is in the green or is moving before engine run-up and mag checks are attempted.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172P Skyhawk/TFA_Cessna_172P_Checklist_2004-01.txt

Rights: licensed

Checklist hash: 0c55c506747c70b4a83f0276d512999733ee9b80c081ba107bc5fc3048ef51e2

Verify against the aircraft's own approved documentation before use.