Cessna 172RG Skyhawk — Sam_Dawson_172RG_ground_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Shutdown and Securing" item count differs between passes (15 vs 11); verify nothing was dropped.

content_gap: Section "Taxi" item count differs between passes (3 vs 2); verify nothing was dropped.

Before Starting Engine

[ ]  Preflight Inspection .... Complete

[ ]  CRM/Passenger Briefing .... Complete

[ ]  Seats and Seat Belts .... Adjust & Lock

[ ]  Brakes .... Test & Set

[ ]  Fuel Selector .... BOTH

[ ]  Cowl Flaps .... OPEN

[ ]  Mixture .... RICH

[ ]  Propeller .... HIGH RPM

[ ]  Carburetor Heat .... COLD

[ ]  Landing Gear Lever .... DOWN

[ ]  Autopilot .... OFF

[ ]  Avionics Master Switch .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Circuit Breakers .... Check In

Starting Engine

[ ]  Propeller Area .... CLEAR

[ ]  Master Switch .... ON

[ ]  Exterior Lights .... As Required

[ ]  Throttle .... PUMP once or twice; leave open ¼”.

NOTE: If engine is hot turn aux fuel pump ON during start.

[ ]  Ignition Switch .... START

[ ]  Oil Pressure .... Check

[ ]  Starter .... CHECK DISENGAGED

Before Taxi

[ ]  Flaps .... Retract

[ ]  Mixture .... Lean

[ ]  Throttle .... 1000 RPM

[ ]  Avionics Master (Radios) .... On

[ ]  ATIS / AWOS .... Receive

[ ]  Clearance .... Obtain

[ ]  Avionics / Transponder .... Set / SBY

[ ]  Flight Instruments .... Check & Set

[ ]  Engine Instruments .... Check

Taxi

[ ]  Brakes & Steering .... Check

[ ]  Flight Instruments .... Check

Before Takeoff

[ ]  Cabin Doors .... Closed & Locked

[ ]  Seats / Seat Belts .... Upright & Secured

[ ]  Flight Controls .... Free & Correct

[ ]  Clock .... Check

[ ]  Fuel Quantity .... Check

[ ]  Auxilary Fuel Pump .... ON, check for rise, then OFF

[ ]  Engine Instruments .... Check

[ ]  Flight Instruments .... Check & Set

[ ]  Radios / Avionics .... Set for Departure

[ ]  Fuel Selector .... Both

[ ]  Elevator and Rudder Trim .... SET for takeoff

[ ]  Flaps .... Set for Takeoff

[ ]  Mixture .... Enrichen

[ ]  Throttle .... 1800 RPM

[ ]  Mixture .... Set for Takeoff

[ ]  Carb Heat .... Check

[ ]  Magnetos .... Check (drop <150, 50 differential)

[ ]  Propeller .... CYCLE

[ ]  Engine Instruments / Ammeter .... Check

[ ]  Vacuum .... Green Arc

[ ]  Throttle .... Idle / 1000 RPM

[ ]  Autopilot .... OFF

[ ]  Departure Briefing .... Complete (Vr, Vx/Vy, obstacles, emergency)

Shutdown and Securing

[ ]  Avionics Master Switch .... Off

[ ]  Exterior Lights (except beacon) .... Off

[ ]  Throttle .... 1000 RPM

[ ]  Mixture .... Idle Cut Off

[ ]  Magneto Switch .... Off

[ ]  Battery and Alternator .... Off

[ ]  Fuel Selector .... Left or Right

[ ]  Gust Lock .... Install

[ ]  Post Flight Inspection .... Accomplish

[ ]  Internal and External Covers .... Install

[ ]  Tie downs and Chocks .... Install

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172RG Skyhawk/Sam_Dawson_172RG_ground_checklist.txt

Rights: licensed

Checklist hash: 39b8a331013f8f09bacc2cf6ce85fb46a3b843141d5d40e47f010a9d4c9b39e2

Verify against the aircraft's own approved documentation before use.