Cessna 172RG Skyhawk — TFA_Cessna_172RG_Checklist_Emergency_2004-02

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "EMERGENCY LANDING WITHOUT ENGINE POWER" item count differs between passes (13 vs 11); verify nothing was dropped.

content_gap: Section "ENGINE FAILURE DURING FLIGHT" item count differs between passes (8 vs 7); verify nothing was dropped.

content_gap: Section "ENGINE FAILURE IMMEDIATELY AFTER TAKE-OFF" item count differs between passes (7 vs 6); verify nothing was dropped.

content_gap: Section "LANDING GEAR FAILS TO RETRACT" item count differs between passes (7 vs 6); verify nothing was dropped.

content_gap: Section "PRECAUTIONARY LANDING WITH ENGINE POWER" item count differs between passes (15 vs 14); verify nothing was dropped.

ENGINE FAILURE DURING TAKEOFF RUN  [EMERGENCY]

[ ]  THROTTLE .... IDLE

[ ]  BRAKES .... APPLY

[ ]  WING FLAPS .... RETRACT

[ ]  MIXTURE .... IDLE

[ ]  IGNITION SWITCH .... OFF

[ ]  MASTER SWITCH .... OFF

ENGINE FAILURE IMMEDIATELY AFTER TAKE-OFF  [EMERGENCY]

[ ]  AIRSPEED .... 70 KIAS

NOTE: 65 KIAS Flaps down

[ ]  MIXTURE .... IDLE

[ ]  FUEL VALVE .... OFF

[ ]  IGNITION SWITCH .... OFF

[ ]  FLAPS .... AS REQ (30° rec)

[ ]  MASTER SWITCH .... OFF

ENGINE FAILURE DURING FLIGHT  [EMERGENCY]

[ ]  AIRSPEED .... 75 KIAS

[ ]  CARB HEAT .... ON

[ ]  FUEL VALVE .... BOTH

[ ]  MIXTURE .... RICH

[ ]  AUX FUEL PUMP .... ON

NOTE: If fuel press is below .5 psi

[ ]  IGNITION SWITCH .... BOTH

[ ]  PRIMER .... IN & LOCKED

EMERGENCY LANDING WITHOUT ENGINE POWER  [EMERGENCY]

[ ]  SEATBELTS .... SECURE

[ ]  AIRSPEED .... 75 KIAS

NOTE: 65 KIAS Flaps down

[ ]  MIXTURE .... IDLE

[ ]  FUEL VALVE .... OFF

[ ]  IGNITION SWITCH .... OFF

[ ]  LANDING GEAR .... DOWN

NOTE: (UP FOR SOFT TERRAIN)

[ ]  WING FLAPS .... AS REQ

[ ]  MASTER SWITCH .... OFF

[ ]  DOORS .... UNLATCHED

[ ]  TOUCHDOWN .... TAIL LOW

[ ]  BRAKES .... HEAVILY

PRECAUTIONARY LANDING WITH ENGINE POWER  [EMERGENCY]

[ ]  SEATBELTS .... SECURE

[ ]  WING FLAPS .... 20°

[ ]  AIRSPEED .... 65 KIAS

[ ]  FIELD .... FLY OVER

[ ]  ELECT SWITCHES .... OFF

[ ]  LANDING GEAR .... DOWN

NOTE: (UP FOR SOFT TERRAIN)

[ ]  WING FLAPS .... 30° FINAL

[ ]  AIRSPEED .... 65 KIAS

[ ]  DOORS .... UNLATCH

[ ]  AVIONICS MASTER .... OFF

[ ]  MASTER SWITCH .... OFF

[ ]  TOUCHDOWN .... TAIL LOW

[ ]  IGNITION SWITCH .... OFF

[ ]  BRAKES .... HEAVILY

DURING START ON GROUND  [EMERGENCY]

[ ]  CRANKING .... CONTINUE

IF ENGINE STARTS:

[ ]  POWER .... 1700 RPM

[ ]  ENGINE .... SHUT DOWN

NOTE: (Inspect for damage)

IF ENGINE FAILS TO START:

[ ]  THROTTLE .... FULL OPEN

[ ]  MIXTURE .... IDLE

[ ]  CRANKING .... CONTINUE

[ ]  FIRE EXT .... OBTAIN

[ ]  ENGINE .... SECURE

[ ]  MASTER SWITCH .... OFF

[ ]  IGN. SWITCH .... OFF

[ ]  FUEL VALVE .... OFF

[ ]  FIRE .... EXTINGUISH

[ ]  DAMAGE .... INSPECT

ENGINE FIRE IN FLIGHT  [EMERGENCY]

[ ]  MIXTURE .... IDLE

[ ]  FUEL VALVE .... OFF

[ ]  MASTER SWITCH .... OFF

[ ]  CABIN HEAT/AIR .... OFF

[ ]  AIRSPEED .... 105 KIAS

NOTE: If fire is not extinguished, increase glide to an airspeed that provides an incombustible mixture.

[ ]  FORCED LAND .... EXECUTE

CABIN FIRE  [EMERGENCY]

[ ]  MASTER SWITCH .... OFF

[ ]  VENTS .... CLOSED

[ ]  FIRE EXT .... ACTIVATE

[ ]  CABIN .... VENTILATE

[ ]  LAND .... ASAP

ELECTRICAL FIRE IN FLIGHT  [EMERGENCY]

[ ]  MASTER SWITCH .... OFF

[ ]  AVIONICS PWR SW .... OFF

[ ]  ELECTRICAL SWS .... OFF

[ ]  VENTS .... CLOSED

[ ]  EXTINGUISHER .... ACTIVATE

[ ]  CABIN .... VENTILATE

[ ]  MASTER SWITCH .... ON

[ ]  C/BREAKERS .... CHECK

[ ]  RADIO SWITCHES .... OFF

[ ]  AVIONICS PWR SW .... ON

[ ]  ELECTRICAL SWITCHES .... ON ONE AT A TIME

[ ]  VENTS .... OPEN

WING FIRE  [EMERGENCY]

[ ]  NAV LIGHTS .... OFF

[ ]  STROBE LIGHTS .... OFF

[ ]  PITOT HEAT SWITCH .... OFF

LANDING WITH FLAT MAIN TIRE  [ABNORMAL]

[ ]  APPROACH .... NORMAL

[ ]  TOUCHDOWN .... GOOD WHEEL FIRST

LANDING GEAR FAILS TO RETRACT  [ABNORMAL]

[ ]  MASTER SWITCH .... ON

[ ]  GEAR LEVER .... CHECK UP

[ ]  CIRCUIT BREAKERS .... IN

[ ]  GEAR UP LIGHT .... CHECK

[ ]  GEAR LEVER .... RECYCLE

[ ]  GEAR MOTOR .... CHECK

NOTE: (ammeter and noise)

LANDING GEAR FAILS TO EXTEND  [ABNORMAL]

[ ]  MASTER SWITCH .... ON

[ ]  GEAR LEVER .... DOWN

[ ]  CIRCUIT BREAKERS .... IN

[ ]  AIRSPEED .... 100 KIAS/LESS

[ ]  EMERGENCY PUMP .... PUMP

[ ]  GEAR DOWN LIGHT .... ON

[ ]  PUMP HANDLE .... STOW

GEAR UP LANDING  [EMERGENCY]

[ ]  GEAR LEVER .... UP

[ ]  CIRCUIT BREAKERS .... IN

[ ]  RUNWAY .... SELECT

[ ]  FLAPS .... 30° ON FINAL

[ ]  AIRSPEED .... 65 KIAS

[ ]  DOORS .... UNLATCHED

[ ]  AVIONICS PWR. SW .... OFF

[ ]  MASTER SWITCH .... OFF

[ ]  TOUCH DWN .... TAIL LOW

[ ]  MIXTURE .... IDLE

[ ]  IGNITION SWITCH .... OFF

[ ]  FUEL VALVE .... OFF

[ ]  AIRPLANE .... EVACUATE

LANDING WITHOUT POSITIVE INDICATION OF GEAR LOCKING  [ABNORMAL]

[ ]  B4 LAND CHECK .... DO

[ ]  APPROACH .... NORMAL

[ ]  CIRCUIT BREAKERS .... IN

[ ]  LANDING .... TAIL LOW

[ ]  BRAKING .... MINIMUM

[ ]  TAXI .... SLOWLY

[ ]  ENGINE .... SHUTDOW

LANDING WITH DEFECTIVE NOSE GEAR  [ABNORMAL]

[ ]  MOVABLE LOAD X-FR. .... TO BAGGAGE AREA

[ ]  PASSENGER .... MOVE TO REAR SEAT

[ ]  B4 LAND CHECK .... DO

[ ]  RUNWAY .... HARD/SMOOTH

[ ]  FLAPS .... 30 DEGREES

[ ]  CABIN DOORS .... UNLATCH

[ ]  AVIONICS PWR SW .... OFF

[ ]  MASTER SWITCH .... OFF

[ ]  LAND .... TAIL LOW

[ ]  MIXTURE .... IDLE

[ ]  IGNITION SWITCH .... OFF

[ ]  FUEL SELECTOR .... OFF

[ ]  HOLD NOSE UP .... HOLD

[ ]  AIRPLANE .... EVACUATE

LOW VOLTAGE LIGHT DURING FLIGHT  [ABNORMAL]

AMMETER INDICATES DISCHARGE

[ ]  AVIONICS PWR SW .... OFF

[ ]  ALTERNATOR C/B .... CHECK IN

[ ]  MASTER SWITCH .... OFF

[ ]  MASTER SWITCH .... ON

[ ]  LOW-VOLT LIGHT .... OFF

[ ]  AVIONICS PWR SW .... ON

IF LOW-VOLTAGE LIGHT ILLUMINATES AGAIN:

[ ]  ALTERNATOR .... OFF

[ ]  ESSENTIAL RADIOS ONLY .... USE

[ ]  FLIGHT .... TERMINATE

AMMETER EXCESSIVE RATE OF CHARGE  [ABNORMAL]

FULL SCALE DEFLECTION

[ ]  ALTERNATOR .... OFF

[ ]  ALTERNATOR C/B .... PULL

[ ]  ONLY ESSENTIAL RADIOS .... USE

[ ]  FLIGHT .... TERMINATE

DITCHING  [EMERGENCY]

[ ]  RADIO MAYDAY .... ON 121.5

[ ]  XPONDER .... SQUAWK 7700

[ ]  HEAVY OBJECTS .... SECURE OR JETISON

[ ]  LANDING GEAR .... UP

[ ]  FLAPS .... 20° to 30°

[ ]  SEATBELTS .... SECURE

[ ]  AIRSPEED .... 60 KIAS

[ ]  DESCENT RATE .... 300 FT/MIN

NOTE: IF NO POWER AVAILABLE, 70 KIAS FLAPS UP OR 65 KIAS WITH 10 DEGREES FLAPS

APPROACH:

NOTE: HIGH WINDS HEAVY SEAS: INTO WIND

NOTE: LIGHT WINDS & HEAVY SWELLS: PARALLEL TO SWELLS

[ ]  DOORS .... UNLATCH

[ ]  TOUCHDOWN .... LEVEL

[ ]  FACE .... CUSHION

[ ]  AIRPLANE .... EVACUATE

[ ]  VESTS & RAFT .... INFLATE

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172RG Skyhawk/TFA_Cessna_172RG_Checklist_Emergency_2004-02.txt

Rights: licensed

Checklist hash: efbbc7189d031218f0086c4c5a1576f49ba873fb72dc9df43abfdab71aa78054

Verify against the aircraft's own approved documentation before use.