Cessna 172RG Skyhawk — TFA_Cessna_172RG_Checklist_Normal_2004-02

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Before Takeoff" item count differs between passes (20 vs 21); verify nothing was dropped.

content_gap: Section "Radio Frequencies" item count differs between passes (10 vs 9); verify nothing was dropped.

Speeds

[ ]  BEST GLIDE .... 73 KIAS

[ ]  Vr .... 55 KIAS

[ ]  Vx .... 67 KIAS

[ ]  Vy .... 84 KIAS

[ ]  Vne .... 164 KIAS

[ ]  Vno .... 145 KIAS

[ ]  Vso .... 42 KIAS

[ ]  Vlo .... 140 KIAS

[ ]  Vle .... 164 KIAS

[ ]  Va .... 89-106 KIAS

[ ]  Vfe .... 130-100 KIAS

[ ]  Max Crosswind .... 15 KIAS

Radio Frequencies

[ ]  Take Flight .... 129.25

NOTE: (monitor when able)

[ ]  Merrill ATIS .... 123.70

[ ]  Merrill Ground .... 121.70

[ ]  Merrill Tower .... 126.00

[ ]  Anch Appch .... N 119.1/S 126.4

[ ]  Elmendorf Tower .... 127.2

[ ]  Big Lake/Willow/Wasilla .... 122.8

[ ]  Goose Bay .... 122.9

[ ]  Birchwood .... 123.0

Cold Weather Ops

NOTE: Ensure oil temp is in the green or moving before engine run-up/mag check and prior to flight. Preheating a cold engine is required before starting during winter operations.

Preflight Inspection - Cabin

[ ]  AROW/100hr/Mtc .... CHECK

[ ]  Landing Gear Lever .... DOWN

[ ]  Hobbs Meter .... CHECK

[ ]  Control Lock .... REMOVE

[ ]  Ignition Switch .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Master Switch .... ON

[ ]  Fuel Qty .... CHECK

[ ]  Landing Gear Light .... ON

[ ]  Av. Cooling Fan .... CHECK

[ ]  Lights .... CHECK

[ ]  Flaps .... 30(

[ ]  Master Switch .... OFF

[ ]  Fuel Selector Valve .... BOTH

[ ]  Alt Static Source .... OFF

[ ]  Baggage Door .... CHECK

Preflight Inspection - Empennage

[ ]  Rudder Gust Lock .... REMOVE

[ ]  Tail Tie Down .... REMOVE

[ ]  Elevator & Rudder .... CHECK

Preflight Inspection - Right Wing

[ ]  Aileron & Flap .... CHECK

[ ]  Skin/Tip/Lights .... CHECK

[ ]  Tie Down .... REMOVE

[ ]  Strut/Tire/Brake .... CHECK

[ ]  Fuel Tank Sump .... SAMPLE

[ ]  Fuel Qty .... CHECK

[ ]  Fuel Filler Cap .... SECURE

Preflight Inspection - Nose

[ ]  Static Source (both) .... CHECK

[ ]  Engine Oil .... 6 to 8 QUARTS

[ ]  Fuel Strainer .... DRAIN & SAMPLE

[ ]  Fuel Selector/Belly Drain .... SAMPLE

[ ]  Prop & Spinner .... CHECK

[ ]  Landing Light .... CHECK

[ ]  Nose Gear Doors .... CHECK

[ ]  Nose Wheel & Strut .... CHECK

Preflight Inspection - Left Wing

[ ]  Fuel Tank Sump .... SAMPLE

[ ]  Fuel Qty .... CHECK

[ ]  Fuel Filler Cap .... SECURE

[ ]  Strut/Tire/Brake .... CHECK

[ ]  Pitot Tube & Drain .... CHECK

[ ]  Stall Warning .... CHECK

[ ]  Fuel Tank Vent .... CHECK

[ ]  Tie Down .... REMOVE

[ ]  Aileron & Flap .... CHECK

Before Starting Engine

[ ]  Preflight .... COMPLETE

[ ]  Passenger Brief .... COMPLETE

[ ]  Seat Belts .... SECURE

[ ]  Fuel Selector Valve .... BOTH

[ ]  Avionics Pwr. Switch .... OFF

[ ]  Avionics Circuit Breakers .... IN

[ ]  Circuit Breakers .... CHECK IN

[ ]  Elec. Equip./Autopilot .... OFF

[ ]  Cowl Flaps .... OPEN

[ ]  Landing Gear Lever .... DOWN

[ ]  Brakes .... SET HOLD

Starting Engine

[ ]  Beacon .... ON

[ ]  Mixture .... RICH

[ ]  Propeller .... HIGH RPM

[ ]  Throttle .... 1/4 INCH

[ ]  Carb Heat .... OFF

[ ]  Prime .... AS REQ.

[ ]  Master Switch .... ON

[ ]  Propeller Area .... CLEAR

[ ]  Ignition Switch .... START

[ ]  Oil Pressure .... CHECK

[ ]  Avionics Pwr. Switch .... ON

[ ]  Lights .... AS REQIRED

[ ]  Flaps .... UP

Before Takeoff

[ ]  Brakes .... TEST/SET

[ ]  Doors .... CLOSED/LOCKED

[ ]  Seat Belts .... FASTENED

[ ]  Controls .... FREE & CORRECT

[ ]  Trim .... SET

[ ]  Flight Instruments .... CHECK

[ ]  Fuel Selector Valve .... BOTH

[ ]  Mixture .... RICH

[ ]  Aux Fuel Pump .... ON/OFF

[ ]  Throttle .... 1800 RPM

[ ]  a. Magnetos .... CHECK

[ ]  b. Prop .... CYCLE

[ ]  c. Carb Heat .... CHECK

[ ]  d. Engine Inst .... CHECK

[ ]  e. Ammeter .... CHECK

[ ]  f. Suction Gg. .... CHECK

[ ]  Throttle .... 800-1000 RPM

[ ]  Radios .... CHECK

[ ]  Autopilot .... RECHECK OFF

[ ]  Throttle Friction .... ADJUST

Taking Runway

[ ]  Clearance Rec’vd .... TO/P&H

[ ]  Approach End .... CLEAR

[ ]  Lights .... ON

[ ]  Camera .... Transponder Alt

[ ]  Action .... D.G. Runway Hdg

[ ]  Time Off .... Record

Normal Takeoff

[ ]  Flaps .... 0(

[ ]  Carb Heat .... OFF

[ ]  Throttle .... FULL OPEN

[ ]  Airspeed/Eng Inst .... CHECK

[ ]  Rotate .... 55 KIAS

[ ]  Climb Speed .... 70-80 KIAS

[ ]  Brakes .... APPLY

[ ]  Landing Gear .... RETRACT

Short Field Takeoff

[ ]  Flaps .... 0 or 10 DEGREES

NOTE: (Zero if within 100lbs. of Gross)

[ ]  Brakes .... HOLD

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... OFF

[ ]  Engine Instr. .... GREEN/NORM

[ ]  Brakes .... RELEASE

[ ]  Rotate .... 55 KIAS

[ ]  Climb Speed .... 63 KIAS

[ ]  Over Obstacle .... 84 KIAS & FLAPS UP

Enroute Climb

[ ]  Airspeed .... 85-95 KIAS

[ ]  Power .... 25 in Hg & 2500 rpm

[ ]  Mixture .... AS REQ

[ ]  Cowl Flaps .... OPEN

Cruise

[ ]  Power .... 15-25 in Hg / 2100-2700 RPM

[ ]  Elev/Rudder .... TRIM

[ ]  Cowl Flaps .... CLOSED

[ ]  Mixture .... AS REQ

[ ]  Lights .... AS REQ

[ ]  Flight Plan .... OPEN

[ ]  Company Freq .... MONITOR

Descent

[ ]  Fuel Selector .... BOTH

[ ]  Mixture .... RICH

[ ]  Carb Heat .... ON AS REQ

[ ]  Cowl Flaps .... CLOSED

[ ]  Throttle .... AS REQ

[ ]  Landing Light .... ON

[ ]  Wing Flaps .... AS REQ

Before Landing

[ ]  Carb Heat .... ON

[ ]  Cowl Flaps .... CLOSED

[ ]  Gas (fuel selector) .... BOTH

[ ]  Undercarriage .... DOWN

[ ]  Mixture .... RICH

[ ]  Propeller .... HIGH RPM

[ ]  Seat Belts .... SECURE

NOTE: ON FINAL CALL OUT & VERIFY AGAIN:

NOTE: “FINAL UMP’s CHECK…….

[ ]  Undercarriage .... Green/Down & Locked

[ ]  Mixture .... RICH

[ ]  Prop .... HIGH

Normal Landing

[ ]  Airspeed .... 65-75 KIAS

[ ]  Flaps .... AS REQ

[ ]  Airspeed .... 60-70 KIAS

[ ]  Throttle .... AS REQ

[ ]  Braking .... MIN REQ

Short Field Landing

[ ]  Airspeed .... 65-75 KIAS

[ ]  Flaps .... 30 DEGREES

[ ]  Short Final .... 63 KIAS

[ ]  Throttle .... AS REQ

[ ]  Brakes .... APPLY HEAVILY

[ ]  Flaps .... RETRACT

Go Around Procedure With Full Flaps

[ ]  Throttle .... FULL OPEN

[ ]  Carb Heat .... COLD

[ ]  Flaps .... 20(

[ ]  Airspeed .... INCREASING

[ ]  Altitude .... INCREASING

[ ]  Flaps .... 10(

[ ]  Vy 87 KIAS .... OBTAIN

[ ]  Flaps .... 0(

[ ]  Cowl Flaps .... OPEN

After Landing

[ ]  Flaps .... UP

[ ]  Carb Heat .... COLD

[ ]  Cowl Flaps .... OPEN

[ ]  Transponder .... OFF

[ ]  Lights .... OFF

[ ]  Taxi Clearance .... OBTAIN

Securing Airplane

[ ]  Avionics Pwr Sw .... OFF

[ ]  Electrical Equip .... OFF

[ ]  Throttle .... 1000 RPM

[ ]  Mixture .... IDLE CUT OFF

[ ]  Throttle .... Close as rpm drops

[ ]  Ignition Switch .... OFF

[ ]  Master Switch .... OFF

[ ]  Cowl Flaps .... (winter) CLOSED

[ ]  Control Lock .... INSTALL

[ ]  Hobbs .... RECORD

[ ]  Flight Plan .... CLOSE

[ ]  Tie Down .... SECURE

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172RG Skyhawk/TFA_Cessna_172RG_Checklist_Normal_2004-02.txt

Rights: licensed

Checklist hash: d31276c68df50358ac78dd7f3c73e597efe1763b2559c1f704a2ad15aa16f4d1

Verify against the aircraft's own approved documentation before use.