Cessna 172S Skyhawk — JetStream_172S_N101GV Front Checklist 4.8.05

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Run Up" item count differs between passes (22 vs 25); verify nothing was dropped.

Before Start

[ ]  Preflight Inspection .... Complete

[ ]  Pass, Briefing .... Complete

[ ]  Seats / Seatbelts .... Adjust, Lock

[ ]  Brakes .... Set

[ ]  Circuit Breakers .... Check In

[ ]  Electrical and Avionics .... Off

Normal Engine Start

[ ]  Fuel Select Valve .... Both

[ ]  Fuel Shutoff Valve .... In (On)

[ ]  Mixture .... Idle Cutoff

[ ]  Throttle .... Open 1/4 inch

[ ]  Master Switch .... On

[ ]  Rotating Beacon .... On

[ ]  Aux Fuel Pump .... On

[ ]  Mixture .... Full Rich 3-5 GPH, then idle

[ ]  Aux Fuel Pump .... Off

[ ]  Area .... Clear

[ ]  Ignition Switch .... Engage, Release

when engine fires

[ ]  Mixture .... Smoothly full rich

[ ]  Oil Pressure .... Check

[ ]  Taxi, Nav. Lights .... As Required

[ ]  Avionics Master .... On

[ ]  Flaps .... Retract

Taxi

[ ]  Mixture .... Lean for Taxi

[ ]  Brakes .... Release, Check

[ ]  Instruments .... Check

Run Up

[ ]  Brakes .... Set

[ ]  Seats / Seatbelts .... Check Secure

[ ]  Doors .... Closed, Locked

[ ]  Flight Controls .... Free and Correct

[ ]  Flight Instruments .... Check and Set

[ ]  Radios / Navigation .... Set for Departure

[ ]  Navigation / GPS .... Set for Departure

[ ]  Fuel Selector .... Both

[ ]  Throttle .... 1800 rpm

[ ]  Mixture .... Set for Altitude

[ ]  Magnetos .... Left, Right, Both

[ ]  Engine Instruments .... Check

[ ]  Vacuum .... Check

[ ]  Ammeter .... Check with load

[ ]  Annunciator panel .... Test, Clear

[ ]  Throttle .... 1000 RPM

[ ]  Autopilot .... Off

[ ]  Throttle friction lock .... Set

[ ]  Elevator Trim .... Set for takeoff

[ ]  Flaps .... Set for takeoff

[ ]  Emergency, dep. brief .... Perform

[ ]  Brakes .... Release

Before Takeoff

[ ]  Landing light, strobes .... On

[ ]  Transponder .... Set to altitude

[ ]  Area .... Clear

Normal Takeoff

[ ]  Throttle .... Full Open

[ ]  Mixture .... Adjust smooth

[ ]  Elevator control .... Lift nose at 55 KIAS

[ ]  Climb Speed .... 75 KIAS

Climb (above 1000' AGL)

[ ]  Climb speed .... 75 KIAS

[ ]  Engine instruments .... Green

[ ]  Taxi, landing lights .... Off

Cruise

[ ]  Power .... 2100-2700 rpm

[ ]  Elevator trim .... Adjust

[ ]  Mixture .... Lean for altitude

Descent

[ ]  Fuel Selector .... Both

[ ]  Mixture .... Set

[ ]  Power .... Set

[ ]  Altimeter .... Set

[ ]  Nav/GPS .... Set

Before Landing

[ ]  Seats / Seatbelts .... Check Secure

[ ]  Fuel Selector .... Both

[ ]  Autopilot .... Off

[ ]  Taxi, landing lights .... On

Normal Landing

[ ]  Airspeed .... 65-75 KIAS flaps up

[ ]  Flaps .... As required

[ ]  Airspeed .... 60-70 KIAS flaps down

After Landing

[ ]  Flaps .... Retract

[ ]  Transponder .... Standby

[ ]  Land, Strobe lights .... Off

[ ]  Trim .... Takeoff

Shutdown and Securing

[ ]  Avionics and Switches .... Off

[ ]  Throttle .... Idle

[ ]  Mixture .... Idle Cutoff

[ ]  Mags .... Off

[ ]  Master .... Off

[ ]  Control Lock .... In

[ ]  Hobbs and Tach .... Record

[ ]  Tie Downs .... Attached

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172S Skyhawk/JetStream_172S_N101GV Front Checklist 4.8.05.txt

Rights: licensed

Checklist hash: 3d2434e630c8ace5f75b9c9fc6da33a3490b0f454a31f36c1a3ffe3d7139efff

Verify against the aircraft's own approved documentation before use.