Cessna 172S Skyhawk — Sam_Dawson_C172S_Ground

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

172S BEFORE STARTING ENGINE

[ ]  Preflight Inspection .... Complete

[ ]  CRM/Passenger Briefing .... Complete

[ ]  Seats and Seat Belts .... Adjust & Lock

[ ]  Brakes .... Test & Set

[ ]  Fuel Selector .... Both

[ ]  Fuel Shutoff .... On (In)

[ ]  Throttle .... Open ¼ “

[ ]  Mixture .... Idle Cutoff

[ ]  Avionics Master Switch .... Off

[ ]  Electrical Equipment .... Off

[ ]  Circuit Breakers .... Check In

STARTING ENGINE

[ ]  Master Switch .... On

[ ]  Lights .... As Required

[ ]  Auxiliary Fuel Pump .... On

[ ]  Mixture .... Full Rich, then off after fuel flow

[ ]  Auxiliary Fuel Pump .... Off

[ ]  Propeller Area .... Clear

[ ]  Ignition Switch .... Start

[ ]  Mixture .... Advance

[ ]  Oil Pressure / Ammeter .... Check

BEFORE TAXI

[ ]  Flaps .... Retract to takeoff setting

[ ]  Mixture .... Lean

[ ]  Throttle .... 1000 RPM

[ ]  Avionics Master Switch (Radios) .... On

[ ]  ATIS / AWOS .... Receive

[ ]  Clearance .... Obtain

[ ]  MFD & GPS .... Initialize

[ ]  Avionics / Transponder .... Set / SBY

[ ]  Flight Instruments .... Check & Set

[ ]  Engine Instruments .... Check

TAXI

[ ]  Brakes & Steering .... Check

[ ]  Flight Instruments .... Check

NOTE: 3 March 2007

BEFORE TAKEOFF

[ ]  Cabin Doors .... Closed & Locked

[ ]  Seats / Seat Belts .... Upright & Secured

[ ]  Flight Controls .... Free & Correct

[ ]  Clock .... Set

[ ]  Fuel Quantity .... Check

[ ]  Engine Instruments .... Check

[ ]  Flight Instruments .... Check & Set

[ ]  MFD & GPS .... Set

[ ]  Radios / Avionics .... Set for Departure

[ ]  NAV / GPS Switch .... Set

[ ]  Autopilot .... Test / Off

[ ]  Fuel Selector .... Both

[ ]  Fuel Shutoff .... On (In)

[ ]  Electric Trim / Elevator Trim .... Check / Set

[ ]  Flaps .... Set for Takeoff

[ ]  Mixture .... Enrichen

[ ]  Throttle .... 1800 RPM

[ ]  Mixture .... Lean for Takeoff

[ ]  Magnetos .... Check

[ ]  Engine Instruments / Ammeter .... Check

[ ]  Vacuum .... Green Arc

[ ]  Annunciator Panel .... Clear

[ ]  Throttle .... Idle / 1000 RPM

[ ]  Throttle Friction Lock .... Adjust

[ ]  Departure Briefing .... Complete

Shutdown and Securing

[ ]  Avionics Master .... Off

[ ]  Exterior Lights (except beacon) .... Off as Required

[ ]  Throttle .... 1000 RPM

[ ]  Mixture .... Idle Cut Off

[ ]  Magneto Switch .... Off

[ ]  Battery and Alternator .... Off

[ ]  Fuel Selector Valve .... Left or Right

[ ]  Gust Lock .... Install

[ ]  Post Flight Inspection .... Accomplish

[ ]  Internal and External Covers .... Install

[ ]  Tiedowns and Chocks .... Install

NOTE: 3 March 2007

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/172S Skyhawk/Sam_Dawson_C172S_Ground.txt

Rights: licensed

Checklist hash: a261dea2e8841a4b94374f5eaa0160c6ee229d1d50f85d746394da7ff12afcf0

Verify against the aircraft's own approved documentation before use.