Cessna 177 Cardinal — C177_checklist_N29655 new

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

BEFORE STARTING ENGINE

[ ]  Preflight Inspection .... COMPLETE

[ ]  Seats Belts .... ADJUST AND LATCH

[ ]  Fuel Selector Valve .... ON BOTH

[ ]  Fuel Shutoff Valve .... ON

[ ]  Electrical Equipment .... OFF

[ ]  Avionics Master .... OFF

[ ]  Brakes .... TEST & SET

[ ]  Circuit Breakers .... CHECK IN

STARTING THE ENGINE

[ ]  Mixture .... RICH

[ ]  Carburetor Heat .... COLD

[ ]  Prime .... AS REQUIRED

[ ]  Throttle .... OPEN 1/8 INCH

[ ]  Propeller Area .... CLEAR

[ ]  Master Switch .... ON

[ ]  Ignition Switch .... START

[ ]  Oil Pressure .... CHECK

[ ]  Throttle .... 1000 RPM

TAXI CHECK

[ ]  Avionics Master .... ON

[ ]  Radios .... ON AS REQUIRED

[ ]  Beacon Light .... ON

[ ]  Flaps .... UP

[ ]  Call Pax Clearance Delivery .... 135.2

[ ]  Call Pax Ground for taxi .... 120.6

[ ]  Brakes .... CHECK

BEFORE TAKE-OFF

[ ]  Parking Brake .... SET

[ ]  Flight Controls .... FREE AND CORRECT

[ ]  Fuel Shutoff Valve .... ON

[ ]  Fuel Selector Valve .... ON BOTH

[ ]  Mixture .... RICH (BELOW 3000 FEET)

[ ]  Elevator Trim .... TAKE-OFF

BEFORE TAKE-OFF Continued

[ ]  Throttle Setting .... 1700 RPM

[ ]  Engine Instruments and Ammeter .... CHECK

[ ]  Carburetor Heat .... CHECK

[ ]  Magnetos .... CHECK, MAX DROP 150 RPM, MAX DIFFERENTIAL 50 RPM

[ ]  Aux Fuel Pump .... CHECK

[ ]  Suction Gage .... CHECK 4.6 IN HG – 5.4 IN HG

[ ]  Throttle .... IDLE

[ ]  Flight Instruments .... SET

[ ]  Radios .... SET

[ ]  NAV Lights .... AS REQUIRED

[ ]  Strobe Lights .... AS REQUIRED

[ ]  Throttle Friction Lock .... ADJUST

[ ]  Cabin Doors & Windows .... CLOSED

[ ]  Call Pax Tower for take-off .... 123.7

[ ]  Transponder .... ON ALT

NORMAL TAKE-OFF

[ ]  Wing Flaps .... 1/4 DOWN

[ ]  Carburetor Heat .... COLD

[ ]  Throttle .... FULL OPEN

[ ]  Elevator Control .... LIFT NOSE AT 60 MPH

[ ]  Climb Speed .... 75 TO 90 MPH

CLIMB

[ ]  Cruise Climb .... 90 TO 100 MPH

[ ]  Vx .... 67 MPH

[ ]  Vy .... 88 MPH

[ ]  Throttle .... FULL OPEN

[ ]  Mixture .... FULL RICH (BELOW 3000 FEET)

CRUISING

[ ]  Power .... 65% CRUISE RPM

[ ]  Trim Tab .... ADJUST

[ ]  Mixture .... LEAN

BEFORE LANDING

[ ]  Seats Belts .... ADJUST AND LATCH

[ ]  Mixture .... RICH

[ ]  Fuel Selector .... BOTH

[ ]  Carburetor Heat .... ON APPLY FULL HEAT BEFORE CLOSING THROTTLE

NORMAL LANDING

[ ]  Airspeed .... 80–75 MPH (FLAPS UP)

[ ]  Wing Flaps .... 1/4 BELOW 130 MPH

[ ]  Wing Flaps .... AS DESIRED (BELOW 105 MPH)

[ ]  Airspeed .... 70-80 MPH (FLAPS DOWN)

[ ]  Touchdown .... MAIN WHEELS FIRST

[ ]  Landing Roll .... LOWER NOSE WHEEL GENTLY

[ ]  Braking .... MINIMUM REQUIRED

AFTER LANDING

[ ]  Wing Flaps .... UP

[ ]  Carburetor Heat .... COLD

[ ]  Strobe Lights .... OFF

SECURING AIRCRAFT

[ ]  Parking Brake .... SET

[ ]  Radios & Electrical Equipment .... OFF

[ ]  Avionics Master .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Ignition & Master Switch .... OFF

[ ]  Control Lock .... INSTALLED

[ ]  Windshield Sun Reflector .... INSTALLED

[ ]  Cowl & Tail Plugs .... INSTALLED

[ ]  Pitot Tube Cover .... INSTALLED

[ ]  Refuel Airplane .... AS REQUIRED

[ ]  Chock & Tie-down Airplane .... CHOCK & TIE-DOWN

[ ]  Record Hobbs, Tach & Gas .... RECORD

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/177 Cardinal/C177_checklist_N29655 new.txt

Rights: licensed

Checklist hash: a8315c9651798ad90792c5bd4d83af99eaabefb9e953ebd8b8dd6cf6a86aacf9

Verify against the aircraft's own approved documentation before use.