Cessna 177 Cardinal — jochman-clc177

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Speeds for Normal Operation" item count differs between passes (14 vs 16); verify nothing was dropped.

Engine Failure During Takeoff  [EMERGENCY]

Engine failure during takeoff

[ ]  Throttle [MEMORY] .... IDLE

[ ]  Brakes [MEMORY] .... APPLY

[ ]  Wing Flaps [MEMORY] .... RETRACT

[ ]  Mixture [MEMORY] .... IDLE CUT-OFF

[ ]  Ignition Switch [MEMORY] .... OFF

[ ]  Master Switch [MEMORY] .... OFF

Engine Failure After Takeoff  [EMERGENCY]

Engine failure after takeoff

[ ]  Airspeed [MEMORY] .... 80 MPH

[ ]  Mixture [MEMORY] .... IDLE CUT-OFF

[ ]  Fuel Selector Valve [MEMORY] .... OFF

[ ]  Ignition Switch [MEMORY] .... OFF

[ ]  Wing Flaps [MEMORY] .... AS REQUIRED

[ ]  Master Switch [MEMORY] .... OFF

Engine Failure During Flight  [EMERGENCY]

Engine failure during flight

[ ]  Airspeed [MEMORY] .... 85 MPH

[ ]  Carburetor Heat [MEMORY] .... ON

[ ]  Fuel Selector Valve [MEMORY] .... BOTH

[ ]  Mixture [MEMORY] .... RICH

[ ]  Ignition Switch [MEMORY] .... BOTH or START

[ ]  Primer [MEMORY] .... IN & LOCKED

Emergency Landing  [EMERGENCY]

Emergency landing

[ ]  Airspeed [MEMORY] .... 80 MPH

[ ]  Mixture [MEMORY] .... IDLE CUT-OFF

[ ]  Fuel Selector Valve [MEMORY] .... OFF

[ ]  Ignition Switch [MEMORY] .... OFF

[ ]  Wing Flaps [MEMORY] .... AS REQUIRED

[ ]  Master Switch [MEMORY] .... OFF

[ ]  Doors [MEMORY] .... UNLATCH PRIOR TO TOUCHDOWN

[ ]  Touchdown [MEMORY] .... SLIGHTLY TAIL LOW

[ ]  Brakes [MEMORY] .... APPLY HEAVILY

Precautionary Landing  [EMERGENCY]

Precautionary landing

[ ]  Wing Flaps .... 15°

[ ]  Airspeed .... 75 MPH

[ ]  Select field .... SUITABLE

[ ]  Avionics Power Switch .... OFF

[ ]  Electrical Switches .... OFF

[ ]  Wing Flaps (on final approach) .... 30°

[ ]  Airspeed .... 75 MPH

[ ]  Master Switch .... OFF

[ ]  Doors .... UNLATCH PRIOR TO TOUCHDOWN

[ ]  Touchdown .... SLIGHTLY TAIL LOW

[ ]  Ignition Switch .... OFF

[ ]  Brakes .... APPLY HEAVILY

Cabin

[ ]  Pilot's Operating Handbook .... AVAILABLE

[ ]  Aircraft Documents .... AVAILABLE

[ ]  Control Wheel Lock .... REMOVE

[ ]  Ignition Switch .... OFF

[ ]  Avionics Power Switch .... OFF

[ ]  Master Switch .... ON

[ ]  Fuel Quanity Indicators .... CHECK QUANTITY

[ ]  Flaps .... DOWN

[ ]  Master Switch .... OFF

Empennage

[ ]  Rudder Gust Lock .... REMOVE

[ ]  Tail Tie-Down .... DISCONNECT

[ ]  Control Surfaces .... CHECK FREE

Right Wing

[ ]  Aileron .... CHECK FREE

[ ]  Wing Tie-Down .... DISCONNECT

[ ]  Main Wheel Tire .... CHECK

[ ]  Fuel Tank Sump .... SAMPLE

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Filler Cap .... SECURE

Nose

[ ]  Engine Oil Level .... CHECK

[ ]  Fuel Strainer .... DRAIN

[ ]  Propeller and Spinner .... CHECK

[ ]  Landing Light .... CHECK

[ ]  Carburetor Air Filter .... CHECK

[ ]  Nose Wheel Strut and Tire .... CHECK

[ ]  Static Source Opening .... CHECK

Left Wing

[ ]  Main Wheel Tire .... CHECK

[ ]  Fuel Tank Sump .... SAMPLE

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Filler Cap .... SECURE

[ ]  Pitot Tube Cover .... REMOVE

[ ]  Fuel Tank Vent Opening .... CHECK

[ ]  Stall Warning Opening .... CHECK

[ ]  Wing Tie-Down .... DISCONNECT

[ ]  Aileron .... CHECK FREE

Before Starting Engine

[ ]  Preflight Inspection .... COMPLETE

[ ]  Seats, Belts, Shoulder Harnesses .... ADJUST & LOCK

[ ]  Fuel Selector Valve .... BOTH

[ ]  Avionics Power Switch .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Brakes .... TEST & SET

[ ]  Circuit Breakers .... CHECK IN

[ ]  Cowl Flaps .... OPEN

Starting Engine

[ ]  Hobbs Time .... RECORD

[ ]  Mixture .... RICH

[ ]  Propeller .... HIGH RPM

[ ]  Carburetor Heat .... COLD

[ ]  Master Switch .... ON

[ ]  Prime .... AS REQUIRED

[ ]  Throttle .... OPEN ½ INCH

[ ]  Propeller Area .... CLEAR

[ ]  Ignition Switch .... START

[ ]  Throttle .... 1000 RPM

[ ]  Oil Pressure .... CHECK

[ ]  Flashing Beacon, Nav. Lights .... ON

[ ]  Avionics Power Switch .... ON

[ ]  Radios .... ON

[ ]  Transponder .... STANDBY

Before Taxi

[ ]  Wing Flaps .... UP

[ ]  Parking Brake .... RELEASE

[ ]  Brakes .... TEST

[ ]  Area .... CLEAR

Before Takeoff

[ ]  Parking Brake .... SET

[ ]  Cabin Doors and Windows .... CLOSED & LOCKED

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Flight Instruments .... SET

[ ]  Fuel Selector Valve .... BOTH

[ ]  Mixture (Below 3000') .... RICH

[ ]  Stabalator Trim .... TAKEOFF

[ ]  Rudder Trim (if installed) .... TAKEOFF

[ ]  Throttle .... 1800 RPM

[ ]  Magnetos .... CHECK

[ ]  Propeller .... CYCLE

[ ]  Auxiliary Fuel Pump .... CHECK

[ ]  Engine Instruments .... CHECK

[ ]  Ammeter .... CHECK

[ ]  Suction Guage .... CHECK

[ ]  Throttle .... 1000 RPM

[ ]  Radios .... SET

[ ]  Transponder .... ON-ALT

Normal Takeoff

Normal takeoff

[ ]  Wing Flaps .... 0° - 10°

[ ]  Carburetor Heat .... COLD

[ ]  Throttle .... FULL OPEN

[ ]  Propeller .... 2700 RPM

[ ]  Elevator Control .... LIFT NOSE AT 60 MPH

[ ]  Climb Speed .... BEST RATE

Short Field Takeoff

Short field takeoff

[ ]  Wing Flaps .... 10°

[ ]  Carburetor Heat .... COLD

[ ]  Brakes .... APPLY

[ ]  Throttle .... FULL OPEN

[ ]  Propeller .... 2700 RPM

[ ]  Mixture .... LEAN above 3000', RICH below 3000'

[ ]  Brakes .... RELEASE

[ ]  Elevator Control .... SLIGHTLY TAIL LOW

[ ]  Climb Speed (Until Obstacles Cleared) .... 69 MPH

Enroute Climb

Enroute climb

[ ]  Airspeed .... 90-100 MPH

[ ]  Power .... 24" –Full Power, 2500-2700 RPM

[ ]  Mixture .... LEAN above 3000', RICH below 3000'

[ ]  Cowl Flaps .... OPEN AS REQUIRED

Cruise

[ ]  Power .... 75% or LESS

[ ]  Stabilator Trim .... ADJUST

[ ]  Rudder Trim (if installed) .... ADJUST

[ ]  Mixture .... LEAN

[ ]  Cowl Flaps .... CLOSED

Descent

[ ]  Fuel Selector Valve .... BOTH

[ ]  Mixture .... ADJUST

[ ]  Power .... AS DESIRED

[ ]  Carburetor Heat .... FULL AS REQUIRED

Before Landing

[ ]  Seats, Belts, Shoulder Harnesses .... SECURE

[ ]  Fuel Selector Valve .... BOTH

[ ]  Mixture .... RICH

[ ]  Propeller .... HIGH RPM

[ ]  Carburetor Heat .... ON

[ ]  Airspeed .... 80-90 MPH

[ ]  Flaps .... AS DESIRED

Balked Landing

Balked landing/go-around

[ ]  Power [MEMORY] .... FULL THROTTLE, 2700 RPM

[ ]  Carburetor Heat [MEMORY] .... COLD

[ ]  Wing Flaps (immediately) [MEMORY] .... 20°

[ ]  Climb Speed [MEMORY] .... BEST ANGLE

[ ]  Wing Flaps (after safe alt. and 75 MPH) [MEMORY] .... RETRACT

After Landing

[ ]  Wing Flaps .... UP

[ ]  Carburetor Heat .... COLD

[ ]  Cowl Flaps .... OPEN

[ ]  Transponder .... STANDBY

Shutdown

[ ]  Parking Brake .... SET

[ ]  Avionics Power Switch .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Ignition Switch .... OFF

[ ]  Master Switch .... OFF

[ ]  Control Lock .... INSTALL

[ ]  Fuel Selector Valve .... RIGHT

Speeds for Normal Operation

NOTE: Vx 69 MPH = Best Angle of Climb Speed

NOTE: Vy 92 MPH = Best Rate of Climb Speed

NOTE: Vbest l/d 85 MPH = Best Glide (lift/drag), flaps up/down

NOTE: Vs 66 MPH = Stall Speed, normal configuration

NOTE: Vso 56 MPH = Stall Speed, landing configuration

NOTE: Vfo 130/105 MPH = Maximum Flap Extension Speed

NOTE: Va 117 MPH = Maneuvering Speed (at gross weight)

NOTE: Vno 155 MPH = Maximum Structural Cruising Speed

NOTE: Vne 185 MPH = Never Exceed Speed

NOTE: 90-100 MPH = Normal Climb Out

NOTE: 69 MPH = Short Field T/O, Flaps Flaps 15°

NOTE: 80-90 MPH = Normal Landing Approach, Flaps Up

NOTE: 70-80 MPH = Normal Landing Approach, Flaps 40°

NOTE: 70 MPH = Short Field Approach, Flaps 40°

Specifications

NOTE: 2500 lbs = Maximum Gross Weight

NOTE: 1598 lbs = Basic Empty Weight

NOTE: 608 lbs = Payload With Full Fuel

NOTE: 10.0 gph = Fuel Flow @ 75% Power

NOTE: 50 gal = Total Fuel Capacity

NOTE: 49 gal = Total Useable Fuel

NOTE: 100LL = Fuel Octane Rating (Blue)

NOTE: 8 qts = Oil Sump Capacity

NOTE: 6 qts = Minimum Oil Quantity

NOTE: 180 hp = Lycoming O-360-A1F6

NOTE: 12 V = Battery

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/177 Cardinal/jochman-clc177.txt

Rights: licensed

Checklist hash: abd9104ed0454ef10822733a52fdf1b465cda6187f555b346cb98f5f2af1c570

Verify against the aircraft's own approved documentation before use.