Cessna 182RG Skylane — Paul_Yust_cessna_c182RG_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "ENGINE FIRE – GROUND" item count differs between passes (8 vs 6); verify nothing was dropped.

PREFLIGHT - COCKPIT

[ ]  W&B / I’M SAFE .... CHECK

[ ]  AROW Documents .... ON BOARD

[ ]  Hobbs/Tach Time .... RECORD

[ ]  Key .... INSERTED

[ ]  Master .... ON

[ ]  Fuel Gauges/Fuel Valve .... CHECK

[ ]  Lights/Flaps .... CHECK

[ ]  Landing Gear Light .... GREEN

[ ]  Master .... OFF

PREFLIGHT - EXTERNAL

[ ]  Tie Downs & Chocks .... REMOVE

[ ]  Fuel Sumps (1st Flight) .... SAMPLE

[ ]  Fuel Quantity .... VISUAL CHECK

[ ]  Oil Level .... 5-8 QTS

[ ]  Windshield .... CLEAN

[ ]  Antennas/Static Ports .... CHECK

[ ]  Wings/ Ailerons/Flaps .... CHECK

[ ]  Rudder/Elevator .... CHECK

[ ]  Pitot Tube & Stall Vane .... CHECK

[ ]  Main Gear/Brakes (50 psi) .... CHECK

[ ]  Nose Gear (68 psi) .... CHECK

[ ]  Propeller & Spinner .... CHECK

[ ]  Baggage Door .... SECURE

ENGINE START

[ ]  Passenger Brief .... COMPLETE

[ ]  Seats & Belts .... SECURE

[ ]  Avionics/Electrics .... OFF

[ ]  Fuel Selector .... BOTH

[ ]  Cowl Flaps .... OPEN

[ ]  Master .... ON

[ ]  Beacon (Always On) .... ON

[ ]  Carb Heat .... OFF

[ ]  Mixture & Prop .... FULL FORWARD

[ ]  Prime (2-6 Strokes) .... AS REQ

[ ]  Throttle .... PUMP then ¼” OPEN

[ ]  Aux Fuel Pump (Check Press.) .... ON/OFF

[ ]  Prop Area .... CLEAR

[ ]  Brakes .... HOLD

[ ]  Mags .... START

[ ]  Throttle .... 1000 RPM

[ ]  Oil Pressure .... CHECK

[ ]  Mixture .... LEAN

[ ]  Avionics/Electrics .... ON & SET

[ ]  Ground ATC .... CLEARENCE

[ ]  Parking Brake .... OFF

[ ]  Taxi Checks .... BRAKES, TC, AI, & DG

V-SPEEDS (KIAS @ Max Wt)

[ ]  VGLIDE .... 80

[ ]  VX .... 65

[ ]  VA .... 112

[ ]  VS0 .... 39

[ ]  VY .... 88

[ ]  VNO .... 158

[ ]  VS .... 41

[ ]  VFE 20° .... 95

[ ]  VNE .... 181

[ ]  VR .... 50

[ ]  VFE 10° .... 140

[ ]  VLO .... 140

RUNUP & TAKEOFF

[ ]  Mixture (<3000’) .... RICH

[ ]  Engine Run-Up .... 1700 RPM

[ ]  -  Mags 175/50, Prop, Carb., Engine Inst. .... CHECK

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Flight Instruments .... SET

[ ]  Elevator & Rudder Trim .... SET

[ ]  Flaps (Short/Soft 20o) .... SET

[ ]  Autopilot .... CHECK & OFF

[ ]  Seats/Belts/Doors/Windows .... SECURE

[ ]  Tower & Traffic .... CLEARENCE

[ ]  Lights & Pitot Heat .... SET

[ ]  Power (Throttle/Prop/Mix) .... ALL FWD

[ ]  Climb Speed .... 88 KTS

[ ]  Gear/Flaps .... UP

[ ]  Landing light .... OFF

CRUISE

[ ]  Cowl Flaps .... CLOSED

[ ]  Power .... AS REQ

[ ]  Mixture .... LEAN BELOW 75% PWR

LANDING

[ ]  ATIS/AWOS .... OBTAIN

[ ]  IFR Approach Brief .... COMPLETE

[ ]  GUMPS Check .... COMPLETE

[ ]  Flaps .... SET

[ ]  Landing Light .... ON

[ ]  Approach Speed .... 70-80 KTS

MISSED APPROACH

[ ]  Cram .... FULL THROTTLE

[ ]  Climb (100 KTS) .... PITCH +6o on AI

[ ]  Cleanup .... GEAR & FLAPS

[ ]  Cool .... COWL FLAPS OPEN

[ ]  Call .... TRAFFIC/TOWER

AFTER LANDING

[ ]  Flaps .... UP

[ ]  Elevator & Rudder Trim .... SET

[ ]  Cowl Flaps .... OPEN

[ ]  Carb Heat .... OFF

- READY FOR TOUCH & GO

[ ]  Mixture .... LEAN

[ ]  Transponder .... STANDBY

[ ]  Pitot Heat, Strobe .... OFF

SHUTDOWN

[ ]  S  Switches/Avionics .... OFF

[ ]  L  Lean Mixture .... IDLE-CUTOFF

[ ]  I   Ignition Mags .... OFF

[ ]  M Master .... OFF

[ ]  Hobbs/Tach Time .... RECORD

[ ]  Gust Lock .... SECURE

[ ]  Flight Plan .... CLOSED

EMERGENCY LANDING  [EMERGENCY]

[ ]  Pitch For Best Glide .... 80 KTS

[ ]  Landing field .... SELECTED

TIME PERMITTING:

[ ]  Aux Fuel Pump .... ON

[ ]  Fuel Tank Selector .... BOTH

[ ]  Mixture .... RICH

[ ]  Mags .... BOTH

[ ]  Radio (121.5 or Tower) .... MAYDAY

[ ]  Transponder .... 7700

[ ]  ELT .... ON

[ ]  Seats & Belts .... READY

[ ]  Door .... AJAR

[ ]  Mixture/Fuel Valve .... OFF

[ ]  Gear/Flaps .... AS REQ

[ ]  Mags/Master .... OFF

[ ]  Short Final (full-no flaps) .... 65-70 KTS

RESTART ENGINE – IN FLIGHT  [EMERGENCY]

[ ]  Airspeed .... 80 KTS

[ ]  Fuel Selector .... BOTH

[ ]  Carb Heat .... HOT

[ ]  Throttle .... ¼” OPEN

[ ]  Mixture & Prop .... FULL FORWARD

[ ]  Mags .... BOTH

[ ]  Aux Fuel Pump (If no pressure) .... ON

[ ]  Mixture (If no restart) .... CYCLE

MANUAL GEAR EXTENSION  [EMERGENCY]

[ ]  Gear Light/Circuit Breaker .... CHECK

[ ]  Airspeed .... 100 KTS

[ ]  A/P (if available) .... ENGAGE

[ ]  Gear Pump Breaker .... PULL OUT

[ ]  Gear Lever .... DOWN

[ ]  Gear Hand Pump .... EXTEND/PUMP 20+

[ ]  Gear Hand Pump .... STOW

[ ]  Gear Pump Breaker .... PUSH IN

GEAR RETRACT FAILURE  [ABNORMAL]

[ ]  Master .... ON

[ ]  Gear Lever .... VERIFY UP

[ ]  Gear & Pump Breakers .... CHECK IN

[ ]  Gear Lever .... CYCLE

[ ]  Gear Motor (amps & noise) .... VERIFY

ENGINE FIRE – GROUND  [EMERGENCY]

[ ]  Mags .... CONTINUE CRANKING

IF ENGINE STARTS WITHOUT FIRE:

[ ]  Power .... 1700 RPM FOR 3 MINS

IF ENGINE DOES NOT START:

[ ]  Throttle .... FULL OPEN

[ ]  Mixture/Fuel Valve .... OFF

[ ]  Mags/Master .... OFF

[ ]  Exit Plane .... ASAP

ENGINE FIRE – IN FLIGHT  [EMERGENCY]

[ ]  Mixture/Fuel Valve .... OFF

[ ]  Mags/Master .... OFF

[ ]  Vents/Window .... OPEN if smoke enters

[ ]  Airspeed (blow out fire) .... 100+ KTS

[ ]  Land .... ASAP

ELECTRICAL FIRE – IN FLIGHT  [EMERGENCY]

[ ]  Master/Avionics .... OFF

[ ]  Vents/Window .... OPEN if smoke enters

[ ]  Extinguisher .... ACTIVATE

IF ELECTRICS REQUIRED:

[ ]  Circuit Breakers .... CHECK

[ ]  All Electric Switches .... OFF

[ ]  Master .... ON

[ ]  Electrical Switches .... 1 AT TIME

AMMETER SHOW DISCHARGE  [ABNORMAL]

[ ]  Alternator (Left Side Master) .... OFF

[ ]  Nonessential Electrics .... OFF

OVER VOLTAGE LIGHT ON  [ABNORMAL]

[ ]  Avionics Master .... OFF

[ ]  Alternator (Left Side Master) .... OFF/ON

[ ]  Avionics Master .... ON

ICING CONDITIONS  [ABNORMAL]

[ ]  Pitot Heat .... ON

[ ]  Direction and/or Altitude .... CHANGE

[ ]  Cabin Heat & Defrost .... ON

[ ]  Carb Heat .... AS REQ

[ ]  Airspeed .... FASTER THAN NORMAL

IFR APPROACH @ ~ 95 KTS

Turn-Time-Twist-Throttle-Talk

[ ]  Level flight approaching FAF .... ~16”, Prop IN, Gear UP, Flaps UP

[ ]  500 FPM ILS precision decent .... ~15”, Prop IN, Gear DN, Flaps 10°

[ ]  700 FPM VOR non-precision decent .... ~13”, Prop IN, Gear DN, Flaps 10°

[ ]  Level flight circle to land .... ~20”, Prop IN, Gear DN, Flaps 10°

WEIGHT & BALANCE (LBS)

[ ]  Gross Weight .... 3100 LBS

[ ]  Empty Weight .... 1886 LBS

[ ]  Useful Load .... 1214 LBS

[ ]  Payload (65 gals @ neck*) .... 824 LBS

[ ]  Payload (88 gals @ full) .... 686 LBS

NOTE: To the bottom filler neck

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/182RG Skylane/Paul_Yust_cessna_c182RG_checklist.txt

Rights: licensed

Checklist hash: b07316fc938a6028329ca3636c57d74af7a4d69ff9fda2badbb727aaae76dc0e

Verify against the aircraft's own approved documentation before use.