Cessna 182S Skylane — JetStream_C182 Front Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Before Start

[ ]  Preflight Inspection .... Complete

[ ]  Passenger Briefing .... Complete

[ ]  Seats / Seatbelts .... Adjust, Lock

[ ]  Fuel Selector Valve .... Both

[ ]  Cowl Flaps .... Open

[ ]  Brakes .... Set

[ ]  Avionics, Electrical .... Off

[ ]  Circuit Breakers .... Check In

Normal Engine Start

[ ]  Mixture .... Rich

[ ]  Propeller .... High RPM

[ ]  Power .... 1/2 Inch

[ ]  Carburetor Heat .... Cold

[ ]  Prime .... 1-6 as req'd

[ ]  Rotating Beacon .... On

[ ]  Master Switch .... On

[ ]  Area .... Clear

[ ]  Ignition Switch .... Engage

[ ]  Mixture .... Lean

[ ]  Engine Instruments .... Check

[ ]  Taxi, Nav. Lights .... As Required

[ ]  Avionics Master .... On

[ ]  Flaps .... Retract

Taxi

[ ]  Weather info/ATIS .... Obtain

[ ]  Avionics and Flight Instruments .... Check/Set

[ ]  Brakes .... Release, Check

Run Up

[ ]  Doors .... Closed, Locked

[ ]  Parking Brake .... Set

[ ]  Flight Controls .... Free and Correct

[ ]  Flight Instruments .... Check and Set

[ ]  Fuel Selector .... Both

[ ]  Trim .... Set for takeoff

[ ]  Power .... 1700 rpm

[ ]  Mixture .... Set

[ ]  Magnetos .... Left, Right, Both

[ ]  Propeller .... Cycle

[ ]  Carb Heat .... Check

[ ]  Engine Instruments .... Check

[ ]  Vacuum .... Check

[ ]  Ammeter .... Check with load

[ ]  Throttle .... Idle, then 1000 RPM

[ ]  Throttle friction lock .... Set

[ ]  Flaps .... Set for takeoff

[ ]  Emergency, dep. brief .... Perform

[ ]  Radios .... Set for Departure

[ ]  Navigation / GPS .... Set for Departure

[ ]  Brakes .... Release

Before Takeoff

[ ]  Landing light, strobes .... On

[ ]  Transponder .... Set to altitude

[ ]  Area .... Clear

Normal Takeoff (to 1000' AGL)

[ ]  Power/Prop .... Full

[ ]  Mixture .... Adjust smooth

[ ]  Elevator control .... Lift nose at 50 KIAS

[ ]  Climb Speed .... 80 KIAS

Climb (above 1000' AGL)

[ ]  Climb speed .... 90-100 KIAS

[ ]  Power/Prop .... Top of the Green

[ ]  Mixture .... Adjust

[ ]  Lights .... As required

Cruise

[ ]  Power/Prop .... 15-23 in., 2100-2450 RPM

[ ]  Elevator/Rudder trim .... Adjust

[ ]  Mixture .... Lean for altitude

[ ]  Cowl Flaps .... Closed/As Req'd

Descent

[ ]  Approach Briefing .... Complete

[ ]  Lights .... On

[ ]  Fuel Selector .... Both

[ ]  Cowl Flaps .... Closed

[ ]  Mixture .... Set

[ ]  Power/Prop .... Set

Before Landing

[ ]  Seats / Seatbelts .... Check Secure

[ ]  Fuel Selector .... Both

[ ]  Carb Heat .... On

[ ]  Propeller .... Full forward

Normal Landing

[ ]  Airspeed .... 70-80 KIAS flaps up

[ ]  Flaps .... As required

[ ]  Airspeed .... 65-75 KIAS flaps down

After Landing

[ ]  Cowl Flaps .... Open

[ ]  Trim .... Takeoff

[ ]  Flaps .... Retract

[ ]  Carb Heat .... Off

[ ]  Land, Strobe lights .... Off

[ ]  Transponder .... Standby

Shutdown and Securing

[ ]  Power .... Idle

[ ]  Avionics and Switches .... Off

[ ]  Mixture .... Idle Cutoff

[ ]  Mags .... Off

[ ]  Master .... Off

[ ]  Control Lock .... In

[ ]  Hobbs and Tach .... Record

[ ]  Tie Downs .... Attached

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/182S Skylane/JetStream_C182 Front Checklist.txt

Rights: licensed

Checklist hash: 3c3dc315b6bf10088596ce2578260b85397f34ea7cdfb742806f7e08ea489d75

Verify against the aircraft's own approved documentation before use.