Cessna 182S Skylane — N358ME_CESSNA_182_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

BEFORE STARTING ENGINE

[ ]  Preflight inspection .... COMPLETE

[ ]  Seats, Seat Belts, and Shoulder Harnesses .... ADJUST and LOCK

[ ]  Brakes .... TEST

[ ]  Circuit Breakers .... CHECK IN

[ ]  Electrical Equipment .... OFF

[ ]  Avionics Master Switch .... OFF

[ ]  Autopilot .... OFF

[ ]  Cowl Flaps .... OPEN

[ ]  Fuel Selector Valve .... BOTH

[ ]  Avionics Circuit Breakers .... CHECK IN

STARTING ENGINE

[ ]  Throttle .... OPEN 1/4"

[ ]  Propeller .... HIGH RPM

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Propeller Area .... CLEAR

[ ]  Master Switch .... ON

[ ]  Flashing Beacon .... ON

[ ]  Auxiliary Fuel Pump .... ON

[ ]  Mixture .... ADVANCE until fuel flow just starts to rise, then return to IDLE CUT-OFF position

[ ]  Auxiliary Fuel Pump .... OFF

[ ]  Ignition Switch .... START (release when engine starts)

[ ]  Mixture .... Advance smoothly to RICH when engine fires

[ ]  Throttle .... 1000 RPM

[ ]  Oil pressure .... CHECK

[ ]  Mixture .... LEAN one inch for taxi

[ ]  Flaps .... UP

[ ]  Navigation, Cockpit, and Taxi Lights .... ON as required

[ ]  Avionics Master Switch .... ON

NOTE: Communication Radios: DVT / SDL / FFZ; ATIS 126.5 / 118.6 / 118.25; Ground 121.8 / 121.6 / 121.3; Tower S: 118.4 / 119.9 / 124.6, N: 120.2; Practice area 122.75 (north), 122.85 (south)

[ ]  Altimeter .... SET to setting given by ATC

[ ]  Annunciator Panel .... TEST

[ ]  GPS .... Confirm self-test and altimeter setting

[ ]  Transponder .... TEST, then STANDBY

[ ]  Autopilot .... enter altimeter setting

[ ]  Taxiing .... CHECK brakes immediately after airplane begins moving

DURING TURNS WHILE TAXIING

[ ]  Turn Coordinator .... Indicates skidding turn in proper direction

[ ]  Heading Indicator .... Turns in proper direction

[ ]  Magnetic Compass .... Turns in proper direction, bowl full of fluid

BEFORE TAKEOFF

[ ]  Passenger Seat Backs .... MOST UPRIGHT POSITION

[ ]  Seats & Seat Belts .... CHECK SECURE

[ ]  Cabin Doors .... CLOSED and LATCHED

[ ]  Flight controls .... FREE and CORRECT

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Selector Valve .... RECHECK BOTH

[ ]  Mixture .... FULL RICH

[ ]  Throttle .... 1800 RPM

[ ]  Magnetos .... CHECK (max drop 150 RPM, max diff. 50 RPM)

[ ]  Propeller .... CYCLE from high to low to high RPM

[ ]  Vacuum .... Normal operating range

[ ]  Ammeter .... CHECK

[ ]  Engine Instruments .... CHECK

[ ]  Annunciator Panel .... No warnings illuminated

[ ]  Throttle .... CHECK IDLE

[ ]  Throttle .... 1000 RPM

[ ]  Throttle Friction Lock .... ADJUST

[ ]  Flight instruments .... SET

[ ]  Attitude Indicator .... Check, Set for level flight

[ ]  Vertical Speed Indicator .... Note indication of 0 fpm climb

[ ]  Altimeter .... Should read within 75 ft of field elevation

[ ]  Airspeed Indicator .... indicates 0

[ ]  Pitot heat .... CHECK

[ ]  Elevator & Rudder Trim .... SET for TAKEOFF

[ ]  Communication Radio .... Tower or CTAF

[ ]  Navigation Radios .... SET if flying cross-country

[ ]  NAV/GPS HIS Switch .... SET

[ ]  Autopilot .... OFF

[ ]  Strobe and Landing Lights .... ON

[ ]  Windows .... CLOSED

[ ]  Parking Brake .... OFF

[ ]  Transponder .... ALT

NORMAL TAKEOFF

[ ]  Flaps .... 0°

[ ]  Throttle .... FULL OPEN

[ ]  Airspeed Indicator .... Indicates increasing speed

[ ]  Elevator Control .... Rotate at 60 KIAS

SHORT FIELD TAKEOFF

[ ]  Flaps .... 20°

[ ]  Brakes .... HOLD

[ ]  Throttle .... FULL OPEN

[ ]  Brakes .... RELEASE

[ ]  Climb Speed .... 58 KIAS until obstacles cleared, then as appropriate

[ ]  Flaps .... RETRACT after reaching 70 KIAS

NORMAL CLIMB

[ ]  Airspeed .... 85-95 KIAS

[ ]  Power .... 23" or FULL THROTTLE (whichever is less)

[ ]  RPM .... REDUCE to 2400 RPM

[ ]  Mixture .... 15 GPH or FULL RICH (whichever is less)

CRUISE

[ ]  Power .... 15-23", 2000-2400 RPM (no more than 80% is recommended)

[ ]  Elevator Trim .... ADJUST

[ ]  Mixture .... lean to peak EGT minus 125° F

[ ]  Cowl Flaps .... CLOSED

BEFORE LANDING

[ ]  Passenger Seat Backs .... MOST UPRIGHT POSITION

[ ]  Seats & Seat Belts .... SECURED & LOCKED

[ ]  Fuel Selector Valve .... BOTH

[ ]  Mixture .... RICH

[ ]  Propeller .... HIGH RPM

[ ]  Strobe and Landing Lights .... ON

[ ]  Autopilot .... OFF

NORMAL LANDING

[ ]  Airspeed .... 70-80 KIAS (flaps UP)

[ ]  Flaps .... 0-10° < 140 KIAS, 10°-20° < 120 KIAS, FULL < 100 KIAS

[ ]  Airspeed .... 60-70 KIAS (flaps DOWN)

SHORT OR SOFT FIELD LANDING

[ ]  Flaps .... FULL DOWN (below 100 KIAS)

[ ]  Airspeed .... 60 KIAS

BALKED LANDING

[ ]  Throttle .... FULL OPEN

[ ]  Flaps .... RETRACT TO 20° (immediately)

[ ]  Airspeed .... 55 KIAS (until obstacles are cleared)

[ ]  Flaps .... RETRACT (after reaching a safe altitude and 70 KIAS)

[ ]  Cowl Flaps .... OPEN

AFTER LANDING

[ ]  Flaps .... RETRACT

[ ]  Cowl Flaps .... OPEN

[ ]  Transponder .... STANDBY

SHUTDOWN

[ ]  Call for fuel .... on 130.65 @ DVT

[ ]  Avionics Master Switch .... OFF

[ ]  Electrical Equipment .... OFF (except flashing beacon)

[ ]  Throttle .... IDLE

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Ignition Switch .... OFF (after propeller stops turning)

[ ]  Flashing Beacon .... OFF

[ ]  Master Switch .... OFF

[ ]  Control Lock .... INSTALL

[ ]  Vents .... CLOSED

[ ]  Tiedowns .... SECURED

[ ]  Flight Plan .... CLOSED

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/182S Skylane/N358ME_CESSNA_182_Checklist.txt

Rights: licensed

Checklist hash: 5d9007ce988a4c590db254f41e7b08c717611dd673913d7342bfb1b034d7beeb

Verify against the aircraft's own approved documentation before use.